Copyright (c) 1985-1997 Jim Lopushinsky

email: jimlop@ix.netcom.com

This is my original documentation:

Dec 10, 1985

Jim Lopushinsky

CPM.EXE allows execution of CP/M-86 .CMD files under MS-DOS and PC-DOS.

A>CPM <cpm-86 file name> <paramaters>

To run CP/M-80 .COM files, run the Z80 emulator under the CPM emulator:

A>CPM Z80 <cpm-80 file name> <paramaters>

Don't try running any funny CP/M stuff that calls the BIOS disk I/O

functions directly.  It won't work.  Other than that, virtually any

CP/M-86 .CMD program will be successfully executed.  Note that programs

which make use of MP/M or Concurrent extended functions will not work.

All functions except one are totally machine independent, and should

run on any hardware configuration of MS-DOS version 3.0 or greater.

The only exception:  There is no DOS function call for printer status,

and the CP/M printer status request is translated to INT 17H.  This

should work fine on any PC or PC Clone.

Nov 30/97:

ANSI.SYS is required if you plan to run .CMD files that use IBM CP/M-86

version 1.1 or newer (or CCPM, CDOS etc) escape sequences.  The following

CP/M escape sequences are converted to ANSI escape sequences:

ESC A           cursor up

ESC B           cursor down

ESC C           cursor forward

ESC D           cursor back

ESC E           clear screen and cursor home

ESC H           cursor home

ESC K           clear to end of line from cursor position

ESC Y           set cursor position

ESC :           program function key

ESC a           set video mode

ESC b           set forground color

ESC c           set background color

ESC j           save cursor position

ESC k           restore cursor position

ESC m           cursor on

ESC n           cursor off

ESC p           reverse video on

ESC q           reverse video off

ESC r           intensity on

ESC s           blink on

ESC t           blink off

ESC u           intensity off

WordStar key translations are preformed based on an IBM compatible keyboard

as follows:

ALT-X           ^KX     Save and exit

Up Arrow        ^E      Cursor up

Down Arrow      ^X      Cursor down

Left Arrow      ^S      Cursor left

Right Arrow     ^D      Cursor right

Page Up         ^R      Scroll screen up

Page Down       ^C      Scroll screen down

Insert          ^V      Toggle insert mode

Delete          ^G      Delete character at cursor

Home            ^QS     Beginning of line

End             ^QD     End of line

Ctrl-Page Down  ^QC     End of document

Ctrl-Page Up    ^QR     Beginning of document

------------------------------

Configuration options:

You will need a hex editor to change configuration.  Search for the

string "USER OPTIONS" in CPM.EXE.  Currently, the following options

can be changed:

BDOS call frequencies.

Offset 655 hex.  0 = no frequencies, 1 = tabulate CP/M BDOS call

frequencies.  Default = off.  If this option is set, after any CP/M-86

CMD file terminates, run CPMFREQ.EXE and examine CPMFREQ.TXT.

CPMFREQ.EXE requires CPM.TXT and CPMFREQ.DAT.  CPMFREQ.DAT is produced

by the CP/M emulator.

WordStar key translation.

Offset 685 hex.  0 = no translation, 1 = translate to WS key sequences.

Default = on.

IBM-PC CP/M-86 screen escape sequences.

Offset 6B5 hex.  0 = no translation, 1 = translate CP/M-86 screen codes

to ANSI.  Default = on.  ANSI.SYS required if enabled.

BDOS version.

Offset 6E5 hex.  This is a BCD value for the BDOS version that CP/M

function 12 returns.  Default value = 31 hex.  Change to 22 hex for

CP/M-86.

CP/M compatibility.

Offset 715 hex.  0 = CP/M compatible, 1 = CCP/M, CDOS compatible.

Default = 0 (CP/M compatible).  This changes how BDOS functions 31

(get_DBP) and 53 (mc_max) operate:

CP/M compatible:        Function 31 (get DBP) returns SPT (sectors per

track) as the number of logical 128 byte sectors.

Function 53 (get max memory) does not allocate

the memory.

CCP/M compatible:       Function 31 (get DBP) returns SPT as the number

of physical sectors.

Function 53 (get max memory) allocates the memory.

-------------------------------

Revision history:

12/10/85       Initial version

10/06/96 1.1   Added option to record frequency of CP/M BDOS calls.

11/03/97 1.2   Implemented segment relocation fixups.  Now DRC.CMD works.

11/30/97 1.3   Implemented Wordstar key translation for IBM keyboard.

Implemented CP/M-86 for IBM-PC screen escape sequences.

Translates to ANSI.  ANSI.SYS required.