MP/M II

Operating System

PROGRAMMER'S GUIDE

Copyright © 1981

Digital Research

P.O. Box 579

801 Lighthouse Avenue

Pacific Grove, CA 93950

(408) 649-3896

TWX 910 360 5001

All Rights Reserved

COPYRIGHT

Copyright 1981 by Digital Research. All rights

reserved. No part of this publication may be

reproduced, transmitted, transcribed, stored in a

retrieval system, or translated into any language or

computer language, in any form or by any means,

electronic, mechanical, magnetic, optical, chemical,

manual or otherwise, without the prior written

permission of Digital Research, Post Office Box 579,

Pacific Grove, California, 93950.

However, this manual is tutorial in nature.

Thus, the reader is granted permission to include

the example programs, either in whole or in part, in

his own programs.

DISCLAIMER

Digital Research makes no representations or

warranties with respect to the contents hereof and

specifically disclaims any implied warranties of

merchantability or fitness for any particular

purpose. Further, Digital Research reserves the

right to revise this publication and to make changes

from time to time in the content hereof without

obligation of Digital Research to notify any person

of such revision or changes.

TRADEMARKS

CP/M is a registered trademark of Digital Research.

CP/NET, MP/M, and MP/M II are trademarks of Digital

Research.

The "MP/M II Programmer's Guide" was prepared using

the Digital Research TEX Text Formatter and printed

in the United States of America by Commercial Press

of Monterey.

*******************************

*  First Edition: August 1981 *

* Second Edition: August 1982 *

*******************************

Foreword

MP/M II T.M., is a multi-user operating system for microcomputers

that use the Intel 8080, the Zilog Z800, or similar 8-bit type

architecture. It will support multi-terminal access with multi

programming at each terminal. It uses the same Basic Disk Operating

System (BDOS) as CP/M@ thus assuring compatibilty of existing

programs running under CP/M.

The minimum hardware environment for MP/M II must include an

8080 or Z80 processor, 32K bytes of random access memory (RAM),a

system console, and a real-time clock. A typical MP/M II kernel

occupies approximately 15K bytes.

This manual describes the programming interface to MP/M II. It

gives a general description of the modules that comprise the

operating system, the manner in which MP/M II manages the memory

resource and monitors running processes, as well as detailed

descriptions of all the system entry points. Also included are

descriptions of several utility programs that are useful for

creating and debugging programs under MP/M II. This manual is not

intended as a tutorial. Therefore, familiarity with the material

covered in the User's Guide and with processor architecture and

assembly language in general is required.

Table of Contents

1  Introduction to MP/M II

1.1		Overview of MP/M II Features  . . . . . . . . . 1

1.2		MP/M II Nucleus . . . . . . . . . . . . . . . . 4

1.2.1 Process Dispatching   . . . . . . . . . . . . . 4

1.2.2 Queue Management  . . . . . . . . . . . . . . . 6

1.2.3 Flag Management   . . . . . . . . . . . . . . . 7

1.2.4 Device Polling  . . . . . . . . . . . . . . . . 8

1.2.5 Console and List Device Management  . . . . . . 8

1.2.6 Memory Management   . . . . . . . . . . . . . . 9

1.2.7 System Timing Functions  . . . . . . . . . . . 10

1.3	MP/M II Memory Structure . . . . . . . . . . . . . . 11

1.4	Terminal Message Process . . . . . . . . . . . . . . 14

1.5	Command Line Interpreter . . . . . . . . . . . . . . 15

1.6	Transient Programs . . . . . . . . . . . . . . . . . 18

1.7	Resident System Processes . .

1.8	BDOS and XDOS Calling Conventions

2  The BDOS Interface

2.1	BDOS Console and List I/O Interface  . . . . . . . . 23

2.2	BDOS File System   . . . . . . . . . . . . . . . . . 24

2.2.1 File Naming Conventions  . . . . . . . . . . . 26

2.2.2 Disk Drive and File Organization   . . . . . . 28

2.2.3 File Control Block Definition  . . . . . . . . 29

2.2.4 User Number Conventions  . . . . . . . . . . . 33

2.2.5 Directory Labels and XFCBs   . . . . . . . . . 34

2.2.6 File Passwords   . . . . . . . . . . . . . . . 36

2.2.7 File Date and Time Stamps  . . . . . . . . . . 37

2.2.8 File Open Modes  . . . . . . . . . . . . . . . 38

2.2.9 File Security  . . . . . . . . . . . . . . . . 39

2.2.10 Concurrent File Access  . . . . . . . . . . . 41

2.2.11 Multi-Sector I/O  . . . . . . . : . . . . . . 43

2.2.12 XIOS Blocking and Deblocking  . . . . . . . . 43

2.2.13 Reset, Access and Free Drive  . . . . . . . . 44

2.2.14 BDOS Error Handling   . . . . . . . . . . . . 47

Table of Contents

(continued)

2.3	Base Page Initialization . . . . . . . . . . . . . .  53

2.4	BDOS Function Calls  . . . . . . . . . . . . . . . .  57

3  XDOS Interface

3.1	Introduction . . . . . . . . . . . . . . . . . . . . 111

3.2	Process Descriptor Data Structure  . . . . . . . . . 111

3.3	Queue Data Structures . . . . . . . . . . . . . . .  116

3.3.1	Circular Queues  . . . . . . . . . . . . . . . 116

3.3.2	Linked Queues  . . . . . . . . . . . . . . . . 118

3.3.3	User Queue Control Block . . . . . . . . . . . 120

3.3.4	Queue Naming Conventions . . . . . . . . . . . 121

3.4	Memory Descriptor Data Structure . . . . . . . . . . 121

3.5	System Data Page . . . . . . . . . . . . . . . . . . 122

3.6	XDOS Internal Data Segment . . . . . . . . . . . . . 124

3.7	XDOS Error Handling  . . . . . . . . . . . . . . . . 125

3.8	XDOS Function Calls  . . . . . . . . . . . . . . . . 126

4  ASM

4.1	Overview

4.2	Program Format

4.3	Forming the Operand  . . . . . . . . . . . . . . . . 154

4.3.1	Labels . . . . . . . . . . . . . . . . . . . . 154

4.3.2 Numeric Constants  . . . . . . . . . . . . . . 154

4.3.3 Reserved Words . . . . . . . . . . . . . . . . 155

4.3.4 String Constants . . . . . . . . . . . . . . . 156

4.3.5 Arithmetic and Logical Operators . . . . . . . 157

4.3.6 Precedence of Operators  . . . . . . . . . . . 158

4.4	Assembler Directives . . . . . . . . . . . . . . . . 159

4.4.1	The ORG Directive  . . . . . . . . . . . . . . 160

4.4.2	The END Directive  . . . . . . . . . . . . . . 160

4.4.3	The EQU Directive  . . . . . . . . . . . . . . 161

Table of Contents

(continued)

4.4.4		The SET Directive . . . . . . . . . . . . . 161

4.4.5 The IF and ENDIF Directives . . . . . . . . . . . 162

4.4.6 The DB Directive  . . . . . . . . . . . . . . . . 163

4.4.7 The DW Directive  . . . . . . . . . . . . . . . . 163

4.4.8 The DS Directive  . . . . . . . . . . . . . . . . 164

4.5		Operation Codes   . . . . . . . . . . . . . . . . 164

4.6		Error Messages    . . . . . . . . . . . . . . . . 171

5  RDT

5.1 RDT Overview

5.2		Invoking RDT . . . . . . . . . . . . . . . . . . . 173

5.3		RDT Command Conventions  . . . . . . . . . . . . . 174

5.4		Terminating RDT  . . . . . . . . . . . . . . . . . 175

5.5		RDT Commands . . . . . . . . . . . . . . . . . . . 175

5.5.1		The A (Assemble) Command . . . . . . . . . . 175

5.5.2 The B (Bitmap Bit Set/Reset) CommLnd . . . . . . . 175

5.5.3 The D (Display) Command  . . . . . . . . . . . . . 176

5.5.4 The F (Fill) Command . . . . . . . . . . . . . . . 176

5.5.5 The G (Go) Command . . . . . . . . . . . . . . . . 177

5.5.6 The I (Input File) Command . . . . . . . . . . . . 177

5.5.7 The L (List) Command . . . . . . . . . . . . . . . 178

5.5.8 The M (Move) Command . . . . . . . . . . . . . . . 178

5.5.9 The N (Normalize) Command  . . . . . . . . . . . . 178

5.5.10  The	R	(Read) Command . . . . . . . . . . . . 179

5.5.11  The	S	(Set) Command  . . . . . . . . . . . . 179

5.5.12  The	T	(Trace) Command  . . . . . . . . . . . 180

5.5.13  The	U	(Untrace) Command  . . . . . . . . . . 181

5.5.14  The	V	(Value) Command  . . . . . . . . . . . 181

5.5.15  The	W	(Write) Command. . . . . . . . . . . . 181

5.5.16  The X (Examine CPU State) Command  . . . . . . . 182

Table of Contents

(continued)

6  Other Programming Utilities

6.1	GENHEX. . . . . . . . . . . . . . . . . . . . . . . 183

6.2	GENMOD. . . . . . . . . . . . . . . . . . . . . . . 183

6.3	PRLCOM. . . . . . . . . . . . . . . . . . . . . . . 184

6.4	DUMP. . . . . . . . . . . . . . . . . . . . . . . . 184

6.5	LOAD. . . . . . . . . . . . . . . . . . . . . . . . 185

7  PRL File Generation

7.1 PRL Format  . . . . . . . . . . . . . . . . . . . . . 187

7.2 Generating a PRL  . . . . . . . . . . . . . . . . . . 187

8  RSP Generation

8.1	RSPs and Resident System Procedures . . . . . . . . 191

8.2 Generating an RSP . . . . . . . . . . . . . . . . . . 191

8.3 RSP Code  . . . . . . . . . . . . . . . . . . . . . . 191

8.4	Banked RSPs . . . . . . . . . . . . . . . . . . . . 192

9  SPR Generation

9.1	System Page Relocatable Files . . . . . . . . . . . 193

9.2	Generating an SPR  . . .. . . . . . . . . . . . . . 193

Appendixes

A Flag Assignments . . . . . . . . . . . . . . . . . . . . 195

B Process Priority Assignments . . . . . . . . . . . . . . 197

C BDOS Function Summary  . . . . . . . . . . . . . . . . . 199

D XDOS Function Summary  . . . . . . . . . . . . . . . . . 201

E Sample Page Relocatable Program  . . . . . . . . . . . . 203

F Sample Resident System Process . . . . . . . . . . . . . 209

G Acronyms and Conventions . . . . . . . . . . . . . . . . 213

H Glossary . . . . . . . . . . . . . . . . . . . . . . . . 215

I ASCII and Hexadecimal Conversions  . . . . . . . . . . . 219

Section I

Introduction to MP/M 11

1.1	Overview of MP/M II Features

MP/M II is a microcomputer operating system that supports

multiple terminals with multi-programming at each terminal. Upward

compatible with CP/M, MP/M II presents a CP/M interface to each

terminal. In fact, most CP/M programs can run without modification

under MP/M II. However, MP/M II is not limited to this model.

Using MP/M II's powerful multi-programming capability, a single

terminal can initiate more than one program. In addition, the

system functions used by MP/M II to control the multi-programming

environment are available to application programs. As a result,

MP/M II supports extended features beyond the CP/M model such as

communication between and synchronization of independently running

programs.

Under MP/M II, there is an important distinction between a

program and a process. A program is simply a block of code residing

somewhere in memory or on disk; it is essentially static. A process

on the other hand, is dynamic, and can be thought of as a "logical

machine" that not only executes the program's code, but also

executes code in the operating system. When MP/M II loads a

program, it also creates a process that is associated with the

loaded program. Subsequently, it is the process, rather than the

program that controls all access to the system's resources. Thus,

MP/M II monitors the process, not the program. This distinction is

a subtle one, but vital to understanding the operation of the system

as a whole.

Programs running under MP/M II fall into three categories:

CP/M programs, MP/M II system processes, and MP/M II Resident System

Processes. The first category consists of CP/M-like programs that

MP/M II loads into an available memory segment. MP/M II supports

from 1 to 7 memory segments or partitions that can be loaded with

programs. once loaded and initiated, a program becomes associated

with a process that is maintained by the MP/M II real-time nucleus.

The second category consists of MP/M II system processes that

perform operating system tasks. For example, the Command Line

Interpreter (CLI), is the system process that loads and initiates

user programs.

The final category consists of those processes that can be

optionally integrated into MP/M II during system generation, thus

becoming a part of the system. These processes are called Resident

System Processes (RSPS) . With RSPs, users can write custom

processes and include them in the system along with those supplied

with MP/M II (see Section 1.7 and Section 8) . All processes running

under MP/M II compete for the CPU and other system resources on a

priority basis under control of the real-time nucleus.

MP/M II Programmer's Guide	1.1		Overview of MP/M II Features

The following list briefly summarizes MP/M II's capabilities.

•	Multi-terminal support. MP/M II supports up to 16

terminals. Also, a single process can access multiple

terminals.

•	Multi-programming at each terminal. Any system console can

initiate multiple programs or processes. In addition, a

process can generate sub-processes.

•	Support for bank-switched memory. MP/M II's memory

segments can either reside in common memory or be

distributed through separate memory banks, thereby

extending the system's effective memory capacity.

•	Inter-process communication, synchronization, and mutual

exclusion. These functions are provided by queues.

•	Logical interrupt mechanism using flags. This allows MP/M

II to interface with any physical interrupt structure.

•	System timing functions. These functions enable processes

running under MP/M II to compute elapsed times, delay

execution for specified intervals, and to access and set

the current date and time. In addition, the user can

schedule programs to be run by date and time. The system

timing is also used to provide round-robin scheduling of

compute-bound processes executing at the same priority.

•	User-selected options at system generation time. The

available options include the number of system consoles,

the number, size, and location of memory segments, and the

maximum number of files and locked records supported by the

system at one time. Also, the user can select which RSPs

to include with MP/M II during system generation.

Functionally, MP/M II is composed of three distinct modules:

the Basic Disk Operating System (BDOS) , the Extended Disk Operating

System (XDOS) , and the Extended I/O System (XIOS) . The MP/M II BDOS

is an upward-compatible version of the single-user CP/M BDOS. In

most cases, CP/M programs that make BDOS calls for I/O or direct

BIOS calls for printer and console I/O, can run under MP/M II

without modification. However, MP/M II's BDOS is extended to

provide support for multiple console and list devices. In addition,

the file system is extended to provide services required in multi

user environments.

MP/M II Programmer's Guide		1.1	Overview of MP/M II Features

Two major extensions to the file system are:

•	File locking. Normally, files opened under MP/M II cannot

be opened or deleted by other users. This feature prevents

accidental conflicts with other users.

•	Shared access to files. As a special option, independent

users can open the same file in shared or unlocked mode.

MP/M II supports record locking and unlocking commands for

files opened in this mode, and protects files opened in

shared mode from deletion by other users.

The XDOS module gives MP/M II its multi-programming

capabilities. It contains the real-time nucleus that monitors the

execution of processes and arbitrates conflicts for the system's

resources. It also includes the Terminal message Process (TMP)

which reads and echoes command lines for the system consoles, and

the Command Line Interpreter (CLI) which accepts TMP command lines

and initiates user programs and RSPs. The XDOS also contains the

set of extended MP/M II functions that can be accessed by user

programs.

The XIOS module is similar to the CP/M BIOS module but is

extended in several ways. Primitive functions such as console I/O

are modified to support multiple consoles. Several new primitive

functions support MP/M II's additional features. Also, new

facilities are added to eliminate wait loops. The XIOS is the

hardware-dependent module that defines MP/M II's interface to a

particular hardware environment. Although a standard XIOS is

supplied by Digital Research, the XIOS is usually customized to

support the user's own hardware environment. Note: processes

running under MP/M II can make direct XIOS calls only for console

and list I/O.

When MP/M II is configured for a single console and is

executing a single program, its speed approximates that of CP/M.

The overhead of the MP/M II dispatcher in such an environment will

be 7 to 15%. In environments where either multiple processes and/or

users are running, the speed of each individual process is degraded

in proportion to the amount of I/O and compute resources required.

A process that performs a large amount of I/O in proportion to

computing exhibits only minor speed degradation. This also applies

to a process that performs a large amount of computing, but is

running concurrently with other processes that are largely I/O

bound. on the other hand, significant speed degradation occurs in

those environments where more than one compute-bound process is

running.

MP/M II Programmer's Guide	1.2		MP/M II Nucleus

1.2 MP/M II Nucleus

MP/M II is controlled by a real-time multi-tasking nucleus that

resides within the XDOS module. This nucleus performs process

dispatching, memory management, and system timing tasks. It also

performs queue management, flag management, device polling, and

console and list device management. The following sections describe

these functions in greater detail. Many of the system functions

that perform these tasks can also be called by user programs with

the XDOS functions.

Although MP/M II is a multi-processing operating system, at any

given point in time, only one process has access to the CPU

resource. Unless it is specifically written to communicate or

synchronize execution with other processes, it runs unaware that

other processes may be competing for the system's resources.

Eventually, the system suspends the process from execution and gives

another process the opportunity to run.

1.2.1		Process Dispatching

The primary task of the nucleus is transferring the CPU

resource from one process to another. This task is called

dispatching and is performed by a part of the nucleus called the

Dispatcher. Under MP/M II, each process is associated with a data

structure called a Process Descriptor (see Section 3.2) The

Dispatcher uses this data structure to save and restore the current

state of a running process. Every process in the system resides in

one of three states: ready, running, or suspended. A ready process

is one that is waiting for the CPU resource. A suspended process is

one that is waiting for some other system resource or a defined

event. A running process is one that the CPU is currently

executing.

A dispatch operation for a running process can be described as

follows:

1)	The Dispatcher suspends the process from execution and

stores the current state in the Process Descriptor.

2)	The Dispatcher scans all the suspended processes on the

Ready List and selects the one with the highest priority.

3)	The Dispatcher restores the state of the selected process

from its Process Descriptor and gives it the CPU resource.

4)	The process executes until it makes a system call, or an

interrupt, or a tick of the system clock occurs. Then,

dispatching is repeated.

MP/M II Programmer's Guide	1.2		MP/M II Nucleus

Only processes that are placed on the Ready List are eligible

for selection during dispatch. By definition, a process is on the

Ready List if it is waiting for the CPU resource only. Processes

waiting for other system resources cannot execute until their

resource requirements are satisfied. Under MP/M II, a process is

blocked from execution if it is waiting for:

• a queue message so that it can complete a read queue

operation.

• space to become available in a queue so it can complete a

queue write operation.

• system flag to be set.

• console or list device to become available.

• a specified number of system clock ticks before it can be

removed from the system Delay List.

• an I/O event to complete.

These situations are discussed in more detail in the following

sections.

MP/M II is a priority-driven system. This means that the

Dispatcher selects the highest priority ready process and gives it

the CPU resource. Processes with the same priority are "round

robin" scheduled. That is, they are given equal CPU time slices

when executing CPU bound code. With priority dispatching, control

is never passed to a lower priority process if there is a higher

priority process on the Ready List. Since high priority compute

bound processes tend to monopolize the CPU resource, it is advisable

to lower their priority to avoid degrading overall system

performance. In addition, compute-bound processes can make XDOS

Dispatch calls periodically to promote sharing of the CPU resource

in those systems that do not support a clock. When a process makes

a Dispatch call, the call appears as a null operation to the

process, but allows other processes to gain access to the CPU

resource.

MP/M II requires that at least one process be running at all

times. To ensure this, the system maintains the IDLE process on the

Ready List so it can be dispatched if there are no other processes

available. The IDLE process runs at a very low priority and is

never blocked from execution. It does not perform any useful task,

but simply gives the system a process to run when no other ready

processes exist.

MP/M II Programmer's Guide	1.2		MP/M II Nucleus

1.2.2		Queue Management

Queues perform several critical functions for processes running

under MP/M II. They are used for communicating messages between

processes, for synchronizing process execution, and for mutual

exclusion. Queues are special data structures, implemented in MP/M

II as "memory files" that contain room for a specified number of

fixed length messages (see Section 3.3). Like files, queues are

made, opened, deleted, read from, and written to with XDOS function

calls. When a queue is created with the XDOS Make Queue command, it

is assigned an 8-character name that identifies the queue in XDOS

Open Queue commands. As the name implies, messages are

read from a queue on a first-in, first-out basis.

A process can read messages from a queue or write messages to a

queue in two ways: conditionally or unconditionally. If no

messages exist in the queue when a conditional read is performed, or

the queue is full when a conditional write is performed, the system

returns an error code to the calling process. On the other hand, if

a process performs an unconditional read from an empty queue, the

system suspends the process from execution until another process

writes a message to the queue. A process suspended in this manner

is placed on the queue's Dequeue list. A similar situation occurs

when a process makes an unconditional write to a full queue. A

process suspended in this way is placed on the queue's Enqueue list.

MP/M II uses these Enqueue/Dequeue lists to synchronize process

execution.

When more than one process resides on a queue's Enqueue or

Dequeue list, preference is given to the higher priority process.

Conflicts involving processes with the same priority are resolved on

a first-come first-serve basis.

Mutual exclusion queues are a special type of queue under MP/M

II. They contain one message of zero length and are assigned a name

beginning with the upper-case letters, MX. In effect, a mutual

exclusion queue is a binary semaphore. Mutual exclusion queues

ensure that only one process has access to a resource at a time.

Access to a resource protected by a mutual exclusion queue takes

place as follows:

1)	The process issues an unconditional Read Queue call from the

queue protecting the resource, thereby suspending itself

until the message is available.

2)	The process accesses the protected resource.

3)	The process writes the message back to the queue when it has

finished using the protected resource, thus freeing the

resource for other processes.

As an example, the disk system mutual exclusion queue, MXdisk,

ensures that processes serially access the BDOS file system.

MP/M II Programmer's Guide	1.2		MP/M II Nucleus

Mutual exclusion queues have one other feature that is

different from normal queues. When a process reads a message from a

mutual exclusion queue, the nucleus saves the address of the Process

Descriptor for the process reading the message in a two-byte buffer

area of the queue. If the process is aborted while it owns the

mutual exclusion message, the nucleus automatically writes the

message back to the queue for the aborted process, thus enabling

other processes to gain access to the protected resource.

1.2.3		Flag Management

MP/M II's nucleus uses flags for signaling and synchronizing

processes with defined events. Processes access the system's flags

with the XDOS functions, Flag Set and Flag Wait. Internally, a flag

can reside in two states: set or reset. The reset state is further

divided into two categories:

• No process is waiting for the flag to be set.

• A process is waiting for the flag to be set, and blocked

from execution until it is set.

Note: Two processes are not allowed to wait on the same flag. This

is an error situation referred to as flag "under-run". Similarly, a

process attempting to set a flag that is already set is another

error situation, called flag "over-run".

Flags provide a logical interrupt system independent of the

physical interrupt system of the microcomputer. They are primarily

intended for use by the XIOS module to support the Interrupt

Handler. For example, when the Interrupt Handler receives a

physical interrupt indicating an I/0 operation is complete, it sets

a flag and branches to the Dispatcher. A process suspended from

execution because it is waiting for the flag to be set, is placed on

the Ready List, making it eligible for selection during dispatch.

once dispatched, the process can assume the I/O operation is

complete.

MP/M II supports 32 flags, several of which are reserved. For

example, Flag 1 is reserved for the system clock tick. Because of

their limited number, their use by the XIOS module, and the single

process nature of their design, flags should not be used in

application software except in very special situations. In most

cases, process communication and synchronization are better

accomplished with queues.

MP/M II Programmer's Guide	1.2		MP/M II Nucleus

1.2.4		Device Polling

Device polling is another mechanism a process can use to wait

for an I/O or external event without using flags or consuming the

CPU resource with a programmed delay loop. Polling is implemented

in the XIOS module exclusively. For example, assuming that the XIOS

supports polled console input, when a process makes a BDOS console

input call, the process eventually reaches the XIOS console input

routine where the actual hardware dependent input operation is

performed. Before performing the input operation, the nucleus tests

to see if a character is ready for input. If it is ready, the

nucleus performs the input operation and execution of the process

continues. If a character is not ready, the process must wait. In

a single-user environment under CP/M, the BIOS can simply loop on

console status until a character is read. Under MP/M II, this

technique cannot be used because it consumes the CPU resource. If

the looping process has a high priority, any other lower-priority

processes on the Ready List are denied the CPU resource.

Device polling avoids this situation because the Dispatcher

makes the console status test. If a character is not ready, the

XIOS makes an XDOS Poll call. This suspends the running process on

the system Poll List. Subsequently, in every dispatch operation,

the Dispatcher makes a single console status call for the process.

When the status call indicates a character is ready, the nucleus

removes the process from the Poll List and places it on the Ready

List. Thus device polling is one of the ways a process can wait for

an external or I/O event to occur without monopolizing the CPU

resource.

1.2.5		Console and List Device Management

Console and List devices are special resources under MP/M II.

When the system gives a console or list device to a process, it

internally stores the address of the Process Descriptor, thereby

recording ownership of the device by that process. If another

process attempts to use the device, the nucleus suspends the calling

process and places it on the device's Wait List. It remains on this

list until the process owning the device either terminates execution

or detaches from the device. When this occurs, the nucleus selects

the highest priority waiting process, gives it the device, places it

on the Ready List, and performs a dispatch.

Processes can own more than one console or list device. Fields

within the Process Descriptor designate which device is to be used

in I/O operations. A process gains ownership of a device by a

mechanism called attaching. If a process attaches a device when the

device is free, the process gains ownership of the device.

otherwise, the process is suspended from execution, as described

above. As an option, a process can conditionally attach to a device

in which case it is notified if another process owns the device.

Conditional attachment gives a process more control over its own

execution instead of leaving it up to the nucleus. Thus a process

can avoid being suspended when it does not depend on a specific

device.

MP/M II Programmer's Guide	1.2		MP/M II Nucleus

1,2.6 Memory Management

The MP/M II nucleus can manage from one to eight memory

segments. These segments are of fixed length, and used primarily as

regions for loading transient programs. The partitions are page

aligned, which means that they must begin on a page boundary.

Because a page is defined as 256 bytes, a page boundary always

begins at an address where the low-order byte is 0. The nucleus

manages the memory resource with XDOS functions that allocate and

free memory segments. Figure 1-1 illustrates how memory is

organized under MP/M II.

Top of Memory

+------------+

: MP/M II    :

(Common)   : Operating  :

: System     :

:            :

Top of Banked    :------------:     +--------+     +--------+

Memory      :            :     :////////:     :////////:

: Segment 0  :     :////////:     :////////:

:            :     :////////:     :////////:

:            :     :////////:     :////////:

:            :     :////////:     :////////:

: MP/M II    :     :////////:     :////////:

(Bank Switched)   : Extension  :     :////////:     :////////:

:------------:     :////////:     :////////:

:////////////:     :////////:     :////////:

:////////////:     :////////:     :////////:

:////////////:     :////////:     :////////:

Low               :////////////:     :////////:     :////////:

Memory            :////////////:     :////////:     :////////:

:////////////:     :////////:     :////////:

+------------+     +--------+     +--------+

Bank 0            Bank 1   ....  Bank N

Figure 1-1. MP/M II Memory Organization

The shaded areas represent those regions that can support memory

segments. If bank-switched memory is not used, available memory is

restricted to bank zero. The total number of memory segments, in

addition to their size and bank locations, are system generation

options. Segment 0, however, is a special segment reserved for

system modules and RSPs. It always resides immediately below the

operating system region in bank 0.

In bank-switched systems, the operating system module resides

in common memory. In addition, all Process Descriptors and queues

must reside in the common memory region. Typically, the common

memory size is 16K but the size can vary on systems capable of

switching memory in units smaller than 16K. As a result, the

MP/M II Programmer's Guide	1.2		MP/M II Nucleus

typical maximum memory segment size is 48K. The largest user memory

segment that can be allocated to bank 0 is usually much less than

this value.

More than one memory segment can be defined in a single bank.

Memory segments that do not begin at 0 can only be used to execute

page relocatable (PRL) programs. Memory segments beginning at 0,

can execute COM or PRL programs.

1.2.7		System Timing Functions

MP/M II's system timing functions include: keeping the time of

day, delaying the execution of a process for a specified period of

time, and scheduling programs to be loaded from disk and executed.

The XDOS internal process, CLOCK, provides the time of day for the

system. This process issues Flag Wait calls on the system one

second flag, Flag 2. When the XIOS Interrupt Handler sets this

flag, it wakes up the CLOCK process which then increments the

internal time and date. Subsequently, the CLOCK process makes

another Flag Wait call and suspends itself until the flag is set

again. MP/M II provides functions that allow the user to set and

access the internal date and time. In addition, the BDOS uses the

internal time and date to record when a file is updated, created, or

last accessed.

The XDOS Delay function replaces the typical programmed delay

loop for delaying process execution. The Delay function requires

that a tick be supported in the XIOS and that Flag 1, the system

tick flag, be set every 16 to 20 milliseconds (usually 60 times a

second) . When a process makes a Delay call, it specifies the number

of ticks it is to be suspended from execution. The system maintains

the address of the Process Descriptor for the process on an internal

Delay List along with its current delay tick count. A system

process, TICK, waits on the tick flag and decrements this delay

count on each system tick. When the delay count goes to zero, the

process is removed from the Delay List and placed on the Ready List.

MP/M II can schedule the execution of a transient program or a

Resident System Process only if the Resident System Process, SCHED,

is included at system generation time.

MP/M II Programmer's Guide	1.3		MP/M II Memory Structure

1.3		MP/M II Memory Structure

The memory structure of the MP/M II operating system is shown

in Figure 1-2.

High	     +-------------+  < - - +- - +

Memory     : SYSTEM.DAT  :        :    :

+-------------+        :    :

: TMPD.DAT	 :        :    :

+-------------+        :    :

: USERSYS.STK :        :    :

+-------------+        :    :

: XIOSJMP.TBL :        :    :

+-------------+        :    :

: RESBDOS.SPR :        :    :--- Common

+-------------+        :    :			Memory

: XDOS.SPR	 :        :    :

+-------------+        :    :

: RSP's       :        :    :

+-------------+        :    :

: BNKX:OS.SPR :        :< - +

+-------------+        :

: BNKBDOS.SPR :        :--- Memory Segment 0

+-------------+        :    Bank 0

: BNKXDOS.SPR :        :

+-------------+        :

: TMP.SPR     :        :

+-------------+        :

: BRS'S       :        :

+-------------+        :

: LCKLSTS.DAT :        :

Low	     +-------------+        :

Memory     : CONSOLE.DAT :        :

+-------------+  < - - +

Figure 1-2. MP/M II Memory Structure

The exact memory addresses of each of the memory segments shown

above vary with the MP/M II version and depend on the user

specifications made during the system generation process.

If the host system is bank-switched, the modules above the

BNKXIOS.SPR module must reside in common memory. Common memory is

always accessible no matter what bank is used. The modules below

the BNKXIOS.SPR module must reside in bank 0, which is defined as

the bank of memory active when MP/M II is loaded. The BNKXIOS.SPR

module itself can reside partly in common memory and partly in bank

0. If bank-switching is not used, then all of memory is common.

The memory segments shown in Figure 1-2 are described below.

The SYSTEM.DAT segment contains 256 bytes used by the MP/M II

GENSYS to dynamically configure the system. After loading, the

system uses this area for storage of system data such as submit

MP/M II Programmer's Guide	1.3	MP/M II Memory Structure

flags.  See Section 3.5 for the details of the SYSTEM.DAT segment

The size of the TMPD.DAT segment depends on the number of

consoles specified for the system during the system generation

process. MP/M II supports from 1 to 16 consoles, and associated

with each console is a Terminal Message Processor (TMP) , identified

as TMPO through TMP15. The TMP provides the command line support

for each console. Each console uses 64 bytes within the TMPD.DAT

segment to contain a TMP Process Descriptor. The size of the

USERSYS.STK segment varies according to the number of consoles, as

shown in Table---1-1.

Table 1-1. TMPD.DAT Segment Size

Size Number of Consoles

OOOH No user system stacks 100H 1 to 4 consoles 200H 5 to 7 consoles

The USERSYS.STK segment is included if the user selects the

option to add system call user stacks during system generation. If

included, the system temporarily uses 64 bytes of stack space in

this segment when user programs make BDOS function calls. This

option allows users to run CP/M *.COM files under MP/M II. Some

BDOS function calls, especially console I/O functions, consume more

stack under MP/M II than CP/M. The system allocates space for user

system stacks from the USERSYS.STK segment for each user memory

segment. The size of the USERSYS.STK segment varies according to

the number of memory segments, as shown in Table 1-2.

Table 1-2. USERSYS.STK Segment Size

Size Number of Memory Segments

OOOH No user system stacks 100H 1 to 4 memory segments 200H 5 to 7 memory segments

The XIOSJMP.TBL segment is a copy of the first page of the

BNKXIOS.SPR module. It is required because the system divides the

BDOS module into two sub-modules, RESBDOS.SPR and BANKBDOS.SPR. The

RESBDOS module accesses the BNKXIOS via the XI.OSJMP.TBL module. The

BANKBDOS module accesses the BNKXIOS module directly. The

XIOSJMP.TBL module is 256 bytes in length.

The RESBDOS.SPR segment contains the resident portion of the

BDOS module. The BDOS functions supported by this segment include

those not involved with the BDOS file system such as console and

list I/O. The RESBDOS.SPR segment is approximately OBOOH bytes in

length.

MP/M II Programmer’s Guide	1.3		MP/M II Memory Structure

The XDOS.SPR segment contains the MP/M II nucleus and the

extended disk operating system. This segment is approximately 2300H

bytes in length.

RSPs can use two segments within MP/M II. The first segment

resides in common memory, and exists only if one or more RSPs are

included during system generation. This common memory segment RSP

contains all RSP Process Descriptors and queues. The second segment

named the BRS segment exists in the non-common portion of memory

segment 0. It is present only when one or more banked RSPs are

included during system generation (See Section 1.7).

The BNKXIOS.SPR module contains the user-customized Basic and

Extended I/O System in page- relocatable format (PRL) . It can extend

across the common memory boundary. In general, code supporting the

BDOS file system can reside in bank 0 while code supporting console

and list I/O must reside in common memory. Refer to the MP/M II

System Guide for more information regarding the BNKXIOS module.

The BNKBDOS.SPR module contains the non-resident portion of the

BDOS module. All BDOS functions related to the file system are

supported by this segment. This segment is approximately 2300H

bytes in length.

The BNKXDOS.SPR module contains the non-resident portion of the

XDOS module. This segment will vary in length with MP/M II version.

The TMP.SPR module contains the code for the reentrant Terminal

Message Process. This module is approximately 300H bytes in length.

The BRS segment contains data and code used by banked RSPs that

does not have to be in common memory. Banked RSPs are valuable

because they minimize the common memory requirement.

The LCKLSTS.DAT segment is a special data structure that

maintains a record of open files and locked records on the system.

Each open file and locked record consumes a 10-byte entry in this

segment. The size of this segment is determined by parameters

specified during system generation.

The size of the CONSOLE.DAT segment depends on the number of

consoles specified for the system during the system generation

process. MP/M II supports from 1 to 16 consoles, and associated

with each console is a Terminal Message Processor (TMP) , identified

as TMPO through TMP15. The TMP provides the command line support

for each console. Each console uses 256 bytes within the

CONSOLE.DAT segment to contain the stack and buffers for its TMP.

The code for the TMP's is reentrant and resides within the TMP.SPR

segment.

The remaining memory is available for allocation to user memory

segments. The size, bank location, and number of user memory

segments are system generation options. MP/M II uses these memory

segments to load and execute transient programs.

MP/M II Programmer's Guide	1.4	Terminal Message Process

1.4	Terminal Message Process

The Terminal Message Process (TMP) refers to one of a

collection of XDOS system processes that are associated with the

system consoles. Each system console has its own TMP designated as

TMPO through TMP15. The number of system consoles implemented

depends on the number supported in the XIOS and how many are

specified during system generation. Clearly, the number of system

consoles cannot exceed the number supported in the XIOS. However, a

smaller number than the XIOS supported maximum can be specified

during system generation.

The system maintains the buffers, stack, and local variables

for each TMP in each system console's region of the CONSOLE.DAT

segment. The process descriptors for the TMP's are located in the

TMPD.DAT segment. The code, which is shared by all the TMP's, is a

single re-entrant routine within the TMP.SPR module. Thus, while

each TMP performs the same function for each system console, they

compete with each other as well as with any other running processes

for the CPU resource.

The TMP provides the command line support for system consoles

within MP/M II. This includes displaying the system prompt at the

console:

OA>

and reading the command line. The TMP reads the command line from

one of two sources: the console or a Submit file. Normally, it

reads from the console with the BDOS Read Buffer Input command.

Alternatively, it reads from the $N$.SUB file (N = the console number)

on the MP/M II system disk. This occurs only if the user has

previously entered a submit file at the console with the SUBMIT

facility.

After reading a command line, the TMP performs one of two

actions depending on the type of command entered. If the command

line is a new drive specification:

OA>B:

the TMP issues a BDOS Select Disk call to select the new drive. If

the system supports the newly selected drive, the TMP updates the

drive field of its Process Descriptor, displays the new prompt:

OB>

and waits for the next command line.

If the command is in any other form, the TMP assigns its

console to another system process, the Command Line Interpreter,

(CLI). The TMP then sends the command line along with fields

specifying its default drive, user number, list device and console

number to the CLI with the XDOS Send CLI Command. It then attempts

to attach the console. This suspends the TMP from execution because

MP/M II Programmer's Guide		1.4	Terminal Message Process

it no longer owns the system console. When the console becomes

free, the TMP reissues the prompt and the cycle repeats.

Note: The command level default drive and current user number

are maintained in the TMP Process Descriptor for each system

console. This information is displayed in the system prompt. If an

application program changes the current disk or user number by

making an explicit BDOS call, the TMP Process Descriptor values are

not changed. The USER utility does update the TMP Process

Descriptor user number when it sets the user number to a new value.

To do this, it locates the TMP Process Descriptor associated with

the console and updates its user number field.

1.5		Command Line Interpreter

When the Command Line Interpreter (CLI) receives a command line

sent to it with the XDOS Send Cli Command, it interprets the

command, and initiates the requested transient program or RSP.

Normally, the TMP sends the command line to the CLI. However, other

processes can also use the Send CLI Command function. Also, the

BDOS Program Chain function is implemented internally with the Send

CLI Command. Note: Any process making a Send Cli Command call must

first assign its console to the CLI.

The Send CLI Command function sends the command line to the CLI

by attempting to write the command line message to the system queue,

"CliQ". The command line message contains the current disk, user

number, list device and system console number in addition to the

ASCII command line. The CLIQ is a single message queue with a

length of 129 bytes. If the CLIQ already contains a command line

message, the nucleus suspends the process issuing the Send CLI

Command, and places it on the CLIQ's Enqueue List, where it remains

until the CLI reads the message. Once the CLI reads the message,

the process must compete with any others that may also reside on the

Enqueue List for the opportunity to write its message and regain the

ready state. The process with the highest priority that has been

on the list the longest always goes first.

The CLI accepts command line messages by reading the CLIQ. If

the queue is empty, the CLI is blocked from execution when it issues

the CLIQ read command. In this case, the CLI is suspended on the

CLIQ Dequeue List until another process issues a Send CLI Command,

at which point the CLI is removed from the Dequeue List and placed

on the Ready List. When it gets the CPU resource, the CLI's read

queue operation is completed and it receives the command line

message.

MP/M II Programmer's Guide	1.5	Command Line Interpreter

The command line read by the CLI must be in ASCII and usually

takes the form:

<command> <command tail>

where

<command>		=> {d:}filename{;password} or

=> queuename

<command tail>	=> <file spec> or

=> <file spec><delimiter><file spec>

<file spec>		=> {d:}filename{.typ}{;password}

<delimiter>		=> one or more blanks or a tab or

one of the following: "=,/[]<>"

d:			=> MP/M II drive specification, "A"

through "P"

filename		=> 1 to 8 character file name

typ			=> 1 to 3 character file type

password		=> 1 to 8 character password value

queuename		=> 1 to 8 character queue name of

Resident System Process

Fields enclosed in curly brackets are optional. If there is no

drive specification {d:}, the current default drive is assumed. If

the type field {.typ} is omitted, a type field of all blanks is

implied. If the password field {;password} is omitted, a password

field of all blanks is implied. No type field is included in the

<command> file specification because the CLI assumes either a PRL or

COM type.

After the CLI reads a command line, it performs the following

steps:

1)	It parses the command line to pick up the <command> field.

2)	If there is no drive specification or password field, the

CLI attempts to open a queue named by the command field.

If the queue open is successful, the CLI assumes the queue

belongs to an RSP, and attempts to assign the console to

that RSP. If the RSP name is the same as its queue name,

the console assignment is made. In fact, this is the way a

RSP controls whether or not it receives the console

resource when it is initiated by the CLI. The CLI then

writes the <command tail> message along with the current

disk, user number, list device and system console number to

MP/M II Programmer's Guide	1.5 Command Line Interpreter

the RSP's queue. Because the RSP is typically blocked from

execution because of a queue read from its queue, this

sequence initiates the RSP for execution.

3)	If the command field does not name a RSP queue, indicated by

an unsuccessful queue open or the presence of a drive

specification or password field, the CLI assumes it names a

file residing on the default or specified drive. It then

attempts to open the file, filename.PRL. If the open is

unsuccessful, it tries again with the file, filename.COM.

When the current user number is non-zero and the file to be

opened does not exist under that user number, the BDOS

attempts to open the file under user 0. The open operation

is successful if the file exists under user 0, and has the

system attribute set.

If neither open is successful, and no explicit drive

reference was made the CLI repeats the same sequence on the

MP/M II system drive. (The system drive is designated

during system generation) . The CLI does not make this

second attempt if the system drive was referenced in the

first attempt. In addition, regardless of the file's user

number, only files with the system attribute set are

accepted in the second open sequence.

In all cases, if the file password specified in the

<command> field does not match the password of a file

protected in Read mode, a password error terminates the

CLI's open operation.

4)	If the command file open is successful, the CLI performs

different actions depending on whether the opened file is

of type PRL or COM. For PRL files, the CLI selects the

smallest available memory segment which can fit the PRL the

file. For COM files, the CLI selects the first available

absolute memory segment to load the file. Note: More than

one absolute memory segment can exist in a bank-switched

system.

5)	If no memory segment is available, the program loading by

the CLI is terminated and the system returns an error

message. otherwise, the CLI loads the program into its

selected memory segment beginning at BASE+100H (BASE =

memory segment base address). If the command file is of

the PRL type and the selected memory segment is not

absolute, the CLI performs a relocation operation at this

time (See Section 1.6).

The load operation can be aborted if a read error occurs,

or in the case of COM files, if the selected memory segment

is not large enough to contain the file.

6)	Once the program has been loaded, the CLI initializes the

memory segment base page beginning at BASE+OOOH. The base

page initialization is covered in more detail in Section

2.4.

MP/M II Programmer's Guide	1.5		Command Line Interpreter

7)	Once the base page is initialized, the CLI sets up a Process

Descriptor for the loaded program, and assigns the command

file name to the process. The CLI also sets the current

disk, user number, list device and console number fields of

the Process Descriptor to the values received in the

command line message, and gives the process a 20-byte

stack. It then initiates the transient program with an

XDOS Create Process call. The CLI is then ready to read

the next command line and repeat the cycle.

1.6		Transient Programs

Under MP/M II, a transient program is one that the CLI loads

and initiates. As the name transient implies, the program is not

system resident. The system must load it into an available memory

segment every time it executes.

MP/M !I can execute two types of transient programs. The first

type, absolute programs, must run in an absolute memory segment. An

absolute memory segment is one that has a base address of zero (BASE

= OOOOH). The command files of absolute transient programs are

identified by a file type field of COM. A COM file contains the

absolute memory image of the file beginning at 100H. Thus, the CLI

loads a COM file into memory beginning at 100H. MP/M II COM files

are equivalent to those in CP/M.

The second type of transient program, Page Relocatable Programs

(PRLS), can run in relocatable or absolute memory segments. PRL

command files have a type field of PRL. A PRL file contains three

regions: a 1-page header, a code region, and a relocation bit map.

The header has a field containing the length of the program's code

region and a field specifying the minimum amount of additional data

space required by the program. The CLI uses this information to

select a memory segment for the program. The code region contains

the code and initialized data for the program. The CLI loads the

code region into memory beginning at BASE+100H, where BASE is the

memory segment base address.

The bit map consists of a bit string where each bit corresponds

to a byte in the code region. The first bit corresponds to the

first byte, the one loaded into BASE+100H. Because the bit map

immediately follows the code region in a PRL file, the offset of the

bit map equals the program length value stored in the PRL header.

Each bit equal to 1 identifies the high byte of an address field

that requires relocation. During the program load operation, the

CLI adds the high byte or page offset of the address BASE to the

bytes identified for relocation by the bit map. This dynamically

relocates the program and allows it to run in relocatable memory

segments. PRL's loaded into absolute memory segments require no

relocation. Note: It is not possible to convert a COM file into a

PRL file. However, the reverse operation is possible and is

performed with the utility, PRLCOM (see Section 6.3).

MP/M II Programmer's Guide	1.6		Transient Programs

As part of the program load operation, the CLI initializes the

memory segment base page as follows:

BASE+OOOH : Direct XIOS and program termination jump

BASE+005H : BDOS and XDOS function jump

BASE+050H : Command file drive

BASE+051H : Password address of lst file in the command tail

BASE+053H : Password length of lst file in the command tail

BASE+054H : Password address of 2nd file in the command tail

BASE+056H : Password length of 2nd file in the command tail

BASE+05CH : Parsed FCB for lst file in the command tail

BASE+06CH : Parsed FCB for 2nd file in the command tail

BASE+080H : Command tail

During execution, a transient program makes BDOS or XDOS system

calls by calling BASE+5. Direct XIOS calls are made with the jump

at BASE+OOOH. Note: Direct XIOS calls are restricted to console

and list I/O. All memory within the segment below the address

contained in BASE+6 is available to the transient program. Thus,

transient programs can use this address to size memory. The

remaining information placed into the base page is data parsed out

of the command line. This information is provided as a convenience

to the programmer and is covered in detail in Section 2.

When the CLI initiates a transient program, it assigns a 20

byte stack area to the process. This stack is initialized in such a

way that if the program returns to the system, its execution is

terminated. A process can also terminate execution with a jump to

BASE+OOOH, a BDOS System Reset call, or an XDOS Terminate Process

call.

1.7		Resident System Processes

Resident System Processes (RSPs) are optional processes that

can be included with MP/M II during system generation. There are

two types of RSPs: standard and banked. A standard RSP is a page

relocatable file that has a filetype of RSP. When integrated into

MP/M II, a standard RSP resides in the common memory region. A

banked RSP consists of two page-relocatable files, both of which

have the same filename but have file type fields of RSP and BRS

respectively. When a banked RSP is included in MP/M II, the RSP

file loads into common memory, whereas the BRS file loads into

memory segment 0 in bank 0. Because all Process Descriptors and

queues must reside in common memory, the common module of a banked

RSP contains its Process Descriptor and any additional Process

Descriptors and queues.

The memory segment field of an RSPs Process Descriptor

designates whether the RSP is standard or banked. Standard RSPs set

the memory segment field to FFH; banked RSPs set the field to zero.

When a RSP is selected during the system generation process, GENSYS

checks this field, and if set to 0, includes the BRS file in memory

segment 0.

MP/M II Programmer's Guide	1.7		Resident System Processes

RSPs load into memory as part of the MPMLDR operation, and are

initiated following the XIOS System Initialization call and prior to

the initialization of the TMPs. Once initiated, an RSP runs like

any other process in the system, competing for the CPU and other

system resources on a priority basis.

If a RSP is to be invoked as a built-in command from the

console command line, it must perform the following steps:

1)	Make a queue with a message length sufficiently large to

accept the command tail. The name of the queue is the

command name of the RSP. Because the CLI converts command

lines to upper-case, RSP queue names must be upper-case.

If the CLI is to assign the console to the RSP, the RSP's

Process Descriptor name must be the same as its queue name.

2)	Make an unconditional Read Queue call to the queue. This

suspends the RSP on its queue's Enqueue List until the CLI

writes it a command line message.

3)	Perform its task by making BDOS and XDOS function calls

using the command line message containing the current

drive, user, list device and system console number obtained

from the queue read. Note: An RSP does not make system

calls by calling location 5. The system initializes the

first two bytes of a standard RSP and the first two bytes

of the common module of an extended RSP to contain the

system entry point address. The system sets the first two

bytes of the bank-zero module of an banked RSP to the

beginning address of its corresponding common module.

RSP's must use these addresses to make system calls.

4)	After performing its task, the RSP must make an XDOS Detach

Console call and an XDOS Detach List call if it is assigned

the console by the CLI. It then returns to step 2 and

awaits another command line.

Another special type of RSP is the Resident System Procedure.

A Resident System Procedure provides a method of serially utilizing

a block of code as a system resource. A Resident System Procedure

is set up by a RSP. The process creates a queue with the name of

the Resident System Procedure and sends it a single two-byte message

containing the address of the procedure to be accessed serially.

once this is accomplished, the RSP terminates itself.

The Resident System Procedure is accessed by opening the queue

and reading the two byte message to obtain the actual memory address

of the procedure. Because only one message resides in the queue,

only one process can gain access to the procedure. When the process

leaves the procedure, it writes the message containing the procedure

address back to the queue. This enables the next waiting process to

use the Resident System Procedure.

MP/M	II Programmer's Guide           1.8 BDOS And XDOS Calling Conventions

1.8		BDOS and XDOS Calling Conventions

MP/M II's BDOS and XDOS system functions can be accessed by

both transient programs and RSPs. Transient programs make system

calls via the primary entry point at location BASE+005H, where BASE

equals the base address of the transient program's memory segment.

Standard RSPs obtain the system entry point address from the first

two bytes of the program. For banked RSPs, the first two bytes of

the common module contain the system entry point address. The first

two bytes of the bank-zero module contain the address of the common

module.

MP/M II uses a standard protocol for system function calls. It

is the same protocol used by CP/M. In general, when making a system

call, register C contains the function number, and register pair DE

contains the information address. Functions return single-byte

values in register A, and double-byte values in register pair HL.

Any system call made with an out-of-range or non-supported function

number, returns a 0FFFFH in register pair HL. Note: CP/M returns

with HL set to 0 on invalid function calls. For compatibility,

register A equals L and register B equals H upon return in all

cases. The register passing conventions of MP/M II agree with those

of Intel's PL/M systems programming language.

When entering a transient program, the system sets the stack

pointer to a 10-level stack, with the address contained in BASE+001H

pushed onto the stack. Thus, a return to the system is equivalent

to a jump to BASE+OOOH. Typically, this stack is sufficiently large

to handle system calls. However, most transient programs set up

their own stack and return to the system via a jump to location

BASE+OOOH. Because of the way RSPs are integrated into the system,

they must set up and initialize their own stack.

The programmer should be aware that BDOS and XDOS function

calls do not restore registers to their entry values upon return to

the calling program. The responsibility for saving and restoring

any critical register values rests with the calling process.

As an example, the following transient program illustrates how

to make system calls. This program reads characters continuously

until it encounters an asterisk, at which time it terminates

execution by returning to the system.

ORG	OOOOH

BASE		EQU	$		;BEGINNING OF MEMORY SEGMENT

BDOS		EQU	BASE+0005H	;MP/M II SYSTEM ENTRY POINT

CONIN		EQU	1		;CONSOLE INPUT FUNCTION

;

ORG	100H		;BASE OF TRANSIENT PROGRAM AREA

NEXTC		MVI	C,CONIN	;READ NEXT CHARACTER FUNCTION #

CALL	BDOS		;RETURN CHARACTER IN A

CPI	1*1		;END OF PROCESSING

JNZ	NEXTC		;LOOP IF NOT

RET			;TERMINATE PROGRAM

END

Section 2

The BDOS Interface

2.1		BDOS Console and List I/O Interface

A primary design objective of MP/M II has been to achieve CP/M

compatibility. Thus, from the perspective of the applications

program there are only minor differences between CP/M and MP/M II

with regard to BDOS console and list I/O functions. These

differences are described in Section 2.4, BDOS Function Calls.

Each program executing under MP/M II has a data structure

called a Process Descriptor which defines the characteristics of the

process. One byte of the Process Descriptor identifies the console

and list I/O device numbers currently being used by the process.

The high-order 4 bits of this byte, labeled the CONSOLE/LIST field,

contain the list device number. The low-order 4 bits contain the

console device number. The BDOS console and list I/O functions

obtain the appropriate device number from the CONSOLE/LIST field of

the Process Descriptor to call the XIOS console or list subroutine.

A process must be attached to a console or list device to

access the device. This applies to both BDOS and direct XIOS

function calls. MP/M II intercepts all BDOS and direct XIOS

function calls for the console and list devices to determine if the

specified device is attached to the calling process. The function

call is permitted only if the device is currently unattached, or

attached to the calling process. If the device is attached to some

other process, MP/M II performs an XDOS Attach call for the

specified device. The calling process then blocks, suspending

execution, until the device is detached from the process owning the

device and the calling process is the highest priority process

requiring the device. Attaching a specific device to a process can

be done explicitly by making XDOS Attach Console or Attach List

calls, or implicitly by making BDOS and direct XIOS function calls

which in turn force device attachment.

MP/M II maintains tables of processes currently owning the

console and list devices. These tables contain Process Descriptor

addresses. It is thus possible for one process to own several

console or list devices by having its Process Descriptor address in

several table entries. Multiple devices can be attached by

repeatedly using the XDOS Set Console or Set List Device function

call followed by an XDOS Attach call. Later, when actual I/O is to

be performed, the specific console or list device must be set in the

Process Descriptor by making an appropriate XDOS Set Console or Set

List Device function call.

All console and list devices are detached from a process when

it terminates, allowing processes that were waiting for the devices

to resume execution.

MP/M II Programmer's Guide		2.1 BDOS Console and List I/O

While performing BDOS console I/O functions, there are several

ASCII control characters that cause MP/M II to take specific

actions. The TC character can abort the process owning the console.

The TD character forces the process owning the console to detach

from the console, allowing another waiting process to gain access to

the console, and then attaches the console again before continuing.

The TS and TQ characters are used to stop and re-start console

display output. The TS character will cause console display output

to be suspended. At that point a TQ can be typed to resume console

display output or a TC can be typed to abort the process owning the

console. Typing any other key when output has been suspended will

cause MP/M II to send the ASCII Bell character (TG) to the console.

2.2		BDOS File System

The Basic Disk Operating System (BDOS) supports a named file

system on one to sixteen logical drives. Each logical drive is

divided into two regions: a directory area and a data area. The

directory area defines the files that exist on the drive and

identifies the data area space that belongs to each file. The data

area contains the file data defined by the directory. The directory

area is subdivided into sixteen logically independent directories,

each identified by user numbers 0 through 15. In general, only

files belonging to the current user number are "visible" in the

directory. For example, the MP/M II DIR utility only displays files

belonging to the current user number.

The BDOS file system automatically allocates directory and data

area space when a file is created or extended and returns previously

allocated space to free space when a file is deleted. If no

directory or data space is available for a requested operation, the

BDOS returns an error to the calling process. These actions are

transparent to the calling process. As a result, the user does not

need to be concerned with directory and drive organization when

using the file system functions.

An eight-character filename field and a three character type

field identifies each file in a directory. An eight character

password can also be assigned to a file to protect it from

unauthorized access. All BDOS functions that involve file

operations specify the requested file by the filename and type

fields. Multiple files can be specified by an ambiguous reference.

An ambiguous reference uses one or more I'?" marks in the name or

type field to indicate that any character matches that position.

Thus, a name and type specification of all "?'"s (equivalent to a

command line file specification of "*.*") matches all files in the

directory that belong to the current user number.

The BDOS file system supports four categories of functions:

file access functions, directory functions, drive related functions,

and miscellaneous functions. The file access category includes

functions to make (create) a new file, open an existing file and

close an existing file. Both the make and open functions activate

the file for subsequent access by read and write functions. After a

MP/M II Programmer's Guide	2.2		BDOS File System

file has been opened, subsequent BDOS functions can read or write to

the file, either sequentially or randomly by record position. BDOS

read and write commands transfer data in 128 byte logical units,

which is the basic record size of the file system. The close

function performs two steps to terminate access to a file. First,

it indicates to the file system that the calling process has

finished accessing the file. The file then becomes available to

other processes. In addition, the function makes any necessary

updates to the directory to permanently record the current status of

the file.

BDOS directory functions operate on existing file entries in a

drive's directory. This category includes functions to search for

one or more files, delete one or more files, rename a file, set file

attributes, assign a password to a file, and compute the size of a

file. The BDOS search and delete functions are the only functions

that allow ambiguous file references. All other directory and file

related functions require a specific file reference. The BDOS file

system does not allow a process to delete, rename, or set the

attributes of a file that is currently opened by another process.

BDOS drive-related functions include those which select a drive

as the default drive, compute a drive's free space, interrogate

drive status and assign a directory label to a drive. The directory

label for a drive controls whether file passwords are to be honored,

and the type of date and time stamping to be performed for files on

the drive. Also included in this category are functions to reset

specified drives and to control whether other processes can reset

particular drives. When a drive is reset, the next operation on the

drive reactivates it by logging it in. The function of the log-in

operation is to initialize the drive for file and directory

operations. Under MP/M II, a successful drive reset operation must

be performed on drives that support removeable media before changing

disks.

Miscellaneous functions include those that set the current DMA

address, access and update the current user number, chain to a new

program, and flush the internal blocking/deblocking buffer. Also

included are functions to set the BDOS multi-sector count and the

BDOS error mode. The BDOS multi-sector count determines the number

of 128-byte records to be processed by BDOS read, write, record

lock, and record unlock functions. It can range from one to sixteen

128-byte records; the default value is one. The BDOS error mode

determines whether the BDOS file system intercepts errors or returns

all errors to the calling process.

MP/M II Programmer's Guide	2.2		BDOS File System

The following list summarizes the operations performed by the

BDOS file system:

Disk System Reset

Drive Selection

File Creation

File Open

File Close

Directory Search

File Delete

File Rename

Random or Sequential Read

Random or Sequential Write

Interrogate Selected Disks

Set DMA Address

Set/Reset File Indicators

Reset Drive

Access/Free Drive

Random Write With Zero Fill

Lock and Unlock Record

Set Multi-Sector Count

Set BDOS Error Mode

Get Disk Free Space

Chain To Program

Flush Buffers

Set Directory Label

Return Directory Label

Read and Write File XFCB

Set/Get Date and Time

Set Default Password

Return BDOS Serial Number

The following sections contain information on important topics

related to the BDOS file system. The reader should be familiar with

the content of these sections before attempting to use the BDOS

functions described individually in Section 2.4.

2.2.1		File Naming Conventions

Under MP/M II, filenames consist of four parts: the drive

select code (d), the filename field, the file type field, and the

file password field. The general format for a command line file

specification is shown below:

{d:}filename{.typ} {;password}

The drive select code field specifies the drive where the file is

located. The filename and type fields identify the file. The

password field specifies the password if a file is password

protected.

MP/M II Programmer's Guide	2.2.1		File Naming Conventions

The drive, type, and password fields are optional and the

delimiters " : . ; " are required only when specifying their associated

field. The drive select code can be assigned a value from "A" to

"P" where the actual drive codes supported on a given system is

determined by the XIOS implementation. When the drive code is not

specified, the current default drive is indicated. The filename

field can contain one to eight non-delimiter characters, the file

type field, one to three non-delimiter characters, and the password

field, one to eight non-delimiter characters. All alphabetic

characters must be in uppercase. In addition, all three fields are

padded with blanks, if necessary. Omitting the optional type or

password fields implies a field specification of all blanks.

The MP/M II Parse Filename function recognizes certain ASCII

characters as valid delimiters when it parses a file from a command

line. The valid characters are shown in Table 2-1.

Table 2-1. Valid Filename Delimiters

| ASCII | HEX EQUIVALENT |
| --- | --- |
| : . ; = , / [ ] < > | 3A 2E 3B 3D 2C 2F 5B 5D 3C 3E |

The Parse Filename function also excludes all control characters

from the file fields and translates all lower-case letters to upper

case.

The characters "(" and ")" should be avoided in filename and

type fields because they are commonly used delimiters. The

characters "*" and "?" must not be used in filename and type fields

unless they are used to make an ambiguous reference. If the Parse

Filename function encounters a "*" in a file name or type field, it

pads the remainder of the field with "?" marks. For example, a

filename of "X*.*" is parsed to "X???????.???". The BDOS search and

delete functions treat a "?" in the filename and type fields as

follows: A "?" in any position matches the corresponding field of

any directory entry belonging to the current user number. Thus, a

search operation for "X???????.???" finds all the current user files

on the directory beginning in "X". Most other file related BDOS

functions treat the presence of a "?" in the filename or type field

as an error.

It is not mandatory to follow the file naming conventions of

MP/M II when creating or renaming a file with BDOS functions.

However, the conventions must be used if the file is to be accessed

from a command line. For example, the CLI cannot locate a command

MP/M II Programmer's Guide	2.2.1		File Naming Conventions

file in the directory if its filename or type field contains a

lower-case letter.

As a general rule, the file type field names the generic

category of a particular file, while the filename distinguishes

individual files in each category. Although they are generally

arbitrary, the file types listed below name some of the generic

categories that have been established.

ASM		Assembler Source		PLI	PL/I Source File

PRN		Printer Listing		REL	Relocatable Module

HEX		Hex Machine Code		TEX	TEX Formatter Source

BAS		Basic Source File		BAK	ED Source Backup

INT		Intermediate File		SYM	SID Symbol File

COM		Command File		$$$	Temporary File

PRL		Page Relocatable		RSP	Resident Sys. Process

SPR		Sys. Page Reloc.		SYS	System File

DAT		Data File			BRS	Banked RSP File

2.2.2		Disk Drive and File Organization

The BDOS file system can support from one to sixteen logical

drives. The maximum file size supported on a drive is 32 megabytes.

The maximum capacity of a drive is determined by the data block size

specified for the drive in the XIOS. The data block size is the

basic unit in which the BDOS allocates disk space to files. Table

2-2 displays the relationship between data block size and drive

capacity.

Table 2-2. Logical Drive Capacity

| Data Block Size | Maximum Drive Capacity |
| --- | --- |
| 1K 2K 4K 8K 16K | 256 Kilobytes 64 Megabytes 128 Megabytes 256 Megabytes 512 Megabytes |

Logical drives are divided into two regions: a directory area

and a data area. The directory area contains from one to sixteen

blocks located at the beginning of the drive. The actual number is

set in the XIOS. This area contains entries that define which files

exist on the drive. The directory entries corresponding to a

particular file define which data blocks in the drive's data area

belong to the file. These data blocks contain the file's records.

The directory area is logically subdivided into sixteen independent

directories identified as user 0 through 15. Each independent

directory shares the actual directory area on the drive. However, a

file's directory entries cannot exist under more than one user

number. In general, only files belonging to the current user number

are visible in the directory.

MP/M II Programmer’s Guide		2.2.2	Disk Drive and Organization

Each disk file consists of a set of up to 242,144 128-byte

records. Each record in a file is identified by its position in the

file. This position is called the record's random record number.

If a file is created sequentially, the first record has a position

of zero, while the last record has a position one less than the

number of records in the file. Such a file can be read sequentially

in record position order beginning at record zero, or randomly by

record position. Conversely, if a file is created randomly, records

are added to the file by specified position. A file created in

this way is called "sparse" if positions exist within the file where

a record has not been written.

The BDOS automatically allocates data blocks to a file to

contain its records on the basis of the record positions consumed.

Thus, a sparse file that contains two records, one at position zero,

the other at position 242,143, would consume only two data blocks in

the data area. Sparse files can only be created and accessed

randomly, not sequentially. Note that any data block allocated to a

file is permanently allocated to the file until the file is deleted.

There is no other mechanism supported by the BDOS for releasing data

blocks belonging to a file.

Source files under MP/M are treated as a sequence of ASCII

characters, where each "line" of the source file is followed by a

carriage-return line-feed sequence (ODH followed by OAH). Thus a

single 128-byte record could contain several lines of source text.

The end of an ASCII file is denoted by a control-Z character (1AH)

or a real end of file, returned by the BDOS read operation.

Control-Z characters embedded within machine code files such as COM

or PRL files are ignored. The end of file condition returned by

BDOS is used to terminate read operations.

2.2.3		File Control Block Definition

The File Control Block (FCB) is a data structure used with the

BDOS file access and directory functions. All of these functions

reference an FCB to determine the file or files to be operated on.

Certain fields in the FCB are also used for invoking special options

associated with some functions. other functions use the FCB to

return data to the calling process. Most importantly, when a

process opens a file and subsequently accesses it with read, write,

lock, and unlock record functions, the BDOS file system maintains

the current file state and position within the user's FCB. In

addition, all BDOS random I/O functions specify the random record

number with a 3-byte field at the end of the FCB.

When making a file access or directory BDOS function call,

register pair DE must address a FCB. The length of the FCB data

area depends on the BDOS function. For most functions, the required

length is 33 bytes. For random I/O functions and the Compute File

Size function, the FCB length must be 36 bytes. When either the

BDOS Open or Make File functions specify a file is to be opened in

unlocked mode, the FCB must be 35 bytes in length. The FCB format

is shown on the next page.

MP/M II Programmer's Guide	2.2.3		File Control Block Definition

----------------------------------------------------------------

:dr:fl:f2: ... :f8:tl:t2:t3:ex:sl:s2:rc:d0: ... :dn:cr:r0:rl:r2:

----------------------------------------------------------------

00 01 02  ...  08 09 10 11 12 13 14 15 16  ...  31 32 33 34 35

where

dr		drive code (0 - 16)

0 => use default drive for file

1 => auto disk select drive A,

2 => auto disk select drive B,

. . .

16 => auto disk select drive P.

fl . . .f8	contain the file name in ASCII

upper case, with high bit = 0.

f1’. . . f8' denote the high

order bit of these positions,

and are file attribute bits.

tl,t2,t3	contain the file type in ASCII

upper case, with high bit = 0.

t1’, t2’ and t3’denote the

high bit of these positions,

and are file attribute bits.

tl' = 1 => Read/Only file

t2' = I => System file

t3’ = 1 => File has been archived

ex		contains the current extent number,

normally set to 0 by the calling process, but

can range 0 - 31 during file I/O

cs		contains the FCB checksum value for

open FCBs.

rs		reserved for internal system use, set

to zero on call to OPEN, MAKE, SEARCH

rc		record count for extent "ex"

takes on values from 0 – 128

dO	dn	filled-in by MP/M, reserved for

system use

cr		current record to read or write in

a sequential file operation, normally

set to zero by the calling process when a

file is opened or created

rO,rl,r2	optional random record number in the

range 0-242,143 (0 - 3FFFFH).

ro,rl,r2 constitute a 18 bit value

with low byte rO, middle byte rl, and

high byte r2.

MP/M II Programmer's Guide		2.2.3	File Control Block Definition

Note : The 2-byte File ID is returned in bytes rO and rl when a

file is successfully opened in unlocked mode (see Section 2.2.8).

For BDOS directory functions, the calling process must

initialize bytes 0 through 11 of the FCB before issuing the function

call. The Set Directory Label and Write File XFCB functions also

require the calling process to initialize byte 12. The BDOS Rename

File function requires the calling process to place the new file

name and type in bytes 17 through 27.

BDOS open or make function calls require the calling process to

initialize bytes 0 through 12 of the FCB before issuing a file open

or make function call. Normally, byte 12 is set to zero. In

addition, if the file is to be processed from the beginning using

sequential read or write functions, byte 32 (cr) must be zeroed.

After an FCB is activated by an open or make operation, the FCB

should not be modified by the user. Open FCBs are checksum verified

to protect the integrity of the file system. In general, if a

process modifies an open FCB, the next read, write, or close

function call will return with a checksum error. See Section 2.2.9

for more on FCB checksums. Normally, sequential read or write

functions do not require initialization of an open FCB. However,

random I/O functions require that a process set bytes 33 through 35

to the requested random record number prior to making the function

call.

File directory elements maintained in the directory area of

each disk drive have the same format as FCB's (excluding bytes 32

through 35), except for byte 0 which contains the file's user

number. Both the Open File and Make File functions bring these

elements (excluding byte 0) into memory in the FCB specified by the

calling process. All read and write operations on a file must

specify an FCB activated in this manner. Otherwise, a checksum

error is returned. The BDOS updates the memory copy of the FCB

during file processing to maintain the current position within the

file. During file write operations, the BDOS updates the memory

copy of the FCB to record the allocation of data to the file, and at

the termination of file processing, the Close File function

permanently records this information on disk. Note that data

allocated to a file during file write operations is not completely

recorded in the directory until the calling process issues a

Close File call. Therefore, it is mandatory that a process which

creates or modifies files, close the files at the termination of any

write processing, otherwise, data may be lost.

As a general rule under MP/M II, a process should close files

as soon as they are no longer needed, even if they have not been

modified. The BDOS file system maintains an entry in the system

lock list (LCKLSTS.DAT memory segment) for each file opened by each

process on the system. This entry is not removed from the system

lock list until the file is closed or the process owning the entry

terminates. The BDOS file system uses this entry to prevent other

processes from accessing the file unless the file was opened in a

mode that supports shared access. Normally, a process must close a

file before other processes on the system can access the file.

MP/M II Programmer's Guide		2.2.3	File Control Block Definition

Keep in mind that the space in the system lock list is a

limited resource under MP/M II. If a process attempts to open a

file and no space exists in the system lock list, or the process

exceeds the process open file limit (specified during system

generation), the BDOS denies the open operation and usually aborts

the calling process.

The high-order bits of the FCB filename (fl',...,f8’) and type

(tl',t2',t3') fields are called attribute bits. Attributes bits are

1 bit boolean fields where 1 indicates on or true, and 0 indicates

off or false. Attribute bits have two functions within the file

system: as file attributes and interface attributes.

The file attributes (fl',...,f4' and tl',t2',t3') are used to

indicate that a file has a defined attribute. These bits are

recorded in a file's directory FCBs. File Attributes can only be

set or reset by the BDOS Set File Attributes function. When the

BDOS Make File function creates a file, it initializes all file

attributes to zero. A process can interrogate file attributes in an

FCB activated by the BDOS Open File function or in directory FCBs

returned by the BDOS Search For First and Search For Next functions.

Note: the BDOS file system ignores the file attribute bits when it

attempts to locate a file in the directory.

The file attributes (tl’,t2',t3') are defined by the file

system as follows:

t1’:	Read/Only attribute - The file system prevents write

operations to a file with the read/only attribute set.

t2’:	System Attribute - This attribute, if set, identifies the file

as a MP/M II system file. System files are not normally

displayed by the MP/M II DIR utility. In addition, user zero

system files can be accessed on a read/only basis from other

user numbers (see Section 2.2.8).

t3’:	Archive Attribute - This attribute is designed for user

written archive programs. When a archive program copies a

file to backup storage, it sets the archive attribute of the

copied files. The file system automatically resets the

archive attribute of a directory FCB that has been issued a

write command. The archive program can test this attribute

in each of the file's directory FCBs via the BDOS Search and

Searchn functions. If all directory FCBs have the archive

attribute set, it indicates that the file has not been

modified since the previous archive. Note that the MP/M II

PIP utility supports file archival.

Attributes fl' through f8’ are available for definition by the user.

The interface attributes are f5' through f8'. These attributes

cannot be used as file attributes. Interface attributes f5’ and f6’

are used to request options for BDOS calls requiring an FCB address

in register pair DE. They are used by the BDOS Open, Make, Close,

MP/M II Programmer's Guide		2.2.3	File Control Block Definition

and Delete File functions. Table 2-3 shows the f5’ and f6’

interface attribute definitions for these functions.

Table 2-3. BDOS Interface Attributes

| open function Make function Close function Delete function | f5’ = 1 : Open in unlocked mode f6’ = 1 : Open in read/only mode f5’ = 1 : Open in unlocked mode f6’ = 1 : Assign password to file f5' = 1 : Partial Close f5' = 1 : Delete file XFCBs only |
| --- | --- |

The interface attributes are discussed in detail for each of the

above functions in Section 2.4. Attributes f5’ and f6’ are always

reset when control is returned to the calling process. Interface

attributes f7' and f8' are reserved for internal use by the BDOS

file system.

The BDOS search and delete functions allow multiple file

(ambiguous) reference. In general, a ? mark in the filename, type,

or extent field matches any value in the corresponding positions of

directory FCBs during a directory search operation. The BDOS search

functions also recognize a ? mark in the drive code field, and if

specified, they return all directory entries on the disk regardless

of user number including empty entries. A directory FCB beginning

with E5H is an empty directory entry.

2.2.4		User Number Conventions

The MP/M II User facility divides each drive directory into

sixteen logically independent directories, designated as user 0

through user 15. Physically, all user directories share the

directory area of a drive. In most other aspects, however, they are

independent. For example, files with the same name can exist on

different user numbers of the same drive with no conflict. However,

a single file cannot reside under more than one user number.

Only one user number is active for a process at one time, and

the current user number applies to all drives on the system.

Furthermore, the FCB format does not contain any field that can be

used to override the current user number. As a result, all file and

directory operations reference directories associated with the

current user number. However, it is possible for a process to

access files on different user numbers by setting the user number to

the file's user number with the BDOS Set User command prior to

issuing the desired BDOS function call for the file. Note that this

technique must be used carefully. If a process attempts to read or

write to a file under a user number that is not the same as the user

number that was active when the file was opened, the BDOS file

system returns a FCB checksum error.

MP/M II Programmer's Guide	2.2.4		User Number Conventions

When the CLI initiates a transient program or RSP, its user

number is set to the value established by the process issuing the

XDOS Send Cli Command. Normally, the sending process is the TMP.

However, the sending process may be another process such as a

transient program that makes a BDOS Chain Program call. A transient

program can change its user number by making a BDOS set user call.

Changing the user number in this way does not affect the command

line user number displayed by the TMP. Thus, when a transient

program that has changed its user number terminates, the original

user number for the console is restored when the TMP regains

control.

User 0 has special properties under MP/M II. With some

restrictions, the file system automatically opens a file under user

zero, if it is not present under the current user number. Of

course, this action is only performed when the current user number

is not zero. In addition, a file on user zero must have the system

attribute (t2') set to be eligible for this operation. This

procedure allows utilities that may include overlays and any other

commonly accessed files to be placed on user zero, but be available

for access from other user numbers. As a result, it eliminates the

need for copying commonly needed utilities to all user numbers on a

directory, and gives the MP/M II user control over which user zero

files are directly accessible from other user numbers. Refer to

Section 2.2.8 for more information on this topic.

2.2.5 Directory Labels and XFCBs

The BDOS file system includes two special types of FCB's, the

XFCB and the Directory Label. The XFCB is an "extended" FCB that

can optionally be associated with a file in the directory. if

present, it contains the file's password field and date and time

stamp information. The format of the XFCB is shown below:

XFCB FORMAT

-----------------------------------------------------

:dr: file : type :pm:sl:s2:rc: password : tsl : ts2 :

:---------------------------------------------------:

:00  01. .  09. . 12 13 14 15  16. . .    25.   29. :

-----------------------------------------------------

dr	drive code (0 - 16)

file	file name field

type	file type field

pm	password mode

bit 7 - Read mode

bit 6 - Write mode

bit 5 - Delete mode

**	- bit references are right to left,

relative to 0

sl,s2,rc 	- reserved for system use

password	- 8 byte password field (encrypted)

tsl		- 4 byte creation or access time stamp field

ts2		- 4 byte update time stamp field

MP/M II Programmer's Guide		2.2.5	Directory Labels and XFCBs

An XFCB can be created for a file in two ways: automatically,

as part of the BDOS Make function or explicitly, by the BDOS

function, Write File XFCB. The BDOS file system does not

automatically create an XFCB for a file unless a Directory Label is

present on the file's drive. The BDOS Read File XFCB function

returns a file's XFCB if it exists in the directory. Note that in

the directory, an XFCB is identified by a drive byte value (byte 0

in the FCB) equal to 16 + N, where N equals the user number.

The Directory Label specifies for a drive if passwords for

password protected files are to be required, if date and time

stamping for files is to be performed, and if XFCBs are to be

created automatically for files by the Make function. The format of

the Directory Label is similar to that of the XFCB as shown below:

DIRECTORY LABEL FORMAT

----------------------------------------------------

:dr: name : type :dl:sl:s2:rc: password : ts : ts2 :

: ---------------------------------------------------:

:00   01. . 09 .. 12 13 14  15  16....    25.  29.      :

: ---------------------------------------------------:

dr      -	drive code (0 - 16)

name    -	Directory Label name

type    -	Directory Label type

dl      -	Directory Label data byte

bit 7 - require passwords for files

bit 6 - perform access time stamping

bit 5 - perform update time stamping

bit 4 - Make creates XFCBs

bit 0 - Directory Label exists

**	- bit references are right to left,

relative to 0

sl,s2,rc -	n/a

password -	8 byte password field (encrypted)

tsl	   -	4 byte creation time stamp field

ts2	   -	4 byte update time stamp field

Only one Directory Label can exist in a drive's directory. The

Directory Label name and type fields are not used to search for a

Directory Label in the directory; they can be used to identify a

diskette or a drive. A Directory Label can be created or its fields

can be updated by the BDOS function, Set Directory Label. This

function can also assign a Directory Label a password. The

Directory Label password, if assigned, cannot be circumvented,

whereas file password protection is an option controlled by the

Directory Label. Thus, access to the Directory Label password

provides a kind of super-user status for that drive.

Note: The BDOS file system provides no function to read the

Directory Label FCB directly. However, the Directory Label data

byte can be read directly with the BDOS function, Return Directory

Label. In addition, the BDOS Search functions (‘?' in FCB drive

MP/M II Programmer's Guide	2.2.5 Directory Labels and XFCBs

byte) can be used to find the Directory Label on the default drive.

In the directory, the Directory Label is identified by a drive byte

value (byte 0 in the FCB) equal to 32 (20H).

2.2.6 File Passwords

Files may be assigned passwords in two ways: by the Make File

function if the Directory Label specifies automatic creation of

XFCBs or by the Write File XFCB function. A file's password can

also be changed by the Write File XFCB function if the original

password is supplied. However, a file's password cannot be changed

without the original password even when password protection for the

drive is disabled by the Directory Label.

Password protection is provided in one of three modes. Table

2-4 shows the difference in access level allowed to BDOS functions

when the password is not supplied.

Table 2-4. Password Protection Modes

| Password Mode | Access level allowed when the password is not supplied |
| --- | --- |
| - Read - Write - Delete | The file cannot be read The file can be read but not modified. The file can be modified but not deleted. |

If a file is password protected in Read mode, the password must be

supplied to open the file. A file protected in Write mode cannot be

written to without the password. A file protected in Delete mode

allows read and write access, but the user must specify the password

to delete the file, rename the file, or to modify the file's

attributes. Thus, password protection in mode 1 implies mode 2 and

3 protection, and mode 2 protection implies mode 3 protection. All

three modes require the user to specify the password to delete the

file, rename the file, or to modify the file's attributes.

If the correct password is supplied, or if password protection

is disabled by the Directory Label, then access to the BDOS

functions is the same as for a file that is not password protected.

In addition, the Search For First and Search For Next functions are

not affected by file passwords.

MP/M II Programmer's Guide			2.2.6  File Passwords

Table 2-5 lists the BDOS functions that test for password.

Table 2-5. BDOS Functions That Test For Password

15. Open File 19. Delete File 23. Rename File 30. Set File Attributes 100. Set Directory Label 103. Write File XFCB

File passwords are eight bytes in length. They are maintained

in the XFCB and Directory Label in encrypted form. To make a BDOS

function call for a file that requires a password, a process must

place the password in the first eight bytes of the current DMA or

specify it with the BDOS function, Set Default Password, prior to

making the function call. Note: the BDOS maintains the assigned

default password on a system console basis and retains it across

process termination.

2.2.7		File Date and Time Stamps

The BDOS file system can record when a file was created or last

accessed, and/or last updated. It records the creation stamp only

when an XFCB is automatically created by the Make File function. If

an XFCB is created by the Make File XFCB function, the creation

stamp is set to zero. The Close File function makes the update

stamp if a write operation is made to the file while the file is

open. The Open File function makes the access stamp if the file is

successfully opened. The creation date stamp is overwritten when

access stamping is performed because only two date and time fields

reside in the XFCB and the access and creation time stamps share the

same field.

The drive's Directory Label determines the type of date and

time stamping supported for files on a drive. If a drive does not

have a Directory Label, or if it is read/only, or if the drive's

directory label does not specify date and time stamping, then no

date and time stamping for files is performed. In addition, a file

must have an XFCB to be eligible for date and time stamping. For

the Directory Label itself, time stamps record when it was created

and last updated. No access stamping for Directory Labels is

supported.

A process can directly access the date and time stamps for a

file by using the Read File XFCB function. No mechanism is provided

to directly update XFCB date and time fields.

The BDOS file system uses the MP/M internal date and time when

it records a date and time stamp. On MP/M II systems that do not

support a clock, date and time stamps record the last initialized

value for the system date and time. The MP/M II TOD utility can be

used to set the system date and time.

MP/M II Programmer's Guide	2.2.8		File open Modes

2.2.8		File Open Modes

The BDOS file system provides three different modes of opening

files. They are defined as follows:

locked mode:

A process can open a file in locked mode only if the file is

not currently opened by another process. once open in locked

mode, no other process can open the file until it is closed.

Thus, if a process successfully opens a file in locked mode,

that process in effect owns the file until the file is closed

or the process terminates. Files opened in locked mode

support read and write operations unless the file is a

read/only file (attribute tl' set) or the file is password

protected in Write mode and the password is not supplied with

the BDOS Open File call. In both of these cases, only read

operations to the file are allowed. Note: locked mode is

the default mode for opening files under MP/M II.

unlocked mode:

A process can open a file in unlocked mode if the file is not

currently open, or if the file has been opened by another

process in unlocked mode. This mode allows more than one

process to open the same file. Files opened in unlocked mode

support read and write operations unless the file is a

read/only file (attribute tl' set) or the file is password

protected in Write mode and the password is not supplied with

the BDOS Open File call. However, when a file opened in

unlocked mode is extended by a write operation, the BDOS

allocates space to the file in data block units, not in 128

byte record units as is normally the case. The BDOS record

locking and unlocking functions are only supported for files

opened in unlocked mode.

When opening a file in unlocked mode, a process must reserve

35 bytes in the FCB, because the Open File function returns a

2-byte value called the File ID in the rO and rl bytes of the

FCB. The File ID is a required parameter for the BDOS record

lock and record unlock commands.

read/only mode:

A process can open a file in read/only mode if the file is

not currently opened by another process, or the file has been

opened by another process in read/only mode. This mode

allows more than one process to open the same file for

read/only access.

The open function performs the following steps for files opened

in locked or read/only mode. If the current user is non-zero, and

the file to be opened does not exist under the current user number,

MP/M II Programmer's Guide	2.2.8		File Open Modes

the open function searches user zero for the file. If the file

exist, under user zero and the file has the system attribute (t2')

set, the file is opened under user zero. The open mode is

automatically forced to read/only when this is done. For more

information on this, refer to Section 2.2.4.

The open function also performs the following action for files

opened in locked mode when the current user number is zero. If the

file exists under user zero and has the system (t2’) and read/only

(tl') attributes set, the open mode is automatically set to

read/only. Thus, the read/only attribute controls whether a user

zero system file can be concurrently opened by a user-zero process

and processes on other user numbers when each process opens the file

in the default locked mode. If the read/only attribute is set, all

processes open the file in read/only mode and concurrent access of

the file is allowed. However, if the read/only attribute is reset,

the user-zero process opens the file in locked mode. if it

successfully opens the file, no other process can open it. if

another process has the file open, its open operation is denied.

Table 2-6 shows the definition of the FCB interface attributes

f5' and f6’ for the BDOS Open File function.

Table 2-6. FCB Interface Attributes F5’ F6’

Open File Function

f5' = 0, f6’ = 0 - open in locked mode (default mode) f5' = 1, f6’ = 0 - open in unlocked mode f5' = 0 or 1, f6’ = 1 - open in read/only

Interface attribute f5’ designates the open mode for the BDOS Make

File function. Table 2-7 shows the definition of the FCB interface

attribute f5' for the Make File function.

Table 2-7. FCB Interface Attribute F6’

Make Function

f5' = 0 - open in locked mode (default mode) f5’ = 1 - open in unlocked mode

Note: the Make File function does not allow opening the file in

read/only mode.

2.2.9		File Security

In general, the security measures implemented in the BDOS file

system are intended to prevent accidental collisions between running

processes. It is not possible to provide total security under MP/M

II because the BDOS file system maintains file allocation

information in open FCBs in the user's memory region, and MP/M II

does not support memory protection. In the worst case, a program

MP/M II Programmer's Guide	2.2.9		File Security

that "crashes" on MP/M II can take down the entire system.

Therefore, MP/M II requires that all processes running on the system

be "friendly." However, the BDOS file system is designed to ensure

that multiple processes can share the same file system without

interfering with each other. It does this in two ways:

•	it performs checksum verification of open FCB's.

•	it monitors all open files and locked records via the system

lock list (LCKLSTS.DAT).

User FCBs are checksum validated before I/O operations to

protect the integrity of the file system from corrupted FCBs. The

Open File and Make File functions compute and assign checksums to

FCBs. The Read, Write, Lock Record, Unlock Record and Close File

functions subsequently verify and recompute the checksums when the

FCB changes. If the BDOS detects an FCB checksum error, it does not

perform the requested command. Instead, it either terminates the

calling process with an error, or if the process is in BDOS return

error mode (see Section 2.2.13), it returns to the process with an

error code.

The system lock list is established during the system

generation process at which time the user can establish the size of

the list and also define limits for the number of files a single

process can open and the number of records a single process can

lock. Each time a process opens a file or locks a record

successfully, the BDOS file system allocates an entry in the system

lock list to record the fact. The file system uses this information

to:

•	prevent a process from deleting, renaming, or updating the

attributes of another process's open file.

•	prevent a process from opening a file currently opened by

another process unless both processes open the file in locked

or read/only mode.

•	prevent a process from resetting a drive on which another

process has an open file.

•	prevent a process from locking or updating a record currently

locked by another process. Refer to Section 2.2.10 for more

information on record locking and unlocking.

For reasons of efficiency, the file system verifies only for certain

functions whether another process has the FCB specified file open.

These functions are: Open File, Make File, Delete File, Rename

File, and Set File Attributes. For open FCBs, the FCB checksum

controls whether the process can use the FCB. By definition, a

valid FCB checksum implies that the file has been successfully

opened and an entry for the file resides in the system lock list.

When a process closes a file permanently, the file system removes

the file from the system lock list and invalidates its FCB checksum

field.

MP/M II Programmer's Guide	2.2.9		File Security

There are several other situations where the file system

removes open file entries from the system lock list for a process.

For example, if a process makes a delete call for a file that it has

open in locked mode, the file system deletes the file and also

removes the file's entry from the system lock list. Deleting an

open file is not recommended practice under MP/M but is supported

for files opened in locked mode (the default open mode) , to provide

compatibility with software written under earlier releases of MP/M

and CP/M. Note that the file system does not delete a file opened

in unlocked or read/only mode.

To ensure that the process does not use the FCB corresponding

to the deleted file, the file system subsequently checks all open

FCBs for the process to ensure that a lock list item exists for the

FCB. Each open FCB is checked the next time it is used. If a lock

list entry exists for the file, the operation is allowed to proceed.

Otherwise, a FCB checksum error is returned.

The file system performs this verification of open FCBs for all

situations where it purges an open file entry from the system lock

list. The following list describes these situations:

•	A process deletes a file it has open in locked mode.

•	A process renames a file it has open in locked mode.

•	A process updates the attributes via the BDOS Set File

Attributes command of a file it has open in locked mode.

•	A process issues a Free Drive call for a drive on which it has

an open file.

•	A change in media is detected on a drive that has open files.

This situation is a special case because a process cannot

control whether it occurs and it can impact more than one

process. Refer to Section 2.2.13 for more information on this

situation.

The automatic verification of open FCBs by the file system after

it purges a file entry from the system lock list can affect

performance. Each verification requires a directory search

operation. Therefore, it is strong y recommended that these

situations be avoided in new programs developed for MP/M II.

2.2.10		Concurrent File Access

More than one process can access the same file if each process

opens the file in the same shared access mode. BDOS supports two

shared access modes, unlocked and read/only. Read/only mode is

functionally identical to the default locked mode except that more

than one process can access the file and no process can change it.

MP/M II Programmer's Guide	2.2.10		Concurrent File Access

Files opened in unlocked mode present a more complex situation

because a file opened in this mode can be modified by multiple

processes concurrently. As a result, unlocked mode differs in some

important ways from the other open modes.

When a process opens a file in unlocked mode, the file system

returns a 2-byte field called the File ID in the rO and rl bytes of

the FCB. The File ID is a required parameter of the BDOS Lock

Record and Unlock Record functions.

The file system supports two mechanisms that allow processes to

coordinate update operations on files open in unlocked mode. The

record locking and unlocking functions allow a process to establish

and relinquish temporary ownership of particular records. A record

lock does not prevent another process from reading the locked

record; only write and lock operations for other processes are

intercepted. As an alternative, the Test and Write function

verifies the current contents of a record before allowing the write

operation to proceed.

The Record locking and unlocking functions and the Test and

Write function provide two fundamentally different approaches to

record update coordination. When a record is locked, the file

system allocates an entry in the system lock list, identifying the

locked record and associating it with the calling process. The

Unlock Record function removes the locked entry from the list.

While the locked record's entry exists in the system lock list, no

other process can lock or write to that record. Because the system

lock list is a limited resource under MP/M, a process is restricted

regarding the number of records it can lock.

The Test and Write function, on the other hand, performs its

verification at the I/O level. In a single indivisible operation,

it verifies that the user's current version of the record matches

the version on disk before allowing the write operation to proceed.

As a result, it is not restricted like the Record Lock function.

However, record update coordination can usually be performed more

efficiently with the lock functions.

The BDOS file system performs additional steps for read and

write operations to a file open in unlocked mode. These added steps

are required because the BDOS file system maintains the current

state of an open file in the user's FCB. When multiple processes

have the same file open, FCBs for the same file exist in each

processes' memory. To ensure that all process' have current

information, the file system updates the directory immediately when

an FCB for an unlocked file is changed. In addition, the file

system verifies error situations such as end of file or reading

unwritten data with the directory before returning an error. As a

result, read and write operations are less efficient for files open

in unlocked mode when compared to equivalent operations for files

opened in the default locked mode.

MP/M II Programmer’s Guide	2.2.10		Concurrent File Access

Extending a file is also a special situation for files opened

in unlocked mode. Normally, when a file is extended, the size of

the file is set to the random record number of the last record + 1.

However, when a file opened in unlocked mode is extended, the size

of the file is set to the random record number + 1 of the last 128

byte record in the file's last data block. A process must keep

track of the actual last record of a file extended while open in

unlocked mode, if that is required.

2.2.11		Multi-Sector I/O

The BDOS file system provides the capability to read or write

multiple 128-byte records in a single BDOS function call. This

multi-sector facility can be visualized as a BDOS "burst" mode,

enabling a process to complete multiple I/O operations without

interference from other running processes. The use of this facility

in an application program can improve its performance, and also

enhance overall system throughput. For example, the PIP utility

performs its sequential I/O with a multi-sector count of 8. Multi

sector I/O has its greatest impact, however, in the performance of

sequential I/O processing on MP/M II systems that support record

blocking/deblocking in their XIOS. Improved performance is achieved

by eliminating the need for a large percentage of XIOS physical

record pre-read operations.

The number of records that can be supported with multi-sector

I/0 ranges from one to sixteen. For transient programs, the default

value is one because the CLI initializes the multi-sector count of a

transient program to one when it initiates the program. The BDOS

Set Multi-Sector Count function can be used to set the count to

another value.

The multi-sector count determines the number of operations to

be performed by the following BDOS functions:

o	Sequential Read and Write functions

o	Random Read and Write functions including Write with Zero Fill

and Test and Write

o	Record Lock and Record Unlock functions

If the multi-sector count is N, calling one of the above functions

is equivalent to making N function calls. If a multi-sector I/O

operation is interrupted with an error, the file system returns the

number of 128-byte records successfully processed in the high-order

nibble of register H.

2.2.12		XIOS Blocking and Deblocking

An optional physical record blocking and deblocking facility

can be implemented as part of the XIOS when it is necessary to

maintain physical records on disk in units greater than 128-bytes.

MP/M II Programmer's Guide		2.2.12	XIOS Blocking and Deblocking

In general, record blocking and deblocking in the XIOS is

transparent to the BDOS file system as well as to programs that make

BDOS file system calls.

If this facility is implemented, then the XIOS sends data to or

receives data from the BDOS file system in logical 128-byte records,

but accesses the disk with a larger physical record size. The XIOS

uses an internal physical record buffer equal in size to the

physical record size to buffer logical records. The process of

building up physical records from 128-byte logical records is called

blocking, and it is required for BDOS write operations. The reverse

process is called deblocking and it is required for BDOS read

operations. For BDOS write operations, the XIOS postpones the

physical write operation for permanent drives (see Section 2.2.13)

if the write operation is not to the directory. For BDOS read

operations, the XIOS performs a physical read only if the current

physical record buffer does not contain the requested logical

record. In addition, if the physical record is "pending" as the

result of a previous write operation, the XIOS performs a physical

write operation prior to the read operation.

Postponing physical record write operations has implications

for some application programs. For those programs that involve file

updating, it is often critical to guarantee that the state of a file

on disk parallels the state of the file in memory after updating the

file. This is only an issue for systems that implement blocking and

deblocking because of the postponement of physical write operations.

If the system should crash while the physical buffer is pending,

data would be lost. To prevent this, the BDOS Flush Buffers

function can be invoked to force the write of any pending physical

buffers in the XIOS.

Note: The XDOS automatically calls this function when a process

terminates. In addition, the BDOS file system automatically makes a

Flush Buffers call in the Close File function.

2.2.13		Reset, Access and Free Drive

The BDOS functions Disk System Reset, Reset Drive, Access

Drive, and Free Drive allow a process to control when a drive's

directory is to be reinitialized for file operations. When MP/M II

is initiated by MPMLDR, all drives are initialized to the reset

state . Subsequently, as drives are referenced, they are

automatically logged-in by the file system. The log-in operation

initializes the drive for BDOS file operations. In general, once a

drive is logged-in, it is not necessary to relog the drive unless a

disk media change is to be made. However, MP/M II requires that a

successful drive reset be performed for a drive before a media

change. If a drive is in the reset state when the media is changed,

the next access to the drive logs in the drive. Note that the Disk

System Reset and Reset Drive functions have similar effects except

that the Disk System Reset function is directed to all drives on the

system. The user can specify any combination of drives to be reset

with the Reset Drive function.

MP/M II Programmer’s Guide		2.2.13	Reset, Access and Free Drive

Under MP/M II, the drive reset operation is conditional in

nature. Generally speaking, the file system cannot reset a drive

for a process if another process has an open file on the drive.

However, the exact action taken by a drive reset operation depends

on whether the drive to be reset is permanent or removeable. MP/M

II determines whether a drive is permanent or removeable by

interrogating a bit in the drive's disk parameter block (DPB) in the

XIOS (refer to the MP/M II System's Guide for a detailed discussion

of the DPB) . A high-order bit of 1 in the DPB checksum vector size

field designates the drive as permanent. Under MP/M II, a drive's

designation is critical to the reset operation, which is described

below.

The BDOS first determines if there are any files currently open

on the drive to be reset. If there are none, the reset takes place.

otherwise, if the drive is a permanent drive and if the drive is not

read/only, the reset operation is not performed but a successful

result is returned to the calling process. However, if the drive is

removeable or read/only, the file system determines whether other

processes have open files on the drive. If they do, the drive reset

operation is denied and an error code is returned to the calling

process. If all the files open on the drive belong to the calling

process, the file system performs a "qualified" reset operation for

the drive and returns a successful result to the calling process.

This means that the next time the drive is accessed, the log-in

operation is only performed if a media change is detected on the

drive. The logic flow of the drive reset operation is shown in

Figure 2-1.

If the file system detects a media change on a drive after a

qualified reset, it purges all open files on the drive from the

system lock list and subsequently verifies all open FCBS in file

operations for the owning process (see Section 2.2.9) . The drive is

also relogged-in. In all other cases where a media change is

detected on a drive, the file system performs the following steps:

All open files on the drive are purged from the system lock list,

and all process owning a purged file are flagged for automatic open

FCB verification. The drive is then placed in read/only status. It

is not relogged-in until a drive reset is issued for the drive.

Note: If a process references a file purged from the system lock

list in a BDOS command that requires an open FCB, it is returned an

FCB checksum error by the BDOS file system.

The Access Drive and Free Drive functions perform special

actions under MP/M II. The Access Drive function inserts a "dummy"

open file item into the system lock list for each specified drive.

While that item exists in the system lock list, the drive cannot be

reset by another process. The Free Drive function purges the open

lock list of all items including open file items belonging to the

calling process on the specified drives. Any subsequent reference

to those files by a BDOS function call requiring an open FCB results

in a FCB checksum error return.

MP/M II Programmer's Guide 2.2.13 Reset, Access and Free Drive

The BDOS function Write Protect Disk function has special

properties under MP/M II. This function can be used to set the

specified drive to read/only. However, MP/M II does not allow a

process to set a drive read/only if another process has an open file

on the drive. This applies to both removeable and permanent drives.

If a process has successfully set a drive read/only, it can prevent

other processes from resetting the drive by either opening a file on

the drive or issuing an Access Drive call for the drive. While the

open file or "dummy" item belonging to the process resides in the

system lock list, no other process can reset the drive to take it

out of read/only status.

+------------+

: open files : yes

: on drive ? :--------+

+------------+        :

:              :

: no           :

:              :

:              :

:      +---------------+

:      : Drive         : yes

:      : removeable ?  :----------+

:      +---------------+          :

:              : no               :

:      +---------------+ yes      :

:      : Drive R/O ?   :----------+

:      +---------------+          :

:              : no               :

:              :                  :

+----------+  +---------------+    +-------------+

:   Reset  :  : don't reset   :    : Open files  :  yes

:   drive  :  : drive         :    : belong to   :---------+

+----------+  +---------------+    : another     :         :

:               :            : process ?   :         :

:               :            +-------------+         :

:               : no                : no             :

:               :            +-------------+         :

:               :            : Qualified   :         :

:               :            : reset       :         :

:               :            : performed   :         :

:               :            +-------------+         :

:               :                   :                :

+----------+          :                   :           +----------+

:	Disk    :          :                   :           : Disk     :

: Reset    :----------+-------------------+           : Reset    :

: success  :                                          : Denied   :

+----------+                                          +----------+

Figure 2-1. Disk System Reset

MP/M II Programmer's Guide	2.2.14		BDOS Error Handling

2.2.14 BDOS Error Handling

The BDOS file system has an extensive error handling

capability. When it detects an error, it can respond in three ways:

1)	It can return to the calling process with return codes in

register A, H, and L identifying the error.

2)	It can display an error message on the console and abort the

process.

3)	It can display an error message on the console and return to

the calling process as in method 1.

The file system handles the majority of errors it detects via method

1. The kinds of errors the file system handles via methods 2 and 3

are called "physical" and "extended" errors. The BDOS Set Error

Mode function determines how the file system handles physical and

extended errors. The BDOS Error Mode can exist in three states. In

the default state, the BDOS displays the error message and

terminates the calling process (method 2). In return error mode,

the BDOS returns control to the calling process with the error

identified in registers A, H, and L (method 1) . In return and

display mode, the BDOS returns control to the calling process with

the error identified in registers A, H, and L, and also displays the

error message at the console (method 3). Both the return modes

ensure that MP/M II does not terminate the process because of a

physical or extended error. The return and display mode also allows

the calling process to take advantage of the built-in error

reporting of the BDOS file system. Physical and extended errors are

displayed on the console in the following format:

BDOS Err on d: error message

BDOS function: nn File: filename.type

where "d" is the name of the drive selected when the error condition

is detected; "error message" identifies the error; "nn" is the BDOS

function number, and "filename.type" identifies the file specified

by the BDOS function. If the BDOS function did not involve a FCB,

the file information is omitted.

The BDOS physical errors are identified by the following error

messages:

o Bad Sector

o Select

o File R/O

o R/O

The "Bad Sector" error results from an error condition returned to

the BDOS from the XIOS module. The file system makes XIOS read and

write calls to execute file related BDOS calls. If the XIOS read or

MP/M II Programmer's Guide	2.2.14		BDOS Error Handling

write routine detects an error, it returns an error code to the BDOS

resulting in this error.

The "Select" error also results from an error condition

returned to the BDOS from the XIOS module. The BDOS makes an XIOS

Select Disk call prior to accessing a drive to perform a requested

BDOS function. If the XIOS does not support the selected disk, it

returns an error code resulting in this error.

The "File R/O" error is returned whenever a process makes a

write operation to a file with the R/O attribute set.

The 11R/01' error is returned whenever a process makes a write

operation to a disk that is in read/only status. A drive can be

placed in read/only status explicitly with the BDOS Write Protect

Disk function, or implicitly if the file system detects a change in

media on the drive.

The BDOS extended errors are identified by the following error

messages:

• File opened in Read/only Mode

• File Currently Opened

. Close Checksum Error

• Password Error

• File Already Exists

• Illegal ? in FCB

• Open File Limit Exceeded

• No Room in System Lock List

The "File Opened in Read/only Mode" error is returned when a process

attempts to write to a file opened in read/only mode. A file can be

opened in read/only mode explicitly, or opened in read/only mode

implicitly in two ways. If a file is opened from user zero when the

current user number is non-zero, the file is opened in read/only

mode. In addition, if a file is password protected in write mode

and the password is not supplied with the open call, this error is

returned if an attempt is made to write to the file.

The "File Currently open" error is returned when a process

attempts to delete, rename, or modify the attributes of a file

opened by another process. This error is also returned when a

process attempts to open a file in a mode incompatible with the mode

in which the file was opened by another process.

The "Close Checksum Error" message is returned when the BDOS

detects a checksum error in the FCB passed to the file system with a

BDOS Close File call.

MP/M II Programmer's Guide	2.2.14		BDOS Error Handling

The "File Password" error is returned when the file password is

not supplied, or it is incorrect.

The "File Already Exists" error is returned for the BDOS Make

File and Rename File functions when the BDOS detects a conflict on

file name and type.

The "Illegal ? in FCB" error is returned whenever the BDOS

detects a "?" in the file name or type field of the passed FCB for

the BDOS Rename File, Set File Attributes, Open File, and Make File

functions.

The "Open File Limit Exceeded" error is returned when a process

exceeds the file lock limit specified in the system lock table

during system generation. The Open File, Make File, and Access

Drive functions can return this error.

The "No Room in System Lock List" error is returned when no

room for new entries exists within the system lock list. The

capacity of the system lock list is a system generation parameter.

The Open File, Make File, and Access Drive functions can return this

error.

The following paragraphs describe the error return code

conventions of the BDOS file system functions. Most BDOS file

system functions fall into three categories in regard to return

codes; they return an Error Code, a Directory Code, or an Error

Flag. The error conventions are designed to allow programs written

for earlier versions of CP/M and MP/M to run without modification

The following BDOS functions return an Error Code in register

A.

20.	Read Sequential

21.	Write Sequential

33.	Read Random

34.	Write Random

40.	Write Random w/Zero Fill

41.	Test and Write Record

42.	Lock Record

43.	Unlock Record

The Error Code definitions for register A are shown in Table 2-8.

MP/M II Programmer's Guide	2.2-14		BDOS Error Handling

Table 2-8. BDOS Error Codes

00 : Function successful 255 : Physical error : refer to register H 01 : Reading unwritten data No available directory space (Write Sequential) 02 : No available data block 03 : Cannot close current extent 04 : Seek to unwritten extent 05 : No available directory space 06 : Random record number out of range 07 : Record match error (Test and Write) * 08 : Record locked by another process (restricted to files opened in unlocked mode) 09 : Invalid FCB (previous BDOS read or write call returned an error code and invalidated the FCB) 10 : FCB checksum error * 11 : Unlocked file unallocated block verify error 12 : Process record lock limit exceeded 13 : Invalid File ID 14 : No room in BDOS internal lock table * - returned only for files opened in unlocked mode ** - returned only by the Lock Record function for files opened in unlocked mode

The following BDOS functions return a Directory Code in

register A:

15. Open File

16. Close File

17. Search For First

18. Search For Next

19. Delete File

22. Make File

23. Rename File

30. Set File Attributes

100. Set Directory Label

101. Read File XFCB

102. Write File XFCB

The Directory Code definitions for register A are shown in Table 2-9

Table 2-9. BDOS Directory Codes

00 - 03  : successful function

255 : unsuccessful function

With the exception of the BDOS search functions, Directory Code

values (0-3) have no significance other than to indicate a

successful result. However, for the search functions, a successful

Directory Code identifies the relative starting position of the

MP/M II Programmer's Guide	2.2.14		BDOS Error Handling

directory element in the calling process' current DMA buffer.

If the Set BDOS Error Mode function is used to place the BDOS

in return error mode, the following functions return an Error Flag

on physical errors:

14. Select Disk

35. Compute File Size

38. Access Drive

46. Get Disk Free Space

48. Flush Buffers

101. Return Directory Label Data

The Error Flag definition for register A is shown in Table 2-9.

Table 2-10. BDOS Error Flags

00 : successful function 255 : physical error : refer to register H

The BDOS returns register H values for all three of the above

categories in the following format:

| Nl | N2 |
| --- | --- |

where Nl denotes the high order nibble and N2 denotes the low order

nibble. The following rules govern the assignment of values to Nl

and N2.

Nl	For functions that return Error Codes, the BDOS sets Nl to the

number of sectors successfully read or written before the error

is encountered. This information is returned only when a

process uses the Set Multi-Sector Count function to set the

BDOS logical sector count to a value other than one; otherwise

the BDOS sets Nl to zero. Successful read and write functions

also set Nl to zero.

Functions that return a Directory Code or an Error Flag set NI

to zero.

N2	The values contained in N2 identify BDOS physical and extended

errors. The BDOS returns values in N2 only if it is in one of

the return error modes; otherwise, it sets N2 to zero. Table

2-10 lists the physical and extended error codes returned in

N2.

MP/M II Programmer's Guide	2.2.14		BDOS Error Handling

Table 2-11. BDOS Physical and Extended Errors

00 - no error or not a register H error 01 - Bad Sector : permanent error 02 - R/O : read/only disk 03 - R/O File : read/only file - File Opened in Read/only Mode 04 - Select : drive select error 05 - File Currently Open 06 - Close Checksum Error 07 - Password Error 08 - File Already Exists 09 - Illegal ? in FCB 10 - Open File Limit Exceeded 11 - No Room in System Lock List

Note: Register H is equal to zero if the called function is

successful. In addition, the BDOS sets N2 to zero when register A

returns a value other than 255. Except for functions that return

Directory Codes, if register A contains a value of 255 upon return,

N2 identifies the error when the BDOS is in return error mode.

The following two functions represent a special case because

they return an address in registers H and L.

27.		Get Addr(Alloc)

31.		Get Addr(Disk Parms)

When the BDOS is in return error mode and it detects a physical

error for these functions, it returns to the calling process with

registers A, H, and L all set to 255. otherwise, they return no

error code.

Under MP/M II, the following functions also represent a special

case.

13.		Reset Disk System

28.		Write Protect Disk

37.		Reset Drive

These functions return to the calling process with registers A, H,

and L all set to 255 if another process has an open file or has made

a BDOS Access Drive call that prevents the reset or write protect

operation (see Section 2.2.13). If the BDOS is not in return error

mode, these functions also display an error message identifying the

process that prevented the requested operation.

MP/M II Programmer's Guide	2.3		Base Page Initialization

2.3 Base Page Initialization

The region of memory located from BASE+OOOOH to BASE+OOFFH is

called the base page of a memory segment (BASE = memory segment base

address) . The base page contains several segments of code and data

that are used by transient programs while running under MP/M II.

The code and data areas are shown below for reference. All

addresses are relative to the beginning of the memory segment.

Table 2-12. Base Page Areas

| Locations from to | Contents |
| --- | --- |
| OOOOH - 0002H Contains a jump instruction to the XIOS process termination entry point at XIOS BASE + 3. This entry point may also be used for direct XIOS calls to the XIOS console status, console input, console output, and list output primitive functions. 0003H - 0004H (Reserved) 0005H - 0007H Contains a jump instruction to the BDOS and XDOS, and serves two purposes: JMP 0005H provides the primary entry point to the BDOS and XDOS, and LHLD 0006H places the address field of the jump instruction in the HL register pair. This value (-l) is the highest address of the memory segment available to the transient program. Note: The RDT program changes the address field to reflect the reduced memory size in debug mode. 0008H - 003AH Reserved interrupt locations for Restarts 1 - 7 003BH - 004FH (not currently used - reserved) 0050H Identifies the drive from which the transient program is read. A value of zero designates the default drive, a value of one to sixteen identifies drives A through P. 0051H - 0052H Contains the address of the password- field of the first command-tail operand in the default DMA buffer beginning at 0080H. The CLI sets this field to zero if no password for the first command-tail operand is specified. |  |

MP/M II Programmer's Guide	2.3		Base Page Initialization

Table 2-12. (continued)

| Location from to | Contents |
| --- | --- |
| 0053H Contains the length of the password field for the first command-tail operand. The CLI also sets this field to zero if no password for the first command-tail is specified. 0054H 0055H Contains the address of the password field of the second command-tail operand in the default DMA buffer beginning at 0080H. The CLI sets this field to zero if no password for the second command-tail operand is specified. 0056H Contains the length of the password field for the second command-tail operand. The CLI also sets this field to zero if no password for the second command-tail is specified. 0057H - 005BH (not currently used - reserved) 005CH - 007BH Default File Control Block (FCB) area 1 initialized by the CLI for a transient program from the first command-tail operand of the command line (if it exists). 006CH - 007BH Default File Control Block (FCB) area 2 initialized by the CLI for a transient program from the second command-tail operand of the command line (if it exists). Note: this area overlays the last 16 bytes of default FCB area 1. To use the information in this area, the transient program must copy it to another location before using FCB area 1. 007CH - 007CH Current record position of default FCB area 1. This field is used with default FCB area 1 in sequential record processing. 007DH - 007FH optional default random record position. This field is an extension of default FCB area 1 used in random record processing. 0080H - OOFFH Default 128-byte disk buffer (also filled with the command tail when the CLI loads a transient program). |  |

MP/M II Programmer's Guide	2.3		Base Page Initialization

The CLI initializes the base page prior to initiating a

transient program. The fields at BASE+0050H and above are

initialized from the command line invoking the transient program.

The command line format of a transient program usually takes the

form:

<command> <command tail>

where

<command>		=> {d:}filename{;password}

<command tail>	=> (no command tail)

=> <file spec>

=> <file spec><delimiter><file spec>

<file spec>		=> {d:}filename{.type}{;password}

If a drive {d:} is specified in the <command> field, the CLI

initializes the command drive field at 0050H to the drive index (A =

1, ... , P = 16). Otherwise, it sets the field to zero.

The default FCB at 005CH is defined if a command tail is

entered. otherwise, the fields at 5CH, 68H to 6BH are set to binary

zeros, the fields from 5DH to 67H are set to blanks. The fields at

51H through 53H are set if a password is specified for the first

<file spec> of the command tail. If not, these fields are set to

zero.

The default FCB at 006Ch is defined if a second <file spec>

exists in the command tail. otherwise, the fields at 6CH, 78H to

7Bh are set to binary zeros, the fields from 5DH to 67H are set to

blanks. The fields at 54H through 56H are set if a password is

specified for the second <file spec> of the command tail. If not,

these fields are set to zero.

Transient programs often use the default FCB at 005CH for file

operations. This FCB may even be used for random file access

because the three bytes starting at 007DH are available for this

purpose. However, a transient program must copy the contents of the

default FCB at 006CH to another area before using the default FCB at

005CH, because an open operation for the default FCB at 005CH

overwrites the FCB data at 006CH.

The default DMA address for transient programs is BASE+0080H.

The CLI also initializes this area to contain the command tail of

the command line. The first position contains the number of

characters in the command line, followed by the command line

characters. The command line characters are preceded by a leading

blank and are translated to ASCII upper-case. Because the 128-byte

region beginning at BASE+0080H is the default DMA, the BDOS file

system moves 128-byte records to this area with read operations and

accesses 128-byte records from this area with write operations. The

transient program must extract the command tail information from

MP/M II Programmer's Guide		2.3	Base Page Initialization

this buffer before performing file operations unless it explicitly

changes the DMA address with the BDOS Set DMA Address function. The

base page fields of 0051H through 0056H locate the password fields

of the first two file specifications in the command tail if they

exist. These fields are provided so that transient programs are not

required to parse the command tail for password fields. However,

the transient program must save the password, or change the DMA

address before performing file operations.

The following example illustrates the initialization of the

command line fields of the base page. Assuming the following

command line is typed at the console:

A:PROGRAM B:FILE.TYP;PASS C:FILE.TYP;PASSWORD

A hexadecimal dump of BASE+0050H to BASE+OOA5H would show the base

page initialization performed by the CLI.

0050H: 01 8D 00 04 9D 00 08 00 00 00 00 00 02 46 49 4C . . . . . .FIL

0060H: 45 20 20 20 20 54 59 50 00 00 00 00 03 46 49 4C E . .TYP. .FIL

0070H: 45 20 20 20 20 54 59 50 00 00 00 00 00 00 00 00 E . .TYP. .

0080H: 24 20 42 3A 46 49 4C 45 2E 54 59 50 3B 50 41 53 . B:FILE.TYP;PAS

0090H: 53 20 43 3A 46 49 4C 45 2E 54 59 50 3B 50 41 53 S C:FILE.TYP;PAS

OOAOH: 53 57 4F 52 44 00                                 SWORD.

MP/M II Programmer's Guide	2.4	BDOS Calls:	Function 0

2.4		BDOS Function Calls

FUNCTION 0: SYSTEM RESET

Entry Parameters: Register C: OOH

The System Reset function terminates the calling process,

releasing all system resources owned by the process. In general, a

process can own one or more of the following resources: memory

segments, consoles, printers, mutual exclusion messages, and system

lock list entries that record open files and locked records. All

released resources become available to other processes on the

system. For example, if a system console is released by a

terminating process, it is usually given back to the console's TMP.

This occurs when the TMP is the highest priority process waiting for

the console.

Normally, the System Reset function operates the same way under

MP/M II as it does under CP/M: the calling program terminates and

the user receives the command prompt. Note that the disk subsystem

is not reset by System Reset under MP/M II.

For transient programs, System Reset is equivalent to a jump to

BASE+O.

FUNCTION 1: CONSOLE INPUT

Entry Parameters: Register C: 01H Returned Value: Register A: ASCII Character

The Console Input function reads the next character from the

console device to register A. Most graphic characters, including

carriage return, line feed, and backspace (CONTROL-H) are echoed to

the console. Tab characters (CONTROL-I) are expanded in columns of

8 characters. However, the terminate process (CONTROL-C) and detach

process (CONTROL-D) characters are intercepted by the BDOS (see

Section 2.1). The BDOS does not return control to the calling

process until a character is typed, thus suspending execution if a

character is not ready.

MP/M II Programmer's Guide   2.4	BDOS Calls:	Function 1

MP/M II performs an XDOS Attach Console call (function 146) for

the calling process if it does not own the console (see Section

2.1).

FUNCTION 2: CONSOLE OUTPUT

Entry Parameters: Register C: 02H Register E: ASCII Character

The Console Output function sends the ASCII character from

register E to the console device. It expands tab characters

(CONTROL-I) in columns of 8 characters, and checks for start scroll

(CONTROL-S) , stop scroll (CONTROL-Q) , terminate process (CONTROL-C) ,

and detach process (CONTROL-D) (see Section 2.1).

MP/M II performs an XDOS Attach Console call (function 146) for

the calling process if it does not own the console (see Section

2.1).

FUNCTION 3: RAW CONSOLE INPUT

Entry Parameters: Register C: 03H Returned Value: Register A: ASCII Character

The Raw Console Input function reads the next console character

to register A. It reads all characters including control

characters, without any testing or interpretation.

MP/M II performs an XDOS Attach Console call (Function 146) for

the calling process if it does not own the console (see Section

2.1).

MP/M II does not support the CP/M Reader Input function because

the system treats all character I/O devices such as the reader/punch

as consoles.

MP/M II Programmer's Guide     2.4		BDOS Calls:	Function 4

FUNCTION 4: RAW CONSOLE OUTPUT

Entry Parameters: Register C: 04H Register E: ASCII Character

The Raw Console Output function sends the ASCII character from

register E to the console device. It does not test the output

character; that is, tabs are not expanded and no checks are made

for control characters.

MP/M II performs an XDOS Attach Console call (function 146) for

the calling process if it does not own the console (see Section

2.1).

MP/M II does not support the CP/M Punch Output function.

FUNCTION 5: LIST OUTPUT

Entry Parameters: Register C: 05H Register E: ASCII Character

The List Output function sends the ASCII character in register

E to the list device.

MP/M II performs an XDOS Attach List call (function 158) for

the calling process if it does not own the list device (see Section

2.1).

MP/M II Programmer's Guide     2.4		BDOS Calls:	Function 6

FUNCTION 6: DIRECT CONSOLE I/O

Entry Parameters: Register C: 06H Register E: OFFH (input/ status) or OFEH (status)or OFDH (input) char (output) Returned Value: Register A: char or status (no value)

MP/M II supports direct console I/O for those specialized

applications where unadorned console input and output is required.

The programmer should use direct console I/O carefully because it

bypasses all the normal control character functions. Programs that

perform direct I/O through the BIOS under previous releases of CP/M

should be changed to use direct I/O under the new BDOS so that they

can be fully supported under future releases of MP/M and CP/M.

A Process calls Function 6 by passing one of four different

values in register E. These are summarized in Table 2-13, below.

Table 2-13. Function 6 Entry Parameters

| Register E value | Meaning |
| --- | --- |
| OFFH console input/status command, returns an input character; if no character is ready, a value of zero is returned. OFEH console status command (On return, register A contains 00 if no character is ready; otherwise it contains FFH.) OFDH console input command, returns an input character; this function will suspend the calling process until a character is ready. ASCII Function 6 assumes register E Character contains a valid ASCII character and sends it to the console. |  |

MP/M II Programmer-Is Guide	2.4		BDOS Calls:	Function 6

Note: MP/M II is not compatible with MP/M 1.1 in regard to

Function 6 with a parameter of E=FFH. Under MP/M 1.1 the direct

console input command (E=FFH) suspends the calling process until a

character is typed, whereas MP/M II returns immediately with a zero

if no character is available. To upgrade programs using Function 6

with E=FFH under MP/M 1.1 to MP/M II, the direct input command

(E=FDH) must be used. The change from MP/M 1.1 was required to

achieve consistent direct console I/O handling between CP/M, MP/M

II, CP/M-86 and MP/M-86.

MP/M II performs an XDOS Attach Console call (Function 146) for

the calling process if it does not own the console (see Section

2.1) . MP/M II performs a dispatch if a direct console input/status

command (E=FFH) is made which returns a zero indicating that a

character is not ready.

FUNCTION 7: GET I/O BYTE

MP/M II does not support the Get I/O Byte function.

FUNCTION 8: SET I/O BYTE

MP/M II does not support the Set I/O Byte function.

FUNCTION 9: PRINT STRING

Entry Parameters: Register C: 09H Registers DE: String Address

The Print String function sends the character string stored in

memory at the location addressed by register pair DE to the console

until it encounters a "$" in the string. Function 9 expands tab

characters (CONTROL-I) in columns of 8 characters. It also checks

for start scroll (CONTROL-S) , stop scroll (CONTROL-Q) , terminate

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 9

process (CONTROL-C) and detach process (CONTROL-D) (see Section

2.1).

MP/M II performs an XDOS Attach Console call (Function 146) for

the calling process if it does not own the console (see Section

2.1).

FUNCTION 10: READ CONSOLE BUFFER

Entry Parameters: Register C: OAH Registers DE: Buffer Address Returned Value: Console Characters in Buffer

The Read Console Buffer function reads a line of edited console

input to a buffer addressed by register pair DE. It terminates

input when the buffer is filled or when it encounters a return

(CONTROL-M) or a line feed (CONTROL-J) character. The input buffer

addressed by DE has the following format:

DE: +0 +1 +2 +3 +4 +5 +6 +7 +8 . . . .        +n

+---------------------------------------------

:mx:nc:cl:c2:c3:c4:c5:c6:c7: . . .        :??:

+---------------------------------------------

where "mx" is the maximum number of characters which the buffer

holds, and "nc" is the number of characters placed in the buffer.

The characters entered by the operator follow the "nc" value. The

value "mx" must be set, prior to making a Function 10 call and may

range in value from 1 to 255. Setting "mx" to zero is equivalent

to setting "mx" to one. The value "nc" is returned to the calling

process and may range from zero to "mx". If nc < mx, then

uninitialized positions follow the last character, denoted by "??"

in the above figure. Note that a terminating return or line feed

character is not placed in the buffer and not included in the count

"nc".

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 10

Function 10 recognizes the edit control characters summarized

in Table 2-14, below.

Table 2-14. Console Buffer Edit Control Characters

| Character | Edit Control Function |
| --- | --- |
| rub/del removes and echoes the last character CONTROL-C reboots when at the beginning of line CONTROL-E causes physical end of line CONTROL-H backspaces one character position CONTROL-J (line feed) terminates input line CONTROL-M (return) terminates input line CONTROL-P echoes console output to the list device CONTROL-R retypes the current line after new line CONTROL-U removes current line after new line CONTROL-X backspaces to beginning of current line |  |

The control functions that return the cursor to the leftmost

position (e.g., CONTROL-X) do so only to the column position where

the prompt ended (in earlier releases, the cursor returned to the

extreme left margin). This convention simplifies data input and

line correction.

MP/M II performs an XDOS Attach Console call (Function 146) for

the calling process if it does not own the console (see Section

2.1).

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 11

FUNCTION 11: GET CONSOLE STATUS

Entry Parameters: Register C: OBH Returned Value: Register A: Console Status

The Get Console Status function checks to see if a character

has been typed at the console. If a character is ready, Function 11

returns the value 01H in register A. If a character is not ready,

it returns a value of OOH.

MP/M II performs an XDOS Attach Console call (Function 146) for

the calling process if it does not own the console (see Section

2.1).

FUNCTION 12: RETURN VERSION NUMBER

Entry Parameters: Register C: OCH Returned Value: Registers HL: Version Number

The Return Version Number function provides information that

allows version independent programming. It returns a two-byte value

in register pair HL: H contains 01H for MP/M and L contains 30H,

the BDOS file system version number. Function 12 is useful for

writing applications programs that provide both random and

sequential file access, and disabling the random access when

operating under early versions of CP/M.

XDOS Function 163 can be called to obtain the MP/M version

number.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 13

FUNCTION 13: RESET DISK SYSTEM

Entry Parameters: Register C: ODH Returned Value: Register A: Return Code

The Reset Disk System function restores the file system to a

reset state where all the disk drives are set to read/write (see

Functions 28 and 29), the default disk is set to drive A, and the

default DMA address is reset to BASE+0080H. This function can be

used, for example, by an application program that requires disk

changes during operation. Function 37 (Reset Drive) can also be

used for this purpose.

This function is conditional under MP/M II. If another process

has an open file on a removeable or read/only drive, the disk reset

is denied and no drives are reset.

Upon return, if the reset operation is successful, register A

is set to zero. Otherwise, register A is set to OFFH (255 decimal) .

If the BDOS error mode is not Return Error mode (see Function 45),

then an error message is displayed at the console, identifying a

process owning an open file.

FUNCTION 14: SELECT DISK

Entry Parameters: Register C: OEH Register E: Selected Disk Returned Value: Register A: Error Flag Register H: Physical Error

The Select Disk function designates the disk drive named in

register E as the default disk for subsequent BDOS file operations.

Register E is set to 0 for drive A, 1 for drive B, and so-forth

through 15 for drive P in a full 16 drive system. In addition,

function 14 logs in the designated drive if it is currently in the

reset state. Logging-in a drive activates the drive's directory

until the next disk system reset or drive reset operation.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 14

FCBs that specify drive code zero (dr = OOH) automatically

reference the currently selected default drive. FCBs with drive

code values between 1 and 16, however, ignore the selected default

drive and directly reference drives A through P.

Upon return, register A contains a zero if the select operation

was successful. If a physical error was encountered, the select

function performs different actions depending on the BDOS error mode

(see Function 45) . If the BDOS error mode is in the default mode, a

message identifying the error is displayed at the console and the

calling process is terminated. Otherwise, the select function

returns to the calling process with register A set to OFFH and

register H set to one of the following physical error codes:

01 : Permanent error

04 : Select error

FUNCTION 15: OPEN FILE

Entry Parameters: Register C: OFH Registers DE: FCB Address Returned Value: Register A: Directory Code Register H: Physical or Extended Error

The Open File function activates the FCB for a file that exists

in the disk directory under the currently active user number or user

zero. The calling process passes the address of the FCB in register

pair DE, with byte 0 of the FCB specifying the drive, bytes 1

through 11 specifying the filename and type, and byte 12 specifying

the extent. Normally, byte 12 of the FCB is initialized to zero.

Interface attributes f5' and W of the FCB specify the mode in

which the file is to be opened as shown below:

f5' = 0,	  f6' = 0 - Open in locked mode (default)

f5' = 1,	  f6’ = 0 - open in unlocked mode

f5' = 0 or 1, f6’ = 1 - Open in read/only mode

If the file is password protected in Read mode, the correct password

must be placed in the first eight bytes of the current DMA or have

been previously established as the default password (see Function

106). Note that the current record field of the FCB ("cr") must be

zeroed by the calling process if the file is to be accessed

sequentially from the first record.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 15

The Open File function performs the following steps for files

opened in locked or read/only mode. If the current user is non

zero, and the file to be opened does not exist under the current

user number, the open function searches user zero for the file. If

the file exists under user zero, and has the system attribute (t2')

set, the file is opened under user zero. The open mode is

automatically set to read/only when this is done.

The Open File function also performs the following action for

files opened in locked mode when the current user number is zero.

If the file exists in the directory under user zero, and has both

the system attribute (t2’) set and the read/only attribute (tl')

set, the open mode is automatically set to read/only. Note that

read/only mode implies the file can be concurrently accessed by

other processes if they open the file in read/only mode.

If the open operation is successful, the user's FCB is

activated for read and write operations as follows. The relevant

directory information is copied from the matching directory FCB into

bytes dO through dn of the FCB. A checksum is computed and assigned

to the FCB. BDOS functions that require an open FCB (e.g. Read

Sequential) verify that the FCB checksum is valid before performing

their operation. if the file is opened in unlocked mode, bytes rO

and rl of the FCB are set to a two byte value called the File ID.

The File ID is a required parameter for the BDOS Lock Record and

Unlock Record functions. If the open mode is forced to read/only

mode, interface attribute f8' is set to one in the user's FCB. In

addition, if the referenced file is password protected in Write mode

and the correct password was not passed in the DMA or did not match

the default password, interface attribute f7' is set to one. Write

operations are not supported for an activated FCB if interface

attribute f7' or f8' is true.

The BDOS file system also creates an open file item in the

system lock list to record a successful open file operation. While

this item exists, no other process can delete, rename, or modify the

file's attributes. In addition, this item prevents other processes

from opening the file if the file was opened in locked mode. It

also requires that other processes match the file's open mode if the

file was opened in unlocked or read/only mode. Normally, this item

remains in the system lock list until the file is permanently closed

or the process that opened the file terminates.

When the open operation is successful, the open function also

makes an Access date and time stamp for the opened file when the

following conditions are satisfied: the reference drive has a

directory label that requests Access date and time stamping, the

opened file has an XFCB, and the referenced drive is read/write.

Upon return, the open function returns a directory code in

register A with the value 0 through 3 if the open was successful, or

FFH (255 decimal) if the file was not found. Register H is set to

zero in both of these cases. If a physical or extended error was

encountered, the open function performs different actions depending

on the BDOS error mode (see Function 45) . If the BDOS error mode is

MP/M II Programmer's Guide     2.4		BDOS Calls:	Function 19

in the default mode, a message identifying the error is displayed at

the console and the process is terminated. otherwise, the open

function returns to the calling process with register A set to OFFH

and register H set to one of the following physical or extended

error codes:

01 :	Permanent error

04 :	Select error

05 :	File is open by another process or by the

current process in an incompatible mode

07 :	File password error

09 :	? in the FCB file name or type field

10 :	Process open file limit exceeded

11 :	No room in the system lock list

FUNCTION 16: CLOSE FILE

Entry Parameters: Register C: 10H Registers DE: FCB Address Returned Value: Register A: Directory Code Register H: Physical or Extended Error

The Close File function performs the inverse of the open file

function. The calling process passes the address of an FCB in the

register pair DE. The referenced FCB must have been previously

activated by a successful open or make function call (see functions

15 and 22). Interface attribute f5l specifies how the file is to be

closed as shown below:

f5l = 0 - Permanent close (default mode)

f5l = 1 - Partial close

The close function first verifies that the referenced FCB has a

valid checksum. If the checksum is valid and the referenced FCB

contains new information because of write operations to the FCB, the

close function permanently records the new information in the

referenced disk directory. Note that the FCB does not contain new

information and the directory update step is bypassed if only read

and/or update operations have been made to the referenced FCB.

However, the close function always attempts to locate the FCB's

corresponding entry in the directory, and returns an error if the

directory entry is not found.

MP/M II Programmer's Guide     2.4		BDOS Calls:	Function 16

If the close function successfully performs the above steps,

and if interface attribute f5’ indicates that the close is

permanent, the close function removes the file's item from the

system lock list. If the FCB was opened in unlocked mode, it also

purges all record lock items belonging to the file from the system

lock list. Because the file's lock list item is removed, the close

function invalidates the FCB's checksum to ensure the referenced FCB

is not subsequently used with BDOS functions that require an open

FCB (e.g. Write Sequential).

The close function also makes an Update date and time stamp for

the closed file when the following conditions are satisfied: the

reference drive has a directory label that requests Update date and

time stamping, the referenced file has an XFCB, the referenced drive

is read/write, and a write operation to the file was made since the

FCB was opened. None of these steps are performed for partial close

operations (f5’ = 1).

Upon return, the close function returns a Directory Code in

register A with the value 0 to 3 if the close was successful, or FFH

(255 Decimal) if the file was not found. Register H is set to zero

in both of these cases. If a physical or extended error was

encountered, the close function performs different actions depending

on the BDOS error mode (see Function 45) . If the BDOS error mode is

in the default mode, a message identifying the error is displayed at

the console and the calling process is terminated. Otherwise, the

close function returns to the calling process with register A set to

OFFH and register H set to one of the following physical or extended

error codes:

01 : Permanent error

02 : Read/only disk

04 : Select error

06 : FCB checksum error

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 17

FUNCTION 17: SEARCH FOR FIRST

Entry Parameters: Register C: 11H Registers DE: FCB Address Returned Value: Register A: Directory Code Register H: Physical Error

Search For First scans the directory for a match with the FCB

addressed by register pair DE. Two types of searches can be

performed. For standard searches, the calling process initializes

bytes 0 through 12 of the referenced FCB, with byte 0 specifying the

drive directory to be searched, bytes 1 through 11 specifying the

file or files to be searched for, and byte 12 specifying the extent.

Normally byte 12 is set to zero. An ASCII question mark (63

decimal, 3F hex) in any of the bytes 1 through 12 matches all

entries on the directory in the corresponding position. This

facility, called ambiguous reference, can be used to search for

multiple files on the directory. When called in the standard mode,

the search function scans for the first file entry in the specified

directory that matches the FCB and belongs to the current user

number.

The search function also initializes the Search For Next

function. After the search function has located the first directory

entry matching the referenced FCB, the Search For Next function can

be called repeatedly to locate all remaining matching entries. In

terms of execution sequence, however, the Search For Next call must

either follow a Search For First or Search For Next call with no

other intervening BDOS disk-related function calls.

If byte 0 of the referenced FCB is set to a question mark, the

search function ignores the remainder of the referenced FCB and

locates the first directory entry residing on the current default

drive. All remaining directory entries can be located by making

multiple Search For Next calls. This type of search operation is

not normally made by application programs, but it does provide

complete flexibility to scan all current directory values. Note

that this type of search operation must be performed to access a

drive's Directory Label (see Section 2.2.5).

Upon return, the search function returns a Directory Code in

register A with the value 0 to 3 if the search was successful, or

OFFH (255 Decimal) if a matching directory entry was not found.

Register H is set to zero in both of these cases. For successful

searches, the current DMA is also filled with the directory record

containing the matching entry, and the relative starting position is

A * 32 (i.e. rotate the A register left 5 bits, or ADD A five

MP/M II Programmer's Guide     2.4		BDOS Calls:	Function 17

times). Although not normally required for application programs,

the directory information can be extracted from the buffer at this

position.

If a physical error was encountered, the search function

performs different actions depending on the BDOS error mode (See

function 45). If the BDOS error mode is in the default mode, a

message identifying the error is displayed at the console and the

calling process is terminated. Otherwise, the search function

returns to the calling process with register A set to OFFH and

register H set to one of the following physical error codes:

01		Permanent error

04		Select error

FUNCTION 18: SEARCH FOR NEXT

Entry Parameters: Register C: 12H Returned Value: Register A: Directory Code Register H: Physical Error

The Search For Next function is identical to the Search For

First function, except that the directory scan continues from the

last entry that was matched. Function 18 returns a Directory code

in register A, analogous to Function 17. Note: In execution

sequence, a Function 18 call must follow either a Function 17 or

another Function 18 call with no other intervening BDOS disk-related

function calls.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 19

FUNCTION 19: DELETE FILE

Entry Parameters: Register C: 13H Registers DE: FCB Address Returned Value: Register A: Directory Code Register H: Extended or Physical Error

The Delete File function removes files and/or XFCBs that match

the FCB addressed in register pair DE. The filename and type may

contain ambiguous references (i.e., question marks in bytes fl

through t3) , but the "dr" byte cannot be ambiguous, as it can in the

Search and Search Next functions. Interface attribute f5’ specifies

the type of delete operation to be performed as shown below:

f5' = 0 - Standard Delete (default mode)

f5' = 1 - Delete only XFCB's

If any of the files specified by the referenced FCB are password

protected, the correct password must be placed in the first eight

bytes of the current DMA buffer, or have been previously established

as the default password (see Function 106).

For standard delete operations, the delete function removes all

directory entries belonging to files that match the referenced FCB.

All disk directory and data space owned by the deleted files is

returned to free space, and becomes available for allocation to

other files. Directory XFCBs that were owned by the deleted files

are also removed from the directory. If interface attribute f5' of

the FCB is set to 1, Function 19 deletes only the directory XFCBs

matching the referenced FCB. Note: If any of the files matching

the input FCB specification fail the password check, are read/only,

or are currently open by another process, then the delete function

deletes no files or XFCBs. This applies to both types of delete

operations.

A process can delete a file that it currently has open if the

file was opened in locked mode. However, a checksum error is

returned if the process makes a subsequent reference to the file

with a BDOS function requiring an open FCB. Files open in read/only

or unlocked mode cannot be deleted by any process.

Upon return, the delete function returns a Directory Code in

register A with the value 0 to 3 if the delete was successful, or

OFFH (255 Decimal) if no file matching the referenced FCB was found.

Register H is set to zero in both of these cases. If a physical or

extended error was encountered, the delete function performs

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 19

different actions depending on the BDOS error mode (see Function

45). If the BDOS error mode is the default mode, a message

identifying the error is displayed at the console and the calling

process is terminated. Otherwise, the delete function returns to

the calling process with register A set to OFFH and register H set

to one of the following physical or extended error codes:

01 :	Permanent error

02 :	Read/only disk

03 :	Read/only file

04 :	Select Error

05 :	File open by another process or open

in read/only or unlocked mode

07 :	File password error

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 20

FUNCTION 20: READ SEQUENTIAL

Entry Parameters: Register C: 14H Registers DE: FCB Address Returned Value: Register A: Error Code Register H: Physical Error

The Read Sequential function reads the next one to sixteen

128-byte records from a file into memory beginning at the current

DMA address. The BDOS Multi-Sector Count (see Function 44)

determines the number of records to be read. The default is one

record. The FCB addressed by register pair DE must have been

previously activated by an Open or Make function call.

Function 20 reads each record from byte "cr" of the extent,

then automatically increments the "cr" field to the next record

position . If the "cr" field overflows then the function

automatically opens the next logical extent and resets the "cr"

field to 0 in preparation for the next read operation. The calling

process must set the "cr" field to 0 following the open call if the

intent is to read sequentially from the beginning of the file.

MP/M II Programmer's Guide     2.4		BDOS Calls:	Function 20

Upon return, the Read Sequential function sets register A to

zero if the read operation was successful. otherwise, register A

contains an error code identifying the error as shown below:

01 : Reading unwritten data (end of file)

09 : Invalid FCB

10 : FCB checksum error

11 : Unlocked file verification error

255 : Physical error; refer to register H

Error Code 01 is returned if no data exists at the next

record position of the file. Normally, the no data situation is

encountered at the end of a file. However, it can also occur if an

attempt is made to read a data block which has not been previously

written, or an extent which has not been created. These situations

are usually restricted to files created or appended with the BDOS

random write functions (see Functions 34 and 40).

Error Code 09 is returned if the FCB was invalidated by a

previous BDOS random read or write call that returned an error. A

Read Random call (Function 33) for an existing record in the file,

can be made to revalidate the FCB.

Error Code 10 is returned if the referenced FCB failed the

FCB checksum test.

Error Code 11 is returned if the BDOS cannot locate the FCB's

directory entry when attempting to verify that the referenced FCB

contains current information. This error is only returned for files

open in unlocked mode.

Error Code 255 is returned if a physical error was

encountered and the BDOS error mode is Return Error mode or Return

and Display Error mode (see Function 45) . If the error mode is the

default mode, a message identifying the physical error is displayed

at the console and the calling process is terminated. When a

physical error is returned to the calling process, it is identified

by the four low-order bits of register H as shown below:

01 : Permanent error

04 : Select error

The Read Sequential function also sets the four high-order

bits of register H on all error returns when the BDOS Multi-Sector

Count is greater than one. In this case, the four bits contain an

integer set to the number of records successfully read before the

error was encountered. This value can range from 0 to 15. The

high-order four bits of register H are always zeroed when the Multi

Sector Count is equal to one.

MY/M 11 Programmer's Guide	2.4		BDOS Calls:	Function 21

FUNCTION 21: WRITE SEQUENTIAL

Entry Parameters: Register C: 15H Registers DE: FCB Address Returned Value: Register A: Error Code Register H: Physical Error

The Write Sequential function writes one to sixteen 128-byte

data records beginning at the current DMA address into the file

named by the FCB addressed in register pair DE. The BDOS Multi

Sector Count (see Function 44) determines the number of 128 byte

records that are written. The default is one record. The

referenced FCB must have been previously activated by a BDOS Open or

Make function call.

Function 21 places the record into the file at the position

indicated by the "cr" byte of the FCB, and then automatically

increments the "cr" byte to the next record position. If the "cr"

field overflows, the function automatically opens or creates the

next logical extent and resets the "cr" field to 0 in preparation

for the next write operation. If Function 21 is used to write to an

existing file, then the newly written records overlay those already

existing in the file. The calling process must set the "cr" field

to 0 following an Open or Make call if the intent is to write

sequentially from the beginning of the file.

Upon return, the Write Sequential function sets register A to

zero if the write operation was successful. otherwise, register A

contains an error code identifying the error as shown below:

01	No available directory space

02	No available data block

08	Record locked by another process

09	Invalid FCB

10	FCB checksum error

11	Unlocked file verification error

255	Physical error : refer to register H

Error Code 01 is returned when the write function attempts to

create a new extent that requires a new directory entry and no

available directory entries exist on the selected disk drive.

Error Code 02 is returned when the write command attempts to

allocate a new data block to the file and no unallocated data blocks

exist on the selected disk drive.

MP/M II Programmer's Guide     2.4		BDOS Calls:	Function 21

Error Code 08 is returned if the write function attempts to

write to a record locked by another process. This error is only

returned for files open in unlocked mode.

Error Code 09 is returned if the FCB was invalidated by a

previous BDOS random read or write call that returned an error. A

Read Random call (Function 33) for an existing record in the file

can be made to revalidate the FCB.

Error Code 10 is returned if the referenced FCB failed the FCB

checksum test.

Error Code 11 is returned if the BDOS cannot locate the FCB's

directory entry when attempting to verify that the referenced FCB

contains current information. This error is only returned for files

open in unlocked mode.

Error Code 255 is returned if a physical error was encountered

and the BDOS error mode is Return Error mode or Return and Display

Error mode (see Function 45). If the error mode is the default

mode, a message identifying the physical error is displayed at the

console and the calling process is terminated. When a physical

error is returned to the calling process, it is identified by the

four low-order bits of register H as shown below:

01 :	Permanent error

02 :	Read/only disk

03 :	Read/only file or

File open in read/only mode or

File password protected in Write mode

04 :	Select error

The Write Sequential function also sets the four high-order bits of

register H on all error returns when the BDOS Multi-Sector Count is

greater than one. In this case, the four bits contain an integer

set to the number of records successfully written before the error

was encountered. This value can range from zero to 15. The high

order four bits of register H are always zeroed when the Multi

Sector Count is equal to one.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 22

FUNCTION 22: MAKE FILE

Entry Parameters: Register C: 16H Registers DE: FCB Address Returned Value: Register A: Directory Code Register H: Physical or Extended Error

The Make File function creates a new directory entry for a file

under the current user number. It also creates an XFCB for the file

if the referenced drive has a Directory Label that invokes automatic

creation of XFCBs. The calling process passes the address of the

FCB in the register pair DE, with byte 0 of the FCB specifying the

drive, bytes 1 through 11 specifying the filename and type, and byte

12 set to the extent number. Normally, byte 12 is set to zero.

Byte 32 of the FCB (the "cr" field) must be initialized to zero

(before or after the Make call) if the intent is to write

sequentially from the beginning of the file.

Interface attribute f5' specifies the mode in which the file is

to be opened. Interface attribute f6l specifies whether a password

is to be assigned to the created file. The interface attributes are

summarized below:

f5’ = 0 - Open in locked mode (default mode)

f5’ = 1 - Open in unlocked mode

f6’ = 0 - Don't assign password (default)

f6’ = 1 - Assign password to created file

When attribute W is set to 1, the calling process must place the

password in the first 8 bytes of the current DMA buffer and set byte

9 of the DMA buffer to the password mode (See Function 102).

The Make function returns with an error if the referenced FCB

names a file that currently exists in the directory under the

current user number. A preceding delete operation can be made if

there is any possibility of duplication.

If the make operation is successful, it activates the

referenced FCB for file operations (opens the FCB) and initializes

both the directory entry and the referenced FCB to an empty file. A

checksum is computed and assigned to the FCB. BDOS functions that

require an open FCB (e.g. Write Random) verify that the FCB checksum

is valid before performing their operation. If the open mode is

unlocked, bytes rO and rl are set to a two byte value called the

File ID. The File ID is a required parameter for the BDOS Lock

Record and Unlock Record functions. Note that the Make File

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 22

function initializes all file attributes to zero.

The BDOS file system also creates an open file item in the

system lock list to record a successful make file operation. While

this item exists, no other process can delete, rename, or modify the

file's attributes.

If the referenced drive contains a Directory Label that invokes

automatic creation of XFCBs, the make function creates an XFCB and

makes a Creation date and time stamp for the created file. Note

that the Creation time stamp is not made (the XFCB Creation time

stamp field is set to zeroes) if an XFCB is assigned to a file by

the BDOS Write File XFCB call. If interface attribute f6’ of the

FCB is 1, the make function also assigns the password passed in the

DMA to the file.

Upon return, the make function returns a directory code in

register A with the value 0 through 3 if the make operation was

successful, or OFFH (255 decimal) if no directory space was

available. Register H is set to zero in both of these cases. If a

physical or extended error was encountered, the make function

performs different actions depending on the BDOS error mode (see

Function 45) . If the BDOS error mode is the default mode, a message

identifying the error is displayed at the console and the calling

process is terminated. Otherwise, the make function returns to the

calling process with register A set to OFFH and register H set to

one of the following physical or extended error codes:

01 : Permanent error

02 : Read/only disk

04 : Select error

08 : File already exists

09 : ? in filename or type field

10 : Process open file limit exceeded

11 : No room in the system lock list

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 23

FUNCTION 23: RENAME FILE

Entry Parameters: Register C: 17H Registers DE: FCB Address Returned Value: Register A: Return Code Register H: Physical or Extended Error

The Rename function uses the FCB addressed by register pair DE

to change all directory entries of the file specified by the

filename in the first 16 bytes of the FCB to the filename in the

second 16 bytes. If the file specified by the first filename is

password protected, the correct password must be placed in the first

eight bytes of the current DMA buffer, or have been previously

established as the default password (see Function 106). The calling

process must also ensure that the filenames specified in the FCB are

valid and unambiguous, and that the new filename does not already

exist on the drive. Function 23 uses the "dr" code at byte 0 of the

FCB to select the drive. The drive code at byte 16 of the FCB is

ignored.

A process can rename a file that it has open if the file was

opened in locked mode. However, if the process subsequently

references the file with a BDOS function requiring an open FCB, a

checksum error is returned. A file open in read/only or unlocked

mode cannot be renamed by any process.

Upon return, the rename function returns a Directory Code in

register A with the value 0 to 3 if the rename was successful, or

OFFH (255 Decimal) if the file named by the first filename in the

FCB was not found. Register H is set to zero in both of these

cases. If a physical or extended error was encountered, the rename

function performs different actions depending on the BDOS error mode

(see Function 45). If the BDOS error mode is the default mode, a

message identifying the error is displayed at the console and the

process is terminated. Otherwise, the rename function returns to

the calling process with register A set to OFFH and register H set

to one of the following physical or extended error codes:

01 : Permanent error

02 : Read/only disk

03 : Read/only file

04 : Select error

05 : File open by another process

07 : File password error

08 : File already exists

09 : ? in file name or type field

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 24

FUNCTION 24: RETURN LOGIN VECTOR

Entry Parameters: Register C: 18H Returned Value: Registers HL: Login Vector

Function 24 returns the login vector in register pair HL. The

login vector is a 16-bit value with the least significant bit of L

corresponding to drive A, and the high-order bit of H corresponding

to the 16th drive, labeled P. A "0" bit indicates that the drive

is not on-line, while a "l" bit indicates the drive is active. A

drive is made active by either an explicit BDOS Select Disk call

(number 14), or an implicit selection when a BDOS file operation

specifies a non-zero "dr" byte in the FCB. Function 24 maintains

compatibilty with earlier releases, since registers A and L contain

the same values upon return.

FUNCTION 25: RETURN CURRENT DISK

Entry Parameters: Register C: 19H Returned Value: Register A: Current Disk

Function 25 returns the currently selected default disk number

in register A. The disk numbers range from 0 through 15

corresponding to drives A through P.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 26

FUNCTION 26: SET DMA ADDRESS

Entry Parameters: Register C: IAH Registers DE: DMA Address

"DMA" is an acronym for Direct Memory Address, which is often

used in connection with disk controllers that directly access the

memory of the computer to transfer data to and from the disk

subsystem. Under MP/M II, the current DMA is usually defined as the

buffer in memory where a record resides before a disk write and

after a disk read operation. If the BDOS Multi-Sector Count is

equal to one (see Function 44) , the size of the buffer is 128 bytes.

However, if the BDOS Multi-Sector Count is greater than one, the

size of the buffer must equal N * 128, where N equals the Multi

Sector Count.

Some BDOS functions also use the current DMA to pass parameters

and to return values. For example, BDOS functions that check and

assign file passwords, require that the password be placed in the

current DMA. As another example, Function 46 (Get Disk Free Space)

returns its results in the first 3 bytes of the current DMA. When

the current DMA is used in this context, the size of the buffer in

memory is determined by the specific requirements of the called

function.

When a transient program is initiated by the CLI, its DMA

address is set to BASE+0080H. The BDOS Reset Disk System function

(Function 13) also sets the DMA address to BASE+0080H. The Set DMA

function can change this default value to another memory address.

The DMA address is set to the value passed in the register pair DE.

The DMA address remains at this value until it is changed by another

Set DMA Address, or Reset Disk System call.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 27

FUNCTION 27: GET ADDR(ALLOC)

Entry Parameters: Register C: 1BH Returned Value: Registers HL: ALLOC Address

MP/M II maintains an "allocation vector" in main memory for

each active disk drive. Many programs commonly use the information

provided by the allocation vector to determine the amount of free

data space on a drive. Note, however, that the allocation

information may be inaccurate if the drive has been marked

read/only.

Function 27 returns in register pair HL, the base address of

the allocation vector for the currently selected drive. If a

physical error is encountered when the BDOS error mode is one of the

return modes (see Function 45) , Function 27 returns the value OFFFFH

in the register pair HL.

In banked switched MP/M II systems, the allocation vector may

be placed in bank zero. This is an XIOS option. In this case, a

transient program that has been loaded into another bank cannot

access the allocation vector. However, the BDOS function, Get Disk

Free Space (Function 46), can be used to directly return the number

of free 128 byte records on a drive. In fact, the MP/M II utilities

that display a drive's free space (STAT,SDIR, and SHOW) use Function

46 for that purpose.

FUNCTION 28: WRITE PROTECT DISK

Entry Parameters: Register C: 1CH Returned value: Register A: Return code

The Write Protect Disk function provides temporary write

protection for the currently selected disk by marking the drive as

read/only. No process can write to a disk that is in the read/only

state. A successful drive reset operation must be performed for a

read/only drive to restore it to the read/write state (see Functions

13 and 37).

MP/M II Programmer's Guide     2.4		BDOS Calls:	Function 28

The Write Protect Disk function is conditional under MP/M II.

If another process has an open file on the drive, this function is

denied and the value OFFH is returned to the calling process.

otherwise, register A is set to zero. Note that a drive in the

read/only state cannot be reset by a process if another process has

an open file on the drive.

FUNCTION 29: GET READ/ONLY VECTOR

Entry Parameters: Register C: 1DH Returned Value: Registers HL: R/O Vector Value

Function 29 returns a bit vector in register pair HL that

indicates which drives have the temporary read/only bit set. The

read/only bit is set either by a BDOS Write Protect Disk call, or by

the automatic software mechanisms within MP/M II that detect changed

disk media.

The format of the bit vector is analogous to that of the login

vector returned by Function 24. The least significant bit

corresponds to drive A, while the most significant bit corresponds

to drive P.

FUNCTION 30: SET FILE ATTRIBUTES

Entry Parameters: Register C: 1EH Registers DE: FCB Address Returned Value: Register A: Directory Code

The Set File Attributes function is the only BDOS function that

allows a program to manipulate file attributes. Other BDOS

functions can interrogate these file attributes but cannot change

them. The file attributes that can be set or reset by Function 30

are: fl' through f4’, R/O (tl'), System (t2’), and Archive (t3').

The register pair DE addresses an FCB containing a filename with the

appropriate attributes set or reset. The calling process must

ensure that it does not specify an ambiguous filename. In addition,

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 30

if the specified file is password protected, the correct password

must be placed in the first eight bytes of the current DMA buffer,

or have been previously established as the default password (see

Function 106).

Function 30 searches the FCB specified directory for an entry

belonging to the current user number that matches the FCB specified

name and type fields. The function then updates the directory to

contain the selected indicators. File attributes tl', t2', and t3'

are defined by MP/M II. They are described in Section 2.2.4.

Attributes fl' through f4’ are not presently used, but may be useful

for application programs, because they are not involved in the

matching process used by the BDOS during Open File and Close File

operations. Indicators f5’ through f8' are reserved for use as

interface attributes.

This function is not performed if the file specified by the

referenced FCB is currently open for another process. It is

performed, however, if the referenced file is open for the calling

process in locked mode. After successfully setting the attributes

of a file opened by the calling process, any subsequent file

reference requiring an open FCB returns a checksum error. This

function does not set the attributes of a file currently open in

read/only or unlocked mode for any process.

Upon return, Function 30 returns a Directory Code in register A

with the value 0 to 3 if the function was successful, or OFFH (255

Decimal) if the file specified by the referenced FCB was not found.

Register H is set to zero in both of these cases. If a physical or

extended error was encountered, the Set File Attributes function

performs different actions depending on the BDOS error mode (see

Function 45). If the BDOS error mode is the default mode, a message

identifying the error is displayed at the console and the process is

terminated. Otherwise, Function 30 returns to the calling process

with register A set to OFFH and register H set to one of the

following physical or extended error codes:

01 : Permanent error

02 : Read/only disk

04 : Select error

05 : File open by another process

07 : File password error

09 : ? in file name or type field

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 31

FUNCTION 31: GET ADDR(DISK PARMS

Entry Parameters: Register C: lFH Returned Value: Registers HL: DPB Address

Function 31 returns in register pair HL, the address of the

XIOS-resident Disk Parameter Block (DPB) for the currently selected

drive. (Refer to the MP/M II System Guide for the format of the

DPB) . The calling process can use this address to extract the disk

parameter values for display or to compute the space on a drive.

If a physical error is encountered when the BDOS error mode is

one of the return modes (see Function 45), Function 31 returns the

value OFFFFH in the register pair HL.

FUNCTION 32: SET/GET USER CODE

Entry Parameters: Register C: 20H Register E: OFFH (get) or User Code (set) Returned Value: Register A: Current Code or (no value)

A process can change or interrogate the currently active user

number by calling Function 32. If register E = OFFH, then the value

of the current user number is returned in register A where the value

is in the range of 0 to 15. If register E is not OFFH, then the

current user number is changed to the value of E (modulo 16).

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 33

FUNCTION 33: READ RANDOM

Entry Parameters: Register C: 21H Registers DE: FCB Address Returned Value: Register A: Error Code Register H: Physical Error

The Read Random function is similar to the Read Sequential

function except that the read operation takes place at a particular

random record number, selected by the 24-bit value constructed from

the three byte (rO, rl, r2) field beginning at position 33 of the

FCB. Note that the sequence of 24 bits is stored with the least

significant byte first (rO) , the middle byte next (rl) , and the high

byte last (r2). The random record number can range from 0 to

242,143. This corresponds to a maximum value of 3 in byte r2.

To read a file with Function 33, the calling process must first

open the base extent (extent 0). This ensures that the FCB is

properly initialized for subsequent random access operations. (The

base extent may or may not contain any allocated data) . Function 33

places the specified record number in the random record field, and

then BDOS reads the record into the current DMA address. The

function automatically sets the logical extent and current record

values, but unlike the Read Sequential function, it does not advance

the record number. Thus a subsequent Read Random call re-reads the

same record. After a random read operation, a file can be accessed

sequentially, starting from the current randomly accessed position.

However, the last randomly accessed record is re-read or re-written

when switching from random to sequential mode.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 33

If the BDOS Multi-Sector count is greater than one (see

Function 44), the Read Random function reads multiple consecutive

records into memory beginning at the current DMA. The rO,rl, and r2

field of the FCB is automatically incremented to read each record.

However, the FCBs random record number is restored to the first

record's value upon return to the calling process. Upon return, the

Read Random function sets register A to zero if the read operation

was successful. otherwise, register A contains one of the following

error codes:

01 :	Reading unwritten data

03 :	Cannot Close current extent

04 :	Seek to unwritten extent

06 :	Random record number out of range

10 :	FCB checksum error

11 :	Unlocked file verification error

255 :	Physical error : refer to register H

Error Code 01 is returned when the Read Random function

accesses a data block that has not been previously written.

Error Code 03 is returned when the Read Random function cannot

close the current extent prior to moving to a new extent.

Error Code 04 is returned when a read random operation accesses

an extent that has not been created.

Error Code 06 is returned when byte 35 (r2) of the referenced

FCB is greater than 3.

Error Code 10 is returned if the referenced FCB failed the FCB

checksum test.

Error Code 11 is returned if the BDOS cannot locate the FCB's

directory entry when attempting to verify that the referenced FCB

contains current information. This error is only returned for files

open in unlocked mode.

Error Code 255 is returned if a physical error was encountered

and the BDOS error mode is one of the return modes (see Function

45). If the error mode is the default mode, a message identifying

the physical error is displayed at the console and the calling

process is terminated. When a physical error is returned to the

calling process, it is identified by the four low-order bits of

register H as shown below:

01 : Permanent Error

04 : Select Error

The Read Random function also sets the four high-order bits of

register H on all error returns when the BDOS Multi-Sector Count is

greater than one. In this case, the four bits contain an integer

set to the number of records successfully read before the error was

encountered. This value can range from 0 to 15. The high-order

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 33

four bits of register H are always zeroed when the Multi-Sector

Count is equal to one.

FUNCTION 34: WRITE RANDOM

Entry Parameters: Register C: 22H Registers DE: FCB Address Returned Value: Register A: Error Code Register H: Physical error

The Write Random function is analogous to the Read Random

Function, except that data is written to the disk from the current

DMA address. If the disk extent and/or data block where the data is

to be written is not already allocated, the BDOS automatically

performs the allocation before the write operation continues.

To write to a file using the Write Random function, the calling

process must first open the base extent (extent 0). This ensures

that the FCB is properly initialized for subsequent random access

operations. The base extent may or may not contain any allocated

data, but opening extent 0 records the file in the directory so that

it is can be displayed by the DIR utility. If a process does not

open extent 0 and allocates data to some other extent, the file will

be invisible to the DIR utility.

The Write Random function sets the logical extent and current

record positions to correspond with the random record being written,

but does not change the random record number. Thus sequential read

or write operations can follow a random write, with the current

record being re-read or re-written as the calling process switches

from random to sequential mode.

If the BDOS Multi-Sector count is greater than one (see

Function 44), the Write Random function reads multiple consecutive

records into memory beginning at the current DMA. The rO,rl, and r2

field of the FCB is automatically incremented to write each record.

However, the FCB's random record number is restored to the first

record's value upon return to the calling process. Upon return, the

Write Random function sets register A to zero if the write operation

was successful.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 34

Otherwise, register A contains one of the following error codes:

02 : No available data block

03 : Cannot Close current extent

05 : No available directory space

06 : Random record number out of range

08 : Record locked by another process

10 : FCB checksum error

11 : Unlocked file verification error

255 : Physical error : refer to register H

Error Code 02 is returned when the write command attempts to

allocate a new data block to the file and no unallocated data blocks

exist on the selected disk drive.

Error Code 03 is returned when the Read Random function cannot

close the current extent prior to moving to a new extent.

Error Code 05 is returned when the write function attempts to

create a new extent that requires a new directory entry and no

available directory entries exist on the selected disk drive.

Error Code 06 is returned when byte 35 (r2) of the referenced

FCB is greater than 3.

Error Code 08 is returned when the Write Random function

attempts to write to a record locked by another process. This error

is only returned for files open in unlocked mode.

Error Code 10 is returned if the referenced FCB failed the FCB

checksum test.

Error Code 11 is returned if the BDOS cannot locate the FCB's

directory entry when attempting to verify that the referenced FCB

contains current information. This error is only returned for files

open in unlocked mode.

Error Code 255 is returned if a physical error was encountered

and the BDOS error mode is one of the return modes (see Function

45). If the error mode is the default mode, a message identifying

the physical error is displayed at the console and the calling

process is terminated. When a physical error is returned to the

calling process, it is identified by the four low-order bits of

register H as shown below:

01 :	Permanent error

02 :	Read/only disk

03 :	Read/only file

File open in read/only mode

File password protected in Write mode

04 :	Select Error

The Write Random function also sets the four high-order bits of

register H on all error returns when the BDOS Multi-Sector Count is

greater than one. In this case, the four bits contain an integer

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 34

set to the number of records successfully read before the error was

encountered. This value can range from 0 to 15. The high-order

four bits of register H are always zeroed when the Multi-Sector

Count is equal to one.

FUNCTION 35: COMPUTE FILE SIZE

Entry Parameters: Register C: 23H Registers DE: FCB Address Returned Value: Register A: Error Flag Register H: Physical or Extended error Random Record Field Set

The Compute File Size function determines the "virtual" file

size, which is, in effect, the address of the record immediately

following the end of the file. The "virtual" size of a file

corresponds to the physical size if the file is written

sequentially. If the file is written in random mode, gaps might

exist in the allocation, and the file might contain fewer records

than the indicated size. For example, if a single record with

record number 262,143 (the MP/M II maximum) is written to a file

using the Write Random function, then the "virtual" size of the file

is 262,144 records even though only 1 data block is actually

allocated.

To compute file size, the calling process passes in register

pair DE, the address of a FCB in random mode format (bytes rO, rl

and r2 present) . Note that the FCB must contain an unambiguous

filename and type. Function 35 sets the random record field of the

FCB to the random record number + 1 of the last record in the file.

If the r2 byte is set to 04, then the file contains the maximum

record count 262,144.

A process can append data to the end of an existing file by

calling Function 35 to set the random record position to the end of

file, then performing a sequence of random writes starting at the

preset record address.

Note:		the BDOS does not require that the file be open to use

Function 35.

Upon return, Function 35 returns a zero in register A if the

file specified by the referenced FCB was found, or a OFFH in

register A if the file was not found. Register H is set to zero in

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 35

both of these cases. If a physical or extended error was

encountered, Function 35 performs different actions depending on the

BDOS error mode (see Function 45). If the BDOS error mode is the

default mode, a message identifying the error is displayed at the

console and the process is terminated. Otherwise, Function 35

returns to the calling process with register A set to OFFH and

register H set to one of the following physical or extended errors:

01 : Permanent error

04 : Select error

09 : ? in file name or type field

FUNCTION 36: SET RANDOM RECORD

Entry Parameters: Register C: 24H Registers DE: FCB Address Returned Value: Random Record Field Set

The Set Random Record function returns the random record number

of the next record to be accessed from a file that has been read or

written sequentially to a particular point. This value is returned

in the random record field (bytes rO, rl, and r2) of the FCB

addressed by the register pair DE. Function 36 can be useful in two

ways.

First, it is often necessary to initially read and scan a

sequential file to extract the positions of various "key" fields.

As each key is encountered, Function 36 is called to compute the

random record position for the data corresponding to this key. If

the data unit size is 128 bytes, the resulting record number minus

one is placed into a table with the key for later retrieval. After

scanning the entire file and tabularizing the keys and their record

numbers, you can move directly to a particular record by performing

a random read using the corresponding random record number that was

saved earlier. The scheme is easily generalized when variable

record lengths are involved since the program need only store the

buffer-relative byte position along with the key and record number

to find the exact starting position of the keyed data at a later

time.

A second use of Function 36 occurs when switching from a

sequential read or write over to random read or write. A file is

sequentially accessed to a particular point in the file, Function 36

is called which sets the record number, and subsequent random read

and write operations continue from the next record in the file.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 37

FUNCTION 37: RESET DRIVE

Entry Parameters: Register C: 25H Register DE: Drive Vector Returned Value Register A: Return Code

The Reset Drive function is used to programmatically restore

specified drives to the reset state (a reset drive is not logged-in

and is in read/write status) . The passed parameter in register pair

DE is a 16 bit vector of drives to be reset, where the least

significant bit corresponds to the first drive A, and the high-order

bit corresponds to the sixteenth drive, labeled P. Bit values of

"l" indicate that the specified drive is to be reset.

This function is conditional under MP/M II. If another process

has a file open on a drive to be reset, and the drive is removeable

or read/only, the Drive Reset function is denied and no drives are

reset.

Upon return, if the reset operation is successful, register A

is set to zero. Otherwise, register A is set to OFFH (255 decimal) .

If the BDOS error mode is not Return Error mode (see Function 45),

then an error message is displayed at the console, identifying a

process owning an open file.

FUNCTION 38: ACCESS DRIVE

Entry Parameters: Register C: 26H Register DE: Drive Vector Returned Value: Register A: Return Code Register H: Extended Error

The Access Drive function inserts a special open file item into

the system lock list for each specified drive. While the item

exists in the lock list, the drive cannot be reset by another

process. As in Function 37, the calling process passes the drive

vector in register pair DE. The format of the drive vector is the

same as that used in Function 37.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 38

The Access Drive function inserts no items if insufficient free

space exists in the lock list to support all the new items or if the

number of items to be inserted puts the calling process over the

lock list open file maximum. This maximum is a MP/M II Gensys

option. If the BDOS error mode is the default mode (see Function

45), a message identifying the error is displayed at the console and

the calling process is terminated. Otherwise, the Access Drive

function returns to the calling process, register A is set to OFFH

and register H is set to one of the following values.

10 : Process Open File limit exceeded

11 : No room in the system lock list

Register A is set to zero if the Access Drive function is

successful.

FUNCTION 39: FREE DRIVE

Entry Parameters: Register C: 27H Register DE: Drive Vector

The Free Drive function purges the open lock list of all file

and locked record items that belong to the calling process on the

specified drives. As in Function 38, the calling process passes the

drive vector in register pair DE.

Function 39 does not close files associated with purged open

file lock list items. In addition, if a process references a

"purged" file with a BDOS function requiring an open FCB, a checksum

error is returned. A file that has been written to should be closed

before making a Free Drive call to the file's drive. otherwise data

may be lost.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 40

FUNCTION 40: WRITE RANDOM WITH ZERO FILL

Entry Parameters: Register C: 28H Register DE: FCB address Returned Value: Register A: Error Code Register H: Physical Error

The Write Random With Zero Fill function is similar to the

Write Random function (Function 34) with the exception that a

previously unallocated data block is filled with zeroes before the

record is written. If this function has been used to create a file,

records accessed by a read random operation that contain all zeroes

identify unwritten random record numbers. Unwritten random records

in allocated data blocks of files created using the Write Random

function contain uninitialized data.

FUNCTION 41: TEST AND WRITE RECORD

Entry Parameters: Register C: 29H DE: FCB Address Returned Value: Register A: Error Code Register H: Physical Error

The Test and Write Record provides a means of verifying the

current contents of a record on disk before updating it. The

calling process must set bytes rO, rl, and r2 of the FCB addressed

by register pair DE to the random record number of the record to be

tested. The original version of the record (i.e. the record to be

tested) must reside at the current DMA address, followed immediately

by the new version of the record. The record size can range from

128 bytes to sixteen times that value depending on the BDOS Multi

Sector Count (see Function 44).

Function 41 verifies that the first record is identical to the

record on disk before replacing it with the new version of the

record. If the record on disk does not match, the record on disk is

not changed and an error code is returned to the calling process.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 41

The Test and Write function is intended for use in situations

where more than one process has read/write access to a common file.

This situation is supported under MP/M II, when more than one

process opens the same file in unlocked mode. Function 41 is a

logical replacement for the record lock/unlock sequence of

operations because it prevents two processes from simultaneously

updating the same record. Note that this function is also supported

for files open in locked mode to provide compatibility between MP/M

II and CP/M.

Upon return, the Test and Write Random function sets register A

to zero if the function was successful. otherwise, register A

contains one of the following error codes:

01 : Reading unwritten data

03 : Cannot Close current extent

04 : Seek to unwritten extent

06 : Random record number out of range

07 : Records did not match

08 : Record locked by another process

10 : FCB checksum error

11 : Unlocked file verification error

255 : Physical error : refer to register H

Error Code 01 is returned when the Test and Write function

accesses a data block that has not been previously written.

Error Code 03 is returned when the Test and Write function

cannot close the current extent prior to moving to a new extent.

Error Code 04 is returned when a read operation accesses an

extent that has not been created.

Error Code 06 is returned when byte 35 (r2) of the referenced

FCB is greater than 3.

Error Code 07 is returned when the Test and Write record test

fails.

Error Code 08 is returned if the specified record is locked by

another process. This error is only returned for files open in

unlocked mode.

Error Code 10 is returned if the referenced FCB failed the FCB

checksum test.

Error Code 11 is returned if the BDOS cannot locate the FCB's

directory entry when attempting to verify that the referenced FCB

contains current information. This error is only returned for files

open in unlocked mode.

Error Code 255 is returned if a physical error was encountered

and the BDOS error mode is one of the return modes (see Function

45). If the error mode is the default mode, a message identifying

the physical error is displayed at the console and the calling

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 41

process is terminated. When a physical error is returned to the

calling process, it is identified by the four low-order bits of

register H as shown below:

01 :	Permanent error

02 :	Read/only disk

03 :	Read/only file or

File open in read/only mode

File password protected in Write mode

04 :	Select Error

The Test and Write function also sets the four high-order bits

of register H on all error returns when the BDOS Multi-Sector Count

is greater than one. In this case, the four bits contain an integer

set to the number of records successfully tested or written before

the error was encountered. This value can range from 0 to 15. The

high-order four bits of register H are always zeroed when the Multi

Sector Count is equal to one.

FUNCTION 42: LOCK RECORD

Entry Parameters: Register C: 2AH DE: FCB Address Returned Value: Register A: Error Code Register H: Physical Error

The Lock Record function locks one or more consecutive records

so that no other program with access to the records can

simultaneously lock or update them. This function is only supported

for files open in unlocked mode. If it is called for a file open in

locked or read/only mode, no locking action is performed and a

successful result is returned. This is done to provide

compatibility between MP/M II and CP/M.

The calling process passes in register pair DE, the address of

an FCB in which the Random Record Field is filled with the random

record number of the first record to be locked. The number of

records to be locked is determined by the BDOS Multi-Sector Count

(see Function 44) . The current DMA must contain the 2-byte File ID

returned by the Open File function when the referenced FCB was

opened. Note that the File ID is only returned by the Open function

when the open mode is unlocked.

The Lock Record function requires that each record number to be

locked reside in an allocated block for the file. In addition,

Function 42 verifies that none of the records to be locked are

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 42

currently locked by another process. Both of these tests are made

before any records are locked.

A MP/M II system generation parameter specifies the maximum

number of records that may be locked by a single process. Each

locked record consumes an entry in the BDOS system lock table which

is shared by locked record and open file entries. Another MP/M II

system generation parameter sets the size of this table. If there

is not sufficient space in the system lock table to lock all the

specified records, or the process record lock limit is exceeded,

then the Lock Record function locks no records and returns an error

code to the calling process.

Upon return, the Lock Record function sets register A to zero

if the lock operation was successful. Otherwise, register A

contains one of the following error codes:

01 : Reading unwritten data

03 : Cannot Close current extent

04 : Seek to unwritten extent

06 : Random record number out of range

08 : Record locked by another process

10 : FCB checksum error

11 : Unlocked file verification error

12 : Process record lock limit exceeded

13 : Invalid File ID

14 : No room in the system lock list

255 : Physical error : refer to register H

Error Code 01 is returned when the Lock Record function

accesses a data block that has not been previously written.

Error Code 03 is returned when the Lock Record function cannot

close the current extent prior to moving to a new extent.

Error Code 04 is returned when the Lock Record function

accesses an extent that has not been created.

Error Code 06 is returned when byte 35 (r2) of the referenced

FCB is greater than 3.

Error Code 08 is returned if the specified record is locked by

another process.

Error Code 10 is returned if the referenced FCB failed the FCB

checksum test.

Error Code 11 is returned if the BDOS cannot locate the

referenced FCB's directory entry when attempting to verify that the

FCB contains current information.

Error Code 12 is returned when the sum of the number of records

currently locked by the calling process and the number of records to

be locked by the Lock Record call, exceeds the maximum allowed

value. This value is an MP/M II Gensys parameter.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 42

Error Code 13 is returned when an invalid File ID is placed in

the current DMA.

Error Code 255 is returned if a physical error was encountered

and the BDOS error mode is one of the return modes (see Function

45). If the error mode is the default mode, a message identifying

the physical error is displayed at the console and the calling

process is terminated. When a physical error is returned to the

calling process, it is identified by the four low-order bits of

register H as shown below:

01 : Permanent error

04 : Select Error

The Lock Record function also sets the four high-order bits of

register H on all error returns when the BDOS Multi-Sector Count is

greater than one. In this case, the four bits contain an integer

set to the number of records successfully locked before the error

was encountered. This value can range from 0 to 15. The high-order

four bits of register H are always zeroed when the Multi-Sector

Count is equal to one.

FUNCTION 43: UNLOCK RECORD

Entry Parameters: Register C: 2BH DE: FCB Address Returned Value: Register A: Error Code Register H: Physical Error

The Unlock Record function unlocks one or more consecutive

records previously locked by the Lock Record function. This

function is only supported for files open in unlocked mode. If it

is called for a file open in locked or read/only mode, no locking

action is performed and a successful result is returned. This is

done to provide compatibility between MP/M II and CP/M.

The calling process passes in register pair DE, the address of

an FCB in which the Random Record Field is filled with the random

record number of the first record to be unlocked. The number of

records to be unlocked is determined by the BDOS Multi-Sector Count

(see Function 44) . The current DMA must contain the 2-byte File ID

returned by the Open File function when the referenced FCB was

opened. Note that the File ID is only returned by the open function

when the open mode is unlocked.

MP/M II Programmer's Guide     2.4		BDOS Calls:	Function 43

The Unlock Record function will not unlock a record that is

currently locked by another process. However, no error is returned

if a process attempts to do that. Thus, if the Multi-Sector Count

is greater than one, the Unlock Record function unlocks all records

locked by the calling process, while skipping those records locked

by other processes.

Upon return, the Unlock Record function sets register A to zero

if the unlock operation was successful. otherwise, register A

contains one of the following error codes:

01 : Reading unwritten data

03 : Cannot Close current extent

04 : Seek to unwritten extent

06 : Random record number out of range

10 : FCB checksum error

11 : Unlocked file verification error

13 : Invalid File ID

255 : Physical error : refer to register H

Error Code 01 is returned when the Unlock Record function

accesses a data block that has not been previously written.

Error Code 03 is returned when the Unlock Record function

cannot close the current extent prior to moving to a new extent.

Error Code 04 is returned when the Unlock Record function

accesses an extent that has not been created.

Error Code 06 is returned when byte 35 (r2) of the referenced

FCB is greater than 3.

Error Code 10 is returned if the referenced FCB failed the FCB

checksum test.

Error Code 11 is returned if the BDOS cannot locate the

referenced FCB's directory entry when attempting to verify that the

FCB contains current information.

Error Code 13 is returned when an invalid File ID is placed in

the current DMA.

Error Code 255 is returned if a physical error was encountered

and the BDOS error mode is one of the return modes (See function

45). If the error mode is the default mode, a message identifying

the physical error is displayed at the console and the calling

process is terminated. When a physical error is returned to the

calling process, it is identified by the four low-order bits of

register H as shown below:

01 : Permanent error

04 : Select Error

The Unlock Record function also sets the four high-order bits

of register H on all error returns when the BDOS Multi-Sector Count

is greater than one. In this case, the four bits contain an integer

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 43

set to the number of records successfully locked before the error

was encountered. This value can range from 0 to 15. The high-order

four bits of register H are always zeroed when the Multi-Sector

Count is equal to one.

FUNCTION 44: SET MULTI-SECTOR CNT

Entry Parameters: Register C: 2CH E: Number of Sectors Returned Value: Register A: Return Code

The Set Multi-Sector Count function provides logical record

blocking under MP/M II. It enables a process to read and write from

1 to 16 "physical" records of 128 bytes at a time during subsequent

BDOS Read and Write functions. It also specifies the number of 128

byte records to be locked or unlocked by the BDOS Lock and Unlock

functions.

Function 44 sets the Multi-Sector Count value for the calling

process to the value passed in register E. Once set, the specified

Multi-Sector Count remains in effect until the calling process makes

another Set Multi-Sector Count function call and changes the value.

Note that the Command Line Interpreter (CLI) sets the Multi-Sector

Count to one when it initiates a transient program.

The Multi-Sector count affects BDOS error reporting for the

BDOS read, write, lock and unlock functions. If an error interrupts

these functions when the Multi-Sector is greater than one, they

return the number of records successfully processed in the high

order four bits of register H.

Upon return, register A is set to zero if the specified value

is in the range of 1 to 16. Otherwise, register A is set to OFFH

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 45

FUNCTION 45: SET BDOS ERROR MODE

Entry Parameters: Register C: 2DH E: BDOS error mode Returned Value: None

The SET BDOS Error Mode function determines how physical and

extended errors (see Section 2.2.13) are handled for a process. The

Error Mode can exist in three modes: the default mode, Return Error

mode and Return and Display Error mode. In the default mode, BDOS

displays a system message at the console identifying the error and

terminates the calling process. In the return modes, BDOS sets

register A to OFFH (255 Decimal), places an error code identifying

the physical or extended error in the four low-order bits of

register H, and returns to the calling process. In Return and

Display mode, BDOS displays the system message before returning to

the calling process. No system messages are displayed, however,

when BDOS is in Return Error mode.

Function 45 sets the BDOS error mode for the calling process to

the mode specified in register E. If register E is set to OFFH (255

Decimal), the error mode is set to Return Error mode. If register E

is set to OFEH (254 Decimal), the error mode is set to Return and

Display mode. If register E is set to any other value, the error

mode is set to the default mode.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 46

FUNCTION 46: GET DISK FREE SPACE

Entry Parameters: Register C: 2EH E: Drive Returned Value: First 3 bytes of current DMA buffer Register A: Error Flag Register H: Physical error

The Get Disk Free Space function determines the number of free

sectors (128 byte records) on the specified drive. The calling

process passes the drive number in register E, with 0 for drive A, 1

for B, etc. , through 15 for drive P in a full 16 drive system.

Function 46 returns a binary number in the first 3 bytes of the

current DMA buffer. This number is returned in the following

format:

| fsO | fsl | fs2 |
| --- | --- | --- |

Disk Free Space Field Format

fso = low byte

fsl = middle byte

fs2 = high byte

Upon return, register A is set to zero if the BDOS Error Mode is the

default mode. However, if the BDOS Error Mode is one of the return

modes (see Function 45) and a physical error was encountered,

register A is set to OFFH (255 Decimal), and register H is set to

one of the following values:

01 - Permanent error

04 - Select error

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 47

FUNCTION 47: CHAIN TO PROGRAM

Entry Parameters: Register C: 2FH

The Chain To Program function provides a means of chaining from

one program to the next without operator intervention. Although

there is no passed parameter for this call, the calling process must

place a command line terminated by a null byte in the default DMA

buffer.

Function 47 does not return any values to the calling process

because any errors encountered are handled by the Command Line

Interpreter (CLI).

Note: Function 47 makes an XDOS Conditional Attach Console

call for the calling process. If the calling process is detached

from its console, the program chain is not performed and Function 47

returns to the calling process.

FUNCTION 48: FLUSH BUFFERS

Entry Parameters: Register C: 30H Returned Value: Register A: Error Flag Register H: Permanent Error

The Flush Buffers function forces the write of any write

pending records contained in internal blocking/deblocking buffers.

This function only affects those systems that have implemented a

write-deferring blocking/deblocking algorithm in their XIOS (see

Section 2.2.12).

Upon return, register A is set to zero if the flush operation

was successful. If a physical error was encountered, the Flush

Buffers function performs different actions depending on the BDOS

error mode (see Function 45). If the BDOS error mode is in the

default mode, a message identifying the error is displayed at the

console and the calling process is terminated. otherwise, the Flush

Buffers function returns to the calling process with register A set

to OFFH and register H set to the following physical error code:

01 : Permanent error

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 100

FUNCTION 100: SET DIRECTORY LABEL

Entry Parameters: Register C: 64H Register DE: FCB Address Returned Value: Register A : Directory Code Register H : Physical or Extended Error

The Set Directory Label function creates a directory label or

updates the existing directory label for the specified drive. The

calling process passes in register pair DE, the address of an FCB

containing the name, type, and extent fields to be assigned to the

directory label. The name and type fields of the referenced FCB are

not used to locate the directory label in the directory; they are

simply copied into the updated or created directory label. The

extent field of the FCB (byte 12) contains the user's specification

of the directory label data byte. The definition of the directory

abel data byte is:

bit 	7 - Require passwords for password-protected files

6 - Perform access date and time stamping

5 - Perform update date and time stamping

4 - Make function creates XFCBs

0 - Assign a new password to the directory label

If the current directory label is password protected, the correct

password must be placed in the first 8 bytes of the current DMA or

have been previously established as the default password (see

function 106). If bit 0 (the low-order bit) of byte 12 of the FCB

is set to 1, it indicates that a password for the directory label

has been placed in the second eight bytes of the current DMA.

Function 100 returns a Directory Code in register A with a

value from 0 to 3 if the directory label create or update was

successful, or OFFH (255 Decimal) if no space existed in the

referenced directory to create a directory label. Register H is set

to zero in both of these cases. If a physical error or extended was

encountered, function 100 performs different actions depending on

the BDOS error mode (see function 45). If the BDOS error mode is

the default mode, a message identifying the error is displayed at

the console and the calling process is terminated. otherwise,

function 100 returns to the calling process with register A set to

OFFH and register H set to one of the following physical or extended

error codes:

01 : Permanent error

02 : Read/only disk

04 : Select Error

07 : File password error

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 101

FUNCTION 101: RETURN DIRECTORY LABEL DATA

Entry Parameters: Register C: 65H Register E: Drive Returned Value: Registers A : Directory label Data Byte Register H : Physical Error

The Return Directory Label Data function returns the data byte

of the directory label for the specified drive. The calling process

passes the drive number in register E with 0 for drive A, 1 for

drive B, and so on through 15 for drive P in a full sixteen drive

system. The format of the directory label data byte is shown below:

bit	7 - Require passwords for password protected files

6 - Perform access date and time stamping

5 - Perform update data and time stamping

4 - Make function creates XFCBs

0 - Directory label exists on drive

Function 101 returns the directory label data byte to the calling

process in register A. Register A equal to zero indicates that no

directory label exists on the specified drive. If a physical error

is encountered by function 101 when the BDOS Error mode is in one of

the return modes (see function 45) , this function returns with

register A set to OFFH (255 Decimal) and register H set to one of

the following:

01 : Permanent error

04 : Select error

MP/M II Programmer's Guide		2.4	BDOS Calls:	Function 102

FUNCTION 102: READ FILE XFCB

Entry Parameters: Register C: 66H Register DE: FCB Address Returned Value: Register A : Directory Code Register H : Physical Error

The Read File XFCB function reads the directory XFCB

information for the specified file into bytes 20 through 32 of the

specified FCB. The calling process passes in register pair DE, the

address of an FCB in which the drive, filename, and type fields have

been defined.

If function 102 is successful, it sets the following fields in

the referenced FCB:

byte 12 :	XFCB password mode field

bit 7 - Read mode

bit 6 - Write mode

bit 5 - Delete mode

Byte 12 equal to zero indicates the file

has not been assigned a password.

byte 13 - 23 : XFCB password field (encrypted)

byte 24 - 27 : XFCB Create or Access time stamp field

byte 28 - 31 : XFCB Update time stamp field

Upon return, function 102 returns a Directory Code in register

A with the value 0 to 3 if the XFCB read operation was successful,

or OFFH (255 Decimal) if the XFCB was not found. Register H is set

to zero in both of these cases. If a physical error or extended was

encountered, function 102 performs different actions depending on

the BDOS error mode (see function 45) . If the BDOS error mode is in

the default mode, a message identifying the error is displayed at

the console and the calling process is terminated. Otherwise,

function 102 returns to the calling process with register A set to

OFFH and register H set to one of the following physical error

codes:

01 : Permanent error

04 : Select Error

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 103

FUNCTION 103: WRITE FILE XFCB

Entry Parameters: Register C: 67H Register DE: FCB Address Returned Value: Register A : Directory Code Register H : Physical or Extended Error

The Write File XFCB function creates a new XFCB or updates the

existing XFCB for the specified file. The calling process passes in

register pair DE, the address of an FCB in which the drive, name,

type, and extent fields have been defined. The "ex" field, if set,

specifies the password mode and whether a new password is to be

assigned to the file. The for-mat of the extent byte is shown below:

FCB	byte 12	(ex) : XFCB password mode

bit 7 -	Read mode

bit 6 -	Write mode

bit 5 -	Delete mode

bit 0 -	assign new password to the file

If bit 0 is set to 1, the new password must reside in the second 8

bytes of the current DMA. If the FCB is currently password

protected, the correct password must reside in the first 8 bytes of

the current DMA, or have been previously established as the default

password (see function 106).

Upon return, function 100 returns a Directory Code in register

A with the value 0 to 3 if the XFCB create or update was successful,

or OFFH (255 Decimal) if no directory label existed on the specified

drive, or the file named in the FCB was not found, or no space

existed in the directory to create an XFCB. Register H is set to

zero in all of these cases. If a physical error or extended was

encountered, function 103 performs different actions depending on

the BDOS error mode (see function 45). If the BDOS error mode is

the default mode, a message identifying the error is displayed at

the console and the calling process is terminated. otherwise,

function 103 returns to the calling process with register A set to

OFFH and register H set to one of the following physical or extended

error codes:

01 : Permanent error

02 : Read/only disk

04 : Select Error

07 : File password error

MP/M II Programmer's Guide     2.4		BDOS Calls:	Function 104

FUNCTION 104: SET DATE AND TIME

Entry Parameters: Register C: 68H Register DE: TOD Address Returned Value: none

The Set Date and Time function sets the system internal date

and time. The calling process passes the address of a 4-byte

structure containing the date and time specification in the register

pair DE. The format of the date and time data structure is:

byte 0 - 1		: Date field

byte 2		: Hour field

byte 3		: Minute field

The date is represented as a 16-bit integer with day 1 corresponding

to January 1, 1978. The time is represented as two bytes: hours and

minutes stored as two BCD digits.

Under MP/M II, this function also sets the second field of the

system date and time to zero.

FUNCTION 105: GET DATE AND TIME

Entry Parameters: Register C: 6911 Register DE: TOD Address Return Value TOD

The Get Date and Time function obtains the system internal date

and time. The calling process passes in register pair DE, the

address of a four-byte data structure which receives the date and

time values. The format of the data structure is the same as the

format described in function 104. This function is equivalent to

MP/M II function 155 except that it does not return the seconds

field of the internal time.

MP/M II Programmer's Guide	2.4		BDOS Calls:	Function 106

FUNCTION 106: SET DEFAULT PASSWORD

Entry Parameters: Register C: 6AH Register DE: Password Address Returned Value: none

The Set Default Password function allows a process to specify a

password value before a file protected by the password is accessed.

When the file system accesses a password protected file, it checks

the current DMA and the default password for the correct value. A

password error is not returned if either password is correct. The

default password is maintained by the BDOS in an internal table

indexed by the calling process's console number. once assigned, it

is maintained until another Set Default Password call is made by a

process having the same console number.

To make a function 106 call, the calling process sets register

pair DE to the address of an eight byte field containing the

password.

FUNCTION 107: RETURN SERIAL NUMBER

Entry Parameters: Register C: 6BH Register DE: Serial number field

Function 107 returns the MP/M II serial number to the six-byte

field addressed by register pair DE.

Section 3

XDOS Interface

3.1		Introduction

This section contains information on data structures used in

the XDOS module. The XDOS uses these data structures to:

• manage the memory resource

• communicate messages between processes

• synchronize process execution

Also included are descriptions of the XDOS functions, including the

entry parameters and returned values, and a discussion of error

handling by the XDOS. The reader should be thoroughly familiar with

the material covered in Section 1 before proceeding.

3.2		Process Descriptor Data Structure

Each process running under MP/M II is associated with a Process

Descriptor that defines all the characteristics of the process. The

XDOS uses the Process Descriptor to save and restore the state of a

process. The Process Descriptor data structure is shown below in

both PL/M and assembly language.

PL/M:

DECLARE CNS$HNDLR STRUCTURE (

PL ADDRESS,

STATUS BYTE,

PRIORITY BYTE,

STKPTR ADDRESS,

NAME (8) BYTE,

CONSOLE$LIST BYTE,

MEMSEG BYTE,

DPARAM ADDRESS,

THREAD ADDRESS,

DISK$SET$DMA ADDRESS,

DISK$SLCT BYTE,

DCNT ADDRESS,

SEARCHL BYTE,

SEARCHA ADDRESS,

PD EXTENT,

REGISTERS (10) ADDRESS,

EXTENSION ADDRESS,

INITIAL (0,0,200,.CNS$STK(19),

'CNS     ‘,1,OFFH);

MP/M II Programmer's Guide	3.2		Process Descriptor

DECLARE CNS$STK (20) ADDRESS INITIAL (

OC7C7H,OC7C7H,OC7C7H,OC7C7H,OC7C7H,OC7C7H,

OC7C7H,OC7C7H,OC7C7H,OC7C7H,OC7C7H,OC7C7H,

OC7C7H,OC7C7H,OC7C7H,OC7C7H,OC7C7H,OC7C7H,

OC7C7H,STRT$CNS);

Assembly Language:

CNSHND:

DW	0	;PL

DB	0	;STATUS

DB	200	;PRIORITY

DW	CNSTK+38 ;STKPTR

DB	'CNS     ‘ ; NAME

DB	0	;CONSOLE/LIST

DB	OFFH	;MEMSEG (FF = resident)

DS	2	;DPARAM

DS	2	;THREAD

DS	2	;DISK SET DMA

DS	I	;DISK SLCT

DS	2	;DCNT

DS	1	;SEARCHL

DS	2	;SEARCHA

DS	2	;PD EXTENT

;REGISTERS:

DS	2	;HL'

DS	2	;DE'

DS	2	;BC'

DS	2	;AF'

DS	2	;IY

DS	2	;IX

DS	2	;HL

DS	2	;DE

DS	2	;BC

DS	2	;AF

DS	2	;EXTENSION

CNSTK:

DW	OC7C7H,OC7C7H,OC7C7H,OC7C7H

DW	OC7C7H,OC7C7H,OC7C7H,OC7C7H

DW	OC7C7H,OC7C7H,OC7C7H,OC7C7H

DW	OC7C7H,OC7C7H,OC7C7H

DW	CNSPR		; CNSTK+38 = PROCEDURE ADR

MP/M II Programmer's Guide	3.2		Process Descriptor

The elements of the Process Descriptor data structure shown

above are defined in Table 3-1.

Table 3-1. Process Descriptor Elements

| Element | Definition |
| --- | --- |
| PL 2-byte link field, initially set by user when creating a process to the address of next Process Descriptor, or zero if no more exist. STATUS 1 byte, process status, set by system. The Dispatcher reads the status byte to determine the operation to be performed on the process. The values of the status byte are shown below: 00 - process is ready to run 01 - process is dequeueing 02 - process is enqueueing 03- process is polling 04 - process is waiting for a flag 05 - process is on delay list 06 - not implemented under MP/M II 07 - terminate process 08 - set process priority 09 - Dispatch 10 - Attach console 11 - Detach console 12 - Set console 13 - Attach list 14 - Detach list PRIORITY 1 byte, process priority, set by user. STKPTR 2 bytes, stack pointer, initially set by user. NAME 8 bytes, ASCII process name, set by user. The high-order bit of each byte of the process name is reserved for use by the system. The function of each of the high order bits , shown as NAME (n)is described below: NAME(O)' The high-order bit of NAME(O) on indicates that the process is performing direct console BIOS calls and that MP/M II should ignore all control characters. It also suppresses the normal console status check done when BDOS disk functions are called. The user may set this bit. |  |

MP/M II Programmer's Guide	3.2		Process Descriptor

Table 3-1. (continued)

| Element | Definition |
| --- | --- |
| NAME(l)' The high-order bit of NAME(l) "on" indicates that the process is currently executing code in the serially re-usable BDOS. MP/M II does not allow a process to Terminate while it is in the BDOS. Any Attempt to abort the process will set NAME(6)' "on". This bit is set by the system; it must not be set by the user. NAME(2)’ The high-order bit of NAME(2) "on" indicates that no stack swap is done for this process upon entering the BDOS. This bit takes precedence over the system boolean indicating whether user system stacks have been allocated. It is required when more than one process shares the same memory segment and makes BDOS function calls. The user may set this bit. NAME(3)' The high-order bit of NAME(3) "on" Indicates that live keyboard simulation is to be suppressed. Live keyboard simulation is done by performing console status calls at each BDOS disk I/O function call. This bit is set by the user. NAME(4)' The high-order bit of NAME(4) "on" indicates that extended errors resulting from BDOS calls are to be returned to the calling program; normally an error message is displayed on the console and the calling program is terminated. This bit is set by the user. NAME(5)' The high-order bit of NAME(5) "on" indicates that extended errors resulting from BDOS calls are to be returned to the calling program and an error message is to be displayed on the console. This bit is set by the user. NAME(6)’ The high-order bit of NAME(6) "on" indicates that an attempt has been made to abort the process while either NAME (1) ' or NAME(7)' has been on. This bit is set by the system. |  |

MP/M II Programmer's Guide	3.2		Process Descriptor

Table 3-1. (continued)

| Element | Definition |
| --- | --- |
| NAME(7)’ The high-order bit of NAME(7) "on" indicates that the process is not to be aborted by any means. An attempt to abort this process results in setting NAME(6)1 "on". This bit is set by the user. CONSOLE/LIST 1 byte, low-order four bits contain the console device number to be used by process, and the high-order four bits contain the list device number, set by user. MEMSEG 1 byte, memory segment table index. DPARAM 2 bytes, reserved for MP/M II. THREAD 2 bytes, process list thread, set by system. DISK$SET$DMA 2 bytes, default DMA address, set by system on BDOS set DMA calls, can be set by user. DISK$SLCT 1 byte, default disk/user code, set by system on BDOS set user and disk select calls, can be set by user. DCNT 2 bytes, reserved for MP/M II. SEARCHL 1 byte, reserved for MP/M II. SEARCHA 2 bytes, reserved for MP/M II. PD EXTENT 2 bytes, reserved for MP/M II. REGISTERS 20 bytes, 8080 / Z80 register save area, can be set by user prior to process creation in order to pass parameters to a created process. The following entries show the register storage allocation. Bytes are stored in the normal 8080/Z80 manner with the low-order register byte preceding the high-order byte. |  |

MP/M II Programmer's Guide	3.2		Process Descriptor

Table 3-1. (continued)

| Element | Definition |
| --- | --- |
| Bytes 0- 1 HL', Alternate Z80 Bytes 2- 3 DE', Alternate Z80 Bytes 4- 5 BC', Alternate Z80 Bytes 6- 7 AF1, Alternate Z80 Bytes 8- 9 IY Bytes 10-11 IX Bytes 12-13 HL Bytes 14-15 DE Bytes 16-17 BC Bytes 18-19 AF EXTENSION 2 bytes, reserved for MP/M II |  |

The following conventions should be used in naming processes

that are to run under MP/M II: processes that wait on queues that

receive command tails from the TMPs should have the same name as the

queue that they read. If a process is to be protected from being

aborted by a user with the ABORT command, its name must have at

least one lower-case character.

3.3		Queue Data Structures

A queue is a first-in first-out (FIFO) mechanism that is

implemented in MP/M II to provide several essential functions in the

multi-programming environment. Queues can be used for the

communication of messages between processes, to synchronize

processes, and to provide mutual exclusion.

MP/M II is designed to simplify queue management for both user

and system processes. Queues are treated like disk files, and can

be created, opened, written to, read from, and deleted.

The queue data structures used by MP/M II include the Queue

Control Block (QCB) and the User Queue Control Block (UQCB). There

are two types of Queue Control Blocks: circular or linked. The

type of QCB used depends upon the size of the message the queue

contains. Message sizes of 0 to 2 bytes use circular queues while

message sizes of 3 or more bytes use linked queues.

3.3.1		Circular Queues

The following example illustrates how to initialize a QCB for a

circular queue containing 80 messages, each of which has a one byte

length. The example is shown in both PL/M and assembly language.

MP/M II Programmer's Guide	3.3		Queue Data Structures

PL/M:

DECLARE CIRCULAR$QUEUE STRUCTURE (

QL ADDRESS,

NAME(8) BYTE,

MSGLEN ADDRESS,

NMBMSGS ADDRESS,

DQPH ADDRESS,

NQPH ADDRESS,

MSG$IN ADDRESS,

MSG$OUT ADDRESS,

MSG$CNT ADDRESS,

BUFFER (80) BYTE        )

INITIAL (O,’CIRCQUE ',1,80);

Assembly Language:

CRCQUE:

DS		2	;QL

DB		'CIRCQUE ‘	; NAME

DW		1	;MSGLEN

DW		80	;NMBMSGS

DS		2	;DQPH

DS		2	;NQPH

DS		2	;MSGIN

DS		2	;MSGOUT

DS		2	;MSGCNT

BUFFER:

DS		80	;BUFFER

The elements of the circular queue shown above are defined in

Table 3-2. The total queue overhead is 24 bytes.

Table 3-2. Circular Queue Elements

| Element | Definition |
| --- | --- |
| QL 2-byte link, set by system. NAME 8 ASCII character queue name, set by user. MSGLEN 2 bytes, length of message, set by user. NMBMSGS 2 bytes, number of messages, set by user. DQPH 2 bytes, Dequeue list process head, set by system. NQPH 2 bytes, Enqueue list process head, set by system. MSG$IN 2 bytes, pointer to next message in, set by system. |  |

MP/M II Programmer's Guide	3.3		Queue Data Structures

Table 3-2. (continued)

| Element | Definition |
| --- | --- |
| MSG$OUT 2 bytes, pointer to next message out, set by system. MSG$CNT 2 bytes, number of messages in the queue, set by system. BUFFER n bytes, where n is equal to the message length times the number of messages. Space allocated by user, set by system. NOTE: Mutual exclusion queues require a 2-byte buffer for the owner Process Descriptor address. |  |

3.3.2		Linked Queues

The following example illustrates how to initialize a QCB for a

linked queue containing 4 messages, each 33 bytes in length.

PL/M:

DECLARE LINKED$QUEUE STRUCTURE (

QL ADDRESS,

NAME (8) BYTE,

MSGLEN ADDRESS,

NMBMSGS ADDRESS,

DQPH ADDRESS,

NQPH ADDRESS,

MH ADDRESS,

MT ADDRESS,

BH ADDRESS,

BUFFER (140) BYTE  )

INITIAL (O,’LNKQUE  ‘,33,4);

MP/M II Programmer's Guide	3.3		Queue Data Structures

Assembly Language:

LNKQUE:

DS		2	QL

DB		'LINKQUE	; NAME

DW		33	MSGLEN

DW		4	NMBMSGS

DS		2	DQPH

DS		2	NQPH

DS		2	MH

DS		2	MT

DS		2	BH

BUFFER:

DS		2	MSG		#1	LINK

DS		33	MSG		#1	DATA

DS		2	MSG		#2	LINK

DS		33	MSG		#2	DATA

DS		2	MSG		#3	LINK

DS		33	MSG		#3	DATA

DS		2	MSG		#4	LINK

DS		33	MSG		#4 	DATA

The elements of the linked queue shown above are defined in Table 3

3. The total queue overhead is 24 bytes.

Table 3-3. Linked Queue Elements

| Element | Definition |
| --- | --- |
| QL 2-byte link, set by system. NAME 8 ASCII character queue name, set by user. MSGLEN 2 bytes, length of message, set by user. NMBMSGS 2 bytes, number of messages, set by user. DQPH 2 bytes, Dequeue list process head, set by system. NQPH 2 bytes, Enqueue list process head, set by system. MH 2 bytes, message head, set by system. MT 2 bytes, message tail, set by system. BH 2 bytes, buffer head, set by system. BUFFER n bytes where n is equal to the message Length plus two, times the number of messages. Space allocated by the user, set by the system. |  |

MP/M II Programmer's Guide	3.3		Queue Data Structures

3.3.3		User Queue Control Block

The User Queue Control Block (UQCB) data structure provides

read/write access to queues in the same manner that an FCB provides

access to a disk file. Like files, queues are "opened" by an

operation that fills in the actual QCB address, which then can be

read from or written to.

If the actual queue address is known, it can be used in the

pointer field of the UQCB, in which case the 8-byte name field can

be omitted, and an open operation is not required to access the

queue. If the address is not known, then an open operation must be

performed (see Function 135).

The following example illustrates how to initialize a UQCB in

both PL/M and assembly language.

PL/M:

DECLARE USER$QUEUE$CONTROL$BLOCK STRUCTURE (

POINTER ADDRESS,

MSGADR ADDRESS,

NAME (8) BYTE         )

INITIAL (0,.BUFFER,’SPOOL   ‘);

DECLARE BUFFER (33) BYTE;

Assembly Language:

UQCB:

DS	2		;POINTER

DW	BUFFER;	SGADR

DB	'SPOOL   ‘	;NAME

BUFFER:

DS	33		;BUFFER

MP/M II Programmer's Guide	3.3		Queue Data Structures

The elements of the UQCB shown above are defined in Table 3-4.

Table 3-4. UQCB Elements

| Element | Definition |
| --- | --- |
| POINTER 2 bytes, set by system to address of actual queue during an open queue operation, or set by the user if the actual queue address is known. MSGADR 2 bytes, address of user buffer, set by user. NAME 8 bytes, ASCII queue name, set by user, may be omitted if the pointer field is set by the user. |  |

3.3.4		Queue Naming Conventions

The following conventions should be used in naming queues under

MP/M II: if the Terminal Message Processor (TMP) is to write

directly to the queue, then the queue must have an upper-case ASCII

name. Thus, when a user at a system console enters the queue name

followed by a command tail, the CLI writes the command tail directly

to the queue (see Section 1.5).

To make a queue inaccessible by a user at a system console, the

queue name must contain at least one lower-case ASCII character.

Mutual exclusion queues should be named upper-case 'MX' followed by

1 to 6 additional ASCII characters. These queues must have a two

byte buffer in which the XDOS places the address of the Process

Descriptor of the process owning the mutual exclusion message.

3.4		Memory Descriptor Data Structure

Each process running under MP/M II is associated with a Process

Descriptor that contains a memory segment index. This index

identifies a specific Memory Descriptor within MP/M II's Memory

Segment Table. In MP/M II the memory segment index can have the

values 0 to 7, corresponding the 8-entry Memory Segment

Table, or FFH, indicating that the process is in common memory and

does not use the Memory Segment Table. The XDOS uses the Memory

Descriptor data structure to allocate and manage the memory

resource. The Memory Descriptor contains four bytes: the memory

segment base page address, the memory segment page size, the memory

segment attributes, and bank. The Memory Descriptor data structure

is shown below in both PL/M and assembly language.

MP/M II Programmer's Guide	3.4		Memory Descriptor

PL/M:

Declare memory$descriptor structure (

base byte,

size byte,

attrib byte,

bank byte           );

Assembly Language:

MEMDES:

DS	1	;base

DS	1	;size

DS	1	;attributes

DS	1	;bank

The elements of the Memory Descriptor shown above are defined

in Table 3-5.

Table 3-5. Memory Descriptor Elements

| Element | Definition |
| --- | --- |
| BASE 1 byte, base page address of the memory segment, set by user. SIZE 1 byte, size in pages of the memory segment, set by user. ATTRIBUTE 1 byte, high-order bit "on" indicates that the memory segment is allocated, other bits are reserved for MP/M II, normally set by system, but a user may pr I e-allocate a memory segment by setting the high-order bit "on". BANK 1 byte, bank number in the range 0 to 7, where bank 0 is the bank which is switched in when MP/M II is loaded and initialized, set by user. |  |

3.5		System Data Page

The System Data Page is the top 256 bytes of the MP/M II

Operating System. It contains static information about the system

configuration which the user enters when executing GENSYS to perform

system generation. It also contains dynamic information which is

used by MP/M II at run time.

MP/M II Programmer's Guide	3.5		System Data Page

Table 3-6 describes the individual byte assignments within the

System Data Page.

Table .3-6. System Data Page Byte Assignments

| Byte | Contents |
| --- | --- |
| 000-000 Mem$top, top page of memory 001-001 Nmb$cns, number of system consoles (TMPs) 002-002 Brkpt$RST, breakpoint RST # 003-003 Add system call user stacks, boolean 004-004 Bank switched, boolean 005-005 Z80 version, boolean 006-006 banked bdos, boolean 007-007 XIOS jump table page 008-008 RESBDOS base page 009-010 CP/NET master configuration table address 011-011 XDOS base page 012-012 RSP's (BNKXIOS top+l) base page 013-013 BNKXIOS base page 014-014 BNKBDOS base page 015-015 Max$mem$seg, max memory segment number 016-047 Initial memory segment table 048-063 Breakpoint vector table, filled in by debuggers 064-079 Reserved for MP/M II 080-095 System call user stack pointer table 096-119 Reserved for MP/M II 120-121 Nmb records in MPM.SYS file 122-122 # ticks/sec 123-123 System Drive 124-124 Common Memory Base Page 125-125 Number of Rsp's 126-127 Listcp array Address 128-143 Subflg, submit flag array 144-186 Reserved for MP/M II 187-187 Max locked records/process 188-188 Max open files/process 189-190 # list items 191-192 Pointer to base of lock table free space 193-193 Total system locked records 194-194 Total system open files 195-195 Dayfile logging, boolean 196-196 Temporary file drive 197-197 Number of printers 197-241 Reserved for MP/M II 242-242 Banked XDOS base page 243-243 TMP process descriptor base 244-244 Console.dat base 245-246 BDOS/XDOS entry point 247-247 TMP.spr base 248-248 Nmbrsps, number of banked RSPs 249-249 Brsp base address 250-251 Brspl, non-resident rsp process link 252-253 Sysdatadr, XDOS internal data segment address 254-255 Rspl, resident system process link |  |

MP/M II Programmer's Guide	3.6		XDOS Internal Data Segment

3.6		XDOS Internal Data Segment

This section contains information regarding the location of

critical variables contained in the XDOS Internal Data Segment. The

information may be useful in some application programs. However, it

must be accessed with caution. The information may also be useful

in debugging a system by permitting access to the Ready List through

the Ready List Root (RLR), both at run time as well as in a post

mortem dump.

The following example, written in assembly language,

illustrates a technique for accessing the Ready List Root.

;	MP/M Internal Data Segment Offsets

0		equ	0000h ;	time of day

osrlr		equ	0005h ;	ready list root

osdlr		equ	0007h ;	delay list root

osdrl		equ	0009h ;	dispatcher ready list

osplr		equ	00OBh ;	poll list root

osslr		equ	00ODh ;	swap list root (not used)

osqlr		equ	OOOFh ;	queue list root

osthrdrt	equ	0011h ;	thread root

osnmbcns	equ	0013h ;	number of consoles

oscnsatt	equ	0014h ;	console attach table

oscnsque	equ	0034h ;	console queue

osnmbflags	equ	0054h ;	number of flags

ossysfla	equ	0055h ;	system flags

osnmbsegs	equ	0095h ;	number of memory segments

osmsegtbl	equ	0096h ;	memory segment table

ospdtbl	equ	0OB6h ;	process descriptor table

osnmblst	equ	0256h ;	number of list devices

oslstatt	equ	0257h ;	list attach table

oslstque	equ	0277h ;	list queue

sysdatadr	equ	154 	;	get system data page addr

. . .

mvi	c,sysdatadr

call	xdos		; HL = system data page

lxi	d,00fch 	; DE = offset to pointer

dad	d

mov	e,m

inx	h

mov	d,m		; DE = base of XDOS intrl dseg

lxi	h,osrlr 	; HL = offset to Ready List Root

dad	d

; HL = Addr of Ready List Root

MP/M II Programmer's Guide	3.7 XDOS Error Handling

3.7		XDOS Error Handling

The XDOS does not require an error handling capability similar

to that of the BDOS, because XDOS functions involve "logical" or

internal rather than "physical" or external operations. That is,

the XDOS functions are implemented entirely within memory resident

data structures, and any physical or extended "error" encountered

would by definition be catastrophic for the system. Therefore,

those XDOS functions that return a value in register A return a

"boolean", which is a code indicating only whether or not the

function is successful. If for some reason the function is not

successful, the calling process must be able to handle this error

condition. The return codes for XDOS functions are defined in Table

3-7.

Table 3-7. XDOS Return Codes

| Register A Value | Meaning |
| --- | --- |
| - Successful operation FFH Unsuccessful operation |  |

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 128

3.8		XDOS Function Calls

The Extended Disk Operating System (XDOS) functions are covered

in this section by describing the entry parameters and returned

values for each XDOS function. The XDOS calling conventions are

identical to those of the BDOS which are described in Section 2.1.3.

FUNCTION 128:ABSOLUTE MEMORY REQUEST

Entry Parameters: Register C: 80H DE: MD Address Returned Value: Register A: Return code MD filled in

The Absolute Memory Request function allocates to the calling

process a segment of memory specified by the Memory Descriptor

parameter. This function allows the Command Line Interpreter (CLI)

to load non-relocatable programs, such as CP/M *.COM files, based at

the absolute TPA address of 0100H. The calling process passes the

address of a Memory Descriptor in register pair DE, setting the base

byte; the XDOS sets the other bytes upon return. The Memory

Descriptor data structure is described in Section 3.4.

Function 128 returns a "boolean" indicating whether or not the

allocation was successful. A returned value of FFH indicates

failure to allocate the requested memory, and a value of 0 indicates

success. If the Absolute Memory Request is a success, the memory

segment index of the calling process is set to reflect that of the

allocated memory. Thus, it is extremely important that this

function only be invoked from a process residing in common memory.

Note that base and size specify base page address and page size

where a page is 256 bytes.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 129

FUNCTION 129: RELOCATABLE MEMORY REQUEST

Entry Parameters: Register C: 81H DE: MD Address Returned Value: Register A: Return code MD filled in

The Relocatable Memory Request function allocates the requested

contiguous memory to the calling process. The calling process

passes the address of a Memory Descriptor in register pair DE,

setting the size byte; the XDOS sets the other bytes upon return

Function 129 returns a "boolean" in register A indicating

whether or not the allocation was successful. A returned value of

FFH indicates failure to satisfy the request, and a value of 0

indicates success. If the Relocatable Memory Request is a success,

the memory segment index of the calling process is set to reflect

that of the allocated memory. Thus, it is extremely important that

this function only be invoked from a process residing in common

memory.

Note that base and size specify base page address and page size

where a page is 256 bytes.

FUNCTION 130: MEMORY FREE

Entry Parameters: Register C: 82H DE: MD Address

The Memory Free function returns the specified memory segment

owned by the calling process back to the operating system. The

calling process passes the address of a Memory Descriptor in

register pair DE. Function 130 does not return a value in register

A.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 131

FUNCTION 131: POLL

Entry Parameters: Register C: 83H E: Device Number

The Poll function polls the specified device until a ready

condition is received. The calling process relinquishes the CPU

until the poll is satisfied, allowing other processes to execute.

Function 131 is intended for use in the custom XIOS because the

XIOS associates the device number with the actual code executed for

the poll operation. This does not exclude other uses of the Poll

function, but it does mean that an application program making a poll

call must be matched to a specific XIOS.

FUNCTION 132: FLAG WAIT

Entry Parameters: Register C: 84H E: Flag Number Returned Value: Register A: Return code

The Flag Wait function causes a process to relinquish the CPU

until the flag specified in the call is set. The flag wait

operation is used in an interrupt-driven system to cause the calling

process to "wait" until a specific interrupt condition occurs.

Function 132 returns a "boolean" in register A indicating

whether or not a successful flag wait was performed. A returned

value of FFH indicates that no flag wait occurred because another

process was already waiting on the specified flag. A returned value

of 0 indicates success.

Note that flags are non-queued, which means that access to

flags must be carefully managed. Typically, the physical interrupt

handlers set flags while a single process waits on each flag.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 133

FUNCTION 133: FLAG SET

Entry Parameters: Register C: 85H E: Flag Number Returned Value: Register A: Return code

The Flag Set function "wakes up" a waiting process. The Flag

Set function is usually called by an interrupt service routine after

servicing an interrupt and determining which flag is to be set.

Function 133 returns a "boolean" in register A indicating

whether or not a successful flag set was performed. A returned

value of FFH indicates that a flag over-run has occurred; that is,

the flag was already set when a flag set function was called. A

returned value of 0 indicates success.

FUNCTION 134: MAKE QUEUE

Entry Parameters: Register C: 86H DE: QCB Address

The Make Queue function sets up a Queue Control Block. A queue

is configured as either circular or linked depending upon the

message size. Message sizes of 0 to 2 bytes use circular queues

while message sizes of 3 or more bytes use linked queues.

The calling process passes the address of the Queue Control

Block (QCB) in register pair DE. The QCB must contain the queue

name, message length, number of messages, sufficient space to

accommodate the messages, and links if the queue is linked.

The QCB data structures for both circular and linked queues are

described in Section 3.3.

Queues can only be created either in common memory or by user

programs in non-banked systems because queues are all maintained on

a linked list that must be accessible at all times. That is, a

queue cannot reside in a memory segment that is bank-switched.

However, a queue created in common memory can be accessed by all

system and user programs.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 135

FUNCTION 135: OPEN QUEUE

Entry Parameters: Register C: 87H DE: UQCB Address Returned Value: Register A: Return code

The Open Queue function places the actual QCB address into the

User Queue Control Block (UQCB) . Function 135 allows a user program

to access queues by specifying only the queue name. The process

obtains the actual address of the itself by calling Function 135,

and then reads from or writes to the queue using the XDOS queue read

and write functions.

Function 135 returns a "boolean" in register A indicating

whether or not the open queue operation was successful. A returned

value of OFFH indicates failure, while a 0 indicates success.

The user Queue Control Block data structure is described in

Section 3.3.

FUNCTION 136: DELETE QUEUE

Entry Parameters: Register C: 88H DE: QCB Address Returned Value: Register A: Return Code

The Delete Queue function removes the specified queue from the

queue list. The calling process passes the address of QCB for the

specified queue in register pair DE.

Function 136 returns a "boolean" in register A indicating

whether or not the queue was deleted. A returned value of OFFH

indicates failure, usually because some process is DQing from the

queue. A returned value of 0 indicates success.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 137

FUNCTION 137: READ QUEUE

Entry Parameters: Register C: 89H DE: UQCB Address Returned Value: Message read

The Read Queue function reads a message from the queue

specified by the UQCB. If no message is available at the queue, the

calling process relinquishes the CPU until another process writes a

message at the queue. The calling process passes the address of the

UQCB in register pair DE, and when a message becomes available at

the queue, Function 137 copies it into the buffer addressed by the

MSGADR field of the UQCB.

FUNCTION 138: CONDITIONAL READ QUEUE

Entry Parameters: Register C: 8AH DE: UQCB Address Returned Value: Register A: Return code Message read if available

The Conditional Read Queue function reads a message from a

queue specified by the UQCB only when the queue contains a message.

This function can be used to prevent the calling process from being

suspended from execution if no messages exist. The calling process

passes the address of the UQCB in register pair DE, and if a message

is available at the queue, Function 138 copies it into the buffer

addressed by the MSGADR field of the UQCB.

Function 138 returns a "boolean" in register A indicating

whether or not a message was available at the queue. A returned

value of OFFH indicates no message, while a zero indicates that a

message was available and was copied into the user buffer.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 139

FUNCTION 139: WRITE QUEUE

Entry Parameters: Register C: 8BH DE: UQCB Address Message to be sent

The Write Queue function writes a message to a queue specified

by the UQCB. If no buffers are available at the queue, the calling

process relinquishes the CPU until one becomes available. The

calling process passes the address of the UQCB in register pair DE,

and when a buffer is available at the queue, the function copies the

buffer addressed by the MSGADR field of the UQCB into the queue.

Function 139 does not return a value in register A.

FUNCTION 140: CONDITIONAL WRITE QUEUE

Entry Parameters: Register C: 8CH DE: UQCB Address Message to be sent Returned Value: Register A: Return code

The Conditional Write Queue function writes a message to queue

specified by the UQCB only when a buffer is available. This

function can prevent the calling process from being suspended from

execution if the queue buffers are full. The calling process passes

the address of the UQCB in register pair DE, and if a buffer is

available at the queue, the function copies the buffer addressed by

the MSGADR field of the UQCB into the actual queue.

Function 140 returns a "boolean" in register A indicating

whether or not a buffer was available at the queue. A returned

value of OFFH indicates no buffer, while a zero indicates that a

buffer was available and that the user buffer was copied into it.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 141

FUNCTION 141: DELAY

Entry Parameters: Register C: 8DH DE: Number of Ticks

The Delay function delays execution of the calling process for

the specified number of system time units, thus allowing other

processes to use the CPU while the specified period of time elapses.

Use of Function 141 avoids the typical programmed delay loop, which

should be avoided under MP/M II because it consumes the CPU.

The system time unit is typically 60 Hz (16.67 milliseconds),

but can vary according to application. For example, it is likely

that in Europe it would be 50 Hz (20 milliseconds).

The calling process passes a 16-bit integer in register pair DE

which specifies the number of ticks the process is to be delayed.

Since calling the delay procedure is usually asynchronous to the

actual time base itself, there is up to one tick of uncertainty in

the exact amount of time delayed. Thus, a delay of 10 ticks

guarantees a delay of at least 10 ticks, but it may be nearly 11

ticks.

FUNCTION 142: DISPATCH

Entry Parameters: Register C: 8EH

The Dispatch function causes MP/M II to determine the highest

priority ready process, and then give that process the CPU.

Function 142 is intended for non-interrupt driven systems in which

it is desirable to enable a compute-bound process to periodically

relinquish the CPU. Since all user processes usually run at the

same priority, invoking Dispatch at various points in a program

allows other processes access to the CPU in a round-robin fashion.

Dispatch can also safely enable interrupts following the execution

of a disable interrupt instruction (DI).

There are no parameters passed in register pair DE, and no

values returned in register A. The process calls Function 142 by

passing the function number 8EH in register C. Note: Calling

Dispatch does not remove the calling process from the Ready List.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 143

FUNCTION 143: TERMINATE PROCESS

Entry Parameters: Register C: 8FH D: Conditional Memory Free E: Terminate Code

The Terminate Process function terminates the calling process,

which passes parameters in registers D and E, indicating whether or

not the process should be terminated if it is a system process, and

if the memory segment owned by the calling process is to be

released. A OFFH in the E register indicates that the process

should be unconditionally terminated; a zero indicates that only a

user process is to be deleted. If the calling process is a user

process and register D contains a OFFH, the memory segment owned by

the process is not released. Thus, a process that is a child of a

parent process, both of which are executing in the same memory

segment, can terminate without freeing the memory segment that is

also occupied by the parent.

Function 143 does not return any value in register A. The

calling process simply ceases to exist as far as MP/M II is

concerned.

FUNCTION 144: CREATE PROCESS

Entry Parameters: Register C: 90H DE: PD Address Returned Value: PD filled in

The Create Process function creates one or more processes by

placing the passed Process Descriptors on the MP/M II Ready List.

The calling process passes the address of a Process Descriptor

in register pair DE. The first field of the Process Descriptor is a

link field that can point to another Process Descriptor.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 144

Processes can only be created either in common memory or by

user programs in non-banked systems because Process Descriptors are

all maintained on linked lists that must be accessible at all times.

The Process Descriptor data structure is described in Section

3.2.

FUNCTION 145: SET PRIORITY

Entry Parameters: Register C: 91H E: Priority

The Set Priority function sets or changes the priority of the

calling process to that of the passed parameter. The calling

process passes the priority in register E. Function 145 does not

return a value in register A.

This function is useful when a process needs to have a high

priority during an initialization phase, but after that is to run at

a lower priority.

FUNCTION 146: ATTACH CONSOLE

Entry Parameters: Register C: 92H

The Attach Console function attaches the console specified in

the CONSOLE field of the Process Descriptor to the calling process.

If the console is already attached to some other process, the

calling process relinquishes the CPU until the other process

detaches from the console. When the console becomes free and the

calling process is the highest priority process waiting for the

console, the attach operation takes place.

There are no parameters passed in registers D and E, and no

values returned in register A. The process calls Function 146 by

passing the function number 92H in register C.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 147

FUNCTION 147: DETACH CONSOLE

Entry Parameters: Register C: 93H

The Detach Console function detaches from the calling process

the console specified in the CONSOLE field of the Process

Descriptor. If the console is not currently attached, no action

takes place.

There are no parameters passed in registers D and E, and no

values returned in register A. The process calls Function 147 by

passing the function number 93H in register C.

FUNCTION 148: SET CONSOLE

Entry Parameters: Register C: 94H E: Console

The Set Console function detaches the currently attached

console and then attaches the specified console. If the console to

be attached is already attached to another process, the calling

process relinquishes the CPU until the other process detaches from

the console. When the console becomes available and the calling

process is the highest priority process waiting for the console, the

attach operation takes place.

The calling process passes the number of the console to be

attached in register E. The function does not return a value in

register A.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 149

FUNCTION 149: ASSIGN CONSOLE

Entry Parameters: Register C: 95H DE: APB Address Returned Value: Register A: Return code

The Assign Console function unconditionally assigns a console

to a specified process. That is, the assignment is made regardless

of whether or not any other process is currently waiting to attach

the console. The calling process passes the address of a data

structure called the Assignment Parameter Block (APB). This data

structure contains the console number for the assignment, an 8

character ASCII process name, and a "boolean" indicating whether or

not a match with the CONSOLE field of the Process Descriptor is

required (true or OFFH indicates it is required).

It is extremely important to note that the calling process must

own the console or the console must be currently unattached for this

function to perform properly.

Function 149 returns a "boolean" in register A indicating

whether or not the assignment was made. A returned value of OFFH

indicates failure to assign the console, either because a Process

Descriptor with the specified name could not be found, or because a

match was required, and the CONSOLE field of the Process Descriptor

did not match the specified console. A returned value of zero

indicates a successful assignment.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 150

FUNCTION 150: SEND CLI COMMAND

Entry Parameters: Register C: 96H DE: CLICMD Address

The Send CLI Command function permits running processes to send

command lines to the Command Line Interpreter (see Section 1.5).

The calling process passes the address of a data structure called

CLI Command (CLICMD) in register pair DE. This data structure

contains: the default disk/user code, the console and the command

line. Initialization of the CLICMD data structure is shown below in

both PL/M and assembly language.

PL/M:

Declare CLI$command structure  (

disk$user byte,

console byte,

command$line (129) byte);

Assembly Language:

CLICMD:

DS	1	;default disk / user code

DS	1	;console number

DS	129	;command line

The default disk/user code is the first byte of the data structure.

The high-order four bits contain the default disk drive and the low

order four bits contain the user code. The second byte of the data

structure contains the console number for the process being

executed. The ASCII command line begins with the third byte and is

terminated with a null byte.

It is extremely important to note that the CLI must own the

console specified in the parameter of the Send CLI Command function.

This assignment of the console to the CLI can be done with the

Function 149, Assign Console.

Function 150 does not return a value in register A.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 151

FUNCTION 151: CALL RESIDENT SYSTEM PROCEDURE

Entry Parameters: Register C: 97H DE: CPB Address Returned Value: Registers HL: Return code

The Call Resident System Procedure function permits a process

to call the optional Resident System Procedures (RSPs) . The calling

process passes the address of a data structure called the Call

Parameter Block, (CPB) in register pair DE. The CPB data contains

the address of an 8-character ASCII RSP name followed by a two-byte

parameter that the calling process passes to the RSP.

Initialization of the CPB data structure is shown below in both PL/M

and assembly language.

PL/M:

Declare CALL$PB structure  (

Name$adr address,

Param address ) initial (

.name,O);

Declare name (8) byte initial (

'Procl   ‘);

Assembly Language:

CALLPB:

DW 		NAME

DW		0	;parameter

NAME:

DB 		'Procl   ‘

Function 151 returns a 0001H in register pair HL if the RSP

called is not present. Otherwise, it returns the code passed back

from the RSP. Typically, a returned value of FFH indicates failure

while a zero indicates success.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 152

FUNCTION 152: PARSE FILENAME

Entry Parameters: Register C: 98H DE: PFCB Address Returned Value: Registers HL: Return code Parsed file control block

The Parse Filename function parses an ASCII file specification

(FILENAME) and prepares a File Control Block (FCB). The calling

process passes the address of a data structure called the Parse

Filename Control Block, (PFCB) in register pair DE. The PFCB

contains the address of the ASCII filename string followed by the

address of the target FCB. Initialization of the PFCB data

structure is shown below in both PL/M and assembly language.

PL/M:

Declare Parse$FN$CB structurem (

File$name$adr address,

FCB$adr address ) initial (

.file$name,.fcb );

Declare file$name (128) byte;

Declare fcb (36) byte;

Assembly Language:

PFNCB:

DW	FLNAME

DW	FCB

FLNAME:

DS	128

FCB:

DS	36

Function 152 assumes the file specification to be in the following

form:

{D:}{FILENAMEH}{.TYP}{;PASSWORD}

where those items enclosed in curly brackets are optional.

The Parse Filename function parses the first file specification

it finds in the input string. The function first eliminates leading

blanks and tabs. The function then assumes that the file

specification ends on the first delimiter it hits that is out of

context with the specific field it is parsing. For instance, if it

MP/M II Programmer's Guide        3.8 XDOS Calls:  Function 152

finds a colon (:) and it is not the second character of the file

specification, the colon delimits the whole file specification. The

function recognizes the following characters as delimiters:

space

tab

return

null

; (semicolon) - except before password field

= (equal)

< (less than)

> (greater than)

. (dot) - except after filename and before type

: (colon) - except before filename and after drive

, (comma)

[ (left square bracket)

] (right square bracket)

/ (slant)

$ (dollar)

If the function reaches a non-graphic character (in the range 1

through 31), not listed above, it treats it as an error.

The Parse Filename function initializes the specified FCB as

follows:

byte 0	The drive field is set to the specified

drive. If the drive is not specified, the

default value is used. O=default, I=A,

2=B, etc.

byte 1-8	The name is set to the specified filename.

All letters are converted to upper-case.

If the name is not eight characters long,

the remaining bytes in the filename field

are padded with blanks. If the filename

has an asterisk (*), all remaining bytes

in the filename field are filled in with

question marks (?). An error occurs if

the filename is more than eight bytes

long.

byte 9-11	The type is set to the specified filetype.

If no type is specified, the type field is

initialized to blanks. All letters are

converted to upper-case. If the type is

not three characters long, the remaining

bytes in the file type field are padded

with blanks. If an asterisk (*) occurs,

all remaining bytes are filled in with

question marks (?). An error occurs if

the type field is more than 3 bytes long.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 152

byte 12-15	Filled in with zeroes.

byte 16-23	The password field is set to the specified

password. If no password is specified, it

is initialized to blanks. If the password

is not eight characters long, remaining

bytes are padded with blanks. All letters

are converted to upper-case. If the

password field is more than eight bytes

long, an error occurs.

byte 24-25	The offset of the beginning of the

password in the FILENAME string is placed

here. If no password is specified, this

field is set to zero. It should be noted

that the password indicated by this field

is in the FILENAME string, which is not

modified by the Parse Filename function.

If there are any lower-case characters in

the password, they will have to be

converted to upper-case to make it the

same as the password field of the FCB.

byte 26	The number of characters in the specified

password is placed here. If no password

is specified, this field is set to zero.

If an error occurs, all fields that have not been parsed are

set to their default values, and the function returns a OFFFFh in

register pair HL indicating the error.

On a successful parse, the Parse Filename function checks the

next item in the FILENAME string. It skips over trailing blanks and

tabs and looks at the next character. If the character is a null or

carriage return, it returns a 0 indicating the end of the FILENAME

string. If the next character is a delimiter, it returns the

address of the delimiter. If the next character is not a delimiter,

it returns the address of the delimiting blank or tab.

If the first non-blank or non-tab character in the FILENAME

string is a null (0) or carriage return, the Parse Filename

function returns a zero indicating the end of string, and the FCB is

initialized to its default values.

If the Parse Filename function is to be used to parse a

subsequent filename in the FILENAME string, the returned address

should be advanced over the delimiter before placing it in the PFCB.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 153

FUNCTION 153: GET CONSOLE NUMBER

Entry Parameters: Register C: 99H Returned Value: Register A: Console Number

The Get Console Number function obtains the value of the

CONSOLE field from the Process Descriptor of the calling process.

The calling process passes the function number 99H in register C,

and the function returns the console number in register A.

FUNCTION 154: SYSTEM DATA ADDRESS

Entry Parameters: Register C: 9AH Returned Value: Registers HL: System Data Page Address

The System Data Address function returns the base address of

the system data page. The system data page resides in the top 256

bytes of the MP/M II Operating System. It contains configuration

information entered by the MP/M II GENSYS program as well as run

time data including the submit flags. The contents of the system

data page are described in Section 3.5.

The calling process passes the function number 9AH in register

C, and the function returns the base address of the system data page

in register pair HL.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 155

FUNCTION 155: GET DATE AND TIME

Entry Parameters: Register C: 9BH DE: TOD Address Returned Value: Time and date

The Get Date and Time function returns the current encoded date

and time. The calling process passes the address of a data

structure called the TOD in register pair DE. The TOD data

structure represents the date as a 16-bit integer, with day 1

corresponding to January 1, 1978. It represents the time as three

bytes: hours, minutes, and seconds, stored as two BCD digits.

Initialization of the TOD data structure is shown below in both

PL/M and assembly language.

PL/M:

Declare TOD structure (

date address,

hour byte,

min byte,

sec byte );

Assembly Language:

TOD:

DS	2	;Date

DS	1	;Hour

DS	1	;Minute

DS	1	;Second

MP/M II Programmer's Guide        XDOS Calls: Function 156

FUNCTION 156: RETURN PROCESS DESCRIPTOR ADDRESS

Entry Parameters: Register C: 9CH Returned Value: Register HL: PD Address

The Return Process Descriptor Address function obtains the

address of the calling processes process descriptor. By

definition, this is the head of the ready list.

FUNCTION 157: ABORT SPECIFIED PROCESS

Entry Parameters: Register C: 9DH Register DE: APB Address Returned Value: Register A: Return Code

The Abort Specified Process function permits a process to

terminate another specified process. The calling process passes the

address of a data structure called an Abort Parameter Block (ABTPB)

in register pair DE. Initialization of the ABTPB is shown below in

both PL/M and assembly language.

PL/M:

Declare Abort$paramter$block structure (

pdadr address,

termination$code address,

name (8) byte,

console byte );

Assembly Language:

APB:

DS		2	;process descriptor address

DS		2	;termination code

DS		8	;process name

DS		1	;console used by process

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 157

If the Process Descriptor address is known, it can be filled in

and the process name and console can be omitted. Otherwise, the

Process Descriptor address field should be a 0, and the process name

and console must be specified. In either case, the termination

code, which is the parameter passed to Function 143, Terminate

Process, must be supplied.

FUNCTION 158: ATTACH LIST

Entry Parameters: Register C: 9EH

The Attach List function attaches the list device specified in

the CONSOLE/LIST field of the Process Descriptor to the calling

process. If the list device is already attached to some other

process, the calling process relinquishes the CPU until the other

process detaches from the list device. When the list device becomes

free and the calling process is the highest priority process waiting

for the list device, the attach operation takes place.

The process calls Function 158 by passing the function number

9EH in register C. The function does not return a value in register

A.

FUNCTION 159: DETACH LIST

Entry Parameters: Register C: 9FH

The Detach List function detaches the list device specified in

the CONSOLE/LIST field of the Process Descriptor from the calling

process. If the list device is not currently attached, no action

takes place.

The process calls Function 159 by passing the function number

9FH in register C. The function does not return a value in register

A.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 160

FUNCTION 160: SET LIST

Entry Parameters: Register C: AOH E: List Device

The Set List function detaches the list device currently

attached to the calling process and then attaches the specified list

device. If the list device to be attached is already attached to

another process, the calling process relinquishes the CPU until the

other process detaches from the list device. When the list device

becomes free and the calling process is the highest priority process

waiting for the device, the attach operation takes place.

The calling process passes the number of the list device to be

attached in register E. The function does not return a value in

register A.

FUNCTION 161: CONDITIONAL ATTACH LIST

Entry Parameters: Register C: AlH Returned Value: Register A: Return Code

The Conditional Attach List function attaches the list device

specified in the CONSOLE/LIST field of the Process Descriptor to the

calling process only if the list device is currently unattached.

If the list device is currently attached to another process,

the function returns a value of OFFH in register A, indicating that

the list device could not be attached. The function returns a value

of zero to indicate that either the list device is already attached

to the process, or that it was unattached and a successful attach

operation was made.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 162

FUNCTION 162: CONDITIONAL ATTACH CONSOLE

Entry Parameters: Register C: A2H Returned Value: Register A: Return Code

The Conditional Attach Console function attaches the console

specified in the CONSOLE/LIST field of the Process Descriptor to the

calling process only if the console is currently unattached.

If the console is currently attached to another process, the

function returns a value of OFFH in register A, indicating that the

console could not be attached. The function returns a value of zero

to indicate that either the console is already attached to the

process or that it was unattached and a successful attach operation

was made.

MP/M II Programmer's Guide	3.8		XDOS Calls:	Function 163

FUNCTION 163: RETURN MP/M VERSION NUMBER

Entry Parameters: Register C: A3H Returned Value: Register HL: Version Number

The Return MP/M Version Number function provides information

that allows version independent programming. The function returns a

two-byte value, with H = 01 for MP/M II and L the MP/M II revision

level.

FUNCTION 164: GET LIST NUMBER

Entry Parameters: Register C: A4H Returned Value: Register A: List Number

The Get List Number function returns the value of the list

device from the Process Descriptor of the calling process. The

process calls Function 164 by passing the function number A4H in

register C. The function returns the list device number in register

A.

Section 4

ASM, An 8080 Assembler

4.1		Overview

ASM reads an assembly language source file from the disk and

produces 8080 machine language in Intel hex format. Invoke ASM by

entering an ASM command in either of the following forms:

ASM filename

ASM filename.parms

In both cases, the assembler assumes there is a file on the disk

with the name:

filename.ASM

that contains an 8080 assembly language source file. The first and

second forms shown above differ only in that the second form passes

parameters to the assembler to control source file access and hex

and print file destinations.

In either case, MP/M II loads ASM, which prints the message:

MP/M ASSEMBLER VER 2.0

where n.n is the current version number. In response to the

command, the assembler reads the source file with assumed filetype

"ASM" and creates two output files:

filename.HEX

filename.PRN

The HEX file contains the machine code corresponding to the original

program in Intel hex format, and the PRN file contains an annotated

listing showing generated machine code, error flags, and source

lines. If errors occur during translation, they are listed in the

PRN file as well as at the console.

The second command form can redirect input and output files

from their defaults. The "parms" portion of the command is a three

letter group that specifies the origin of the source file, the

destination of the hex file, and the destination of the print file.

The form is:

filename.plp2p3

where pl, p2, and p3 are replaced by single letters whose meanings

are defined in Table 4-1.

MP/M II Programmer's Guide	4.1 		overview

Table 4-1. ASM Parameters

| Symbol | Valid Letters | Meaning |
| --- | --- | --- |
| P1 A,B,. . ,P designates the drive that contains the source file p2 A,B,. . ,P designates the drive that receives the hex file. z skips the generation of the hex file p3 A,B,. . ,P designates the drive that receives the print file X places the listing at the console z skips generation of the print file |  |  |

Thus, the command:

ASM PROG.AAA

indicates that the assembler takes the source file (PROG.ASM) from

drive A, and also creates the hex (PROG.HEX) and print (PROG.PRN)

files on drive A. This command is the default if the assembler is

run from drive A without the optional parameters, as shown below:

OA>ASM PROG

The command:

OA>ASM PROG.ABX

indicates that the assembler takes the source file from drive A,

place the hex file on drive B and sends the listing file to the

console. The command:

OA>ASM PROG.BZZ

takes the source file from drive B and skips the generation of the

hex and print files. Use this command for a fast execution of the

assembler to check program syntax.

The source program format is compatible with the Intel 8080

assembler, although macros are not supported. However, certain

extensions in the MP/M II assembler make it easier to use. These

extensions are described below.

MP/M II Programmer's Guide	4.2		Program Format

4.2		Program Format

An assembly language program acceptable as input to the ASM

assembler consists of a sequence of statements of the form:

line#		label	operation	operand 	;comment

where any or all the fields can be present in a particular instance.

Each assembly language statement must be terminated with a carriage

return and line feed (ED automatically inserts a line feed) or with

an exclamation mark, !, which is treated as an end-of-line by the

assembler. Thus, multiple assembly language statements can be

written on the same physical line if separated by exclamation marks.

The line# is an optional decimal integer value representing the

source program line number, which is allowed on any source line.

Because these line numbers are inserted automatically by line

oriented editors, ASM ignores this field if present. The label

field takes either of the forms below:

identifier

identifier:

Labels are optional, except where noted in particular statement

types. An identifier is a sequence of any alphanumeric characters,

but the first character must be alphabetic. You can use identifiers

freely to label elements such as program steps and assembler

directives. only the first 16 characters are significant in an

identifier, except for an embedded dollar symbol, $, which can be

used to improve readability of the name. All lower-case alphabetics

are treated as if they were upper-case. optionally, a colon can

follow the identifier. Thus, the following are all valid labels:

X		xy	long$name

X:		yxl:	longer$named$data:

XlY2		Xlx2 	x234$5678$9012$3456:

The operation field contains either an assembler directive, a

pseudo operation, or an 8080 machine operation code. The pseudo

operations and machine operation codes are described in Section 4.5.

Section 4.4 describes the assembler directives.

The operand field of the statement generally contains an

expression formed out of constants and labels, along with arithmetic

and logical operations on these elements. The complete details of

properly formed expressions are given in Section 4.3.

The comment field can contain any characters following the

semicolon, ;, until the next real or logical end-of-line. These

characters are read and listed, but are otherwise ignored by the

assembler. The MP/M assembler also treats statements that begin

with an asterisk, *, in column one as comment statements. These are

listed and ignored in the assembly process.

MP/M II Programmer's Guide	4.2		Program Format

The assembly language source program is a sequence of

statements as defined above, optionally terminated by an END

statement. ASM ignores statements following END.

4.3		Forming the Operand

To completely describe the operation codes and pseudo

operations, it is necessary to first present the form of the operand

field, because it appears in nearly all statements. Expressions in

the operand field consist of simple operands (labels, constants, and

reserved words), combined in properly formed subexpressions by

arithmetic and logical operators. ASM evaluates each expression as

the assembly proceeds. Each expression must evaluate to a 16-bit

value. Further, the number of significant digits in the result must

not exceed the intended use. That is, if an expression is to be

used in a byte move immediate instruction, then the most significant

8 bits of the expression must be zero. The restrictions on the

expression significance are given with the individual instructions.

4.3.1		Labels

As discussed above, a label is an identifier that appears as

part of a particular statement. In general, the label is given a

value determined by the type of statement that it precedes. If the

label appears in a statement that generates machine code or reserves

memory space (for example, a MOV instruction or a DS pseudo

operation) , then the label is given the value of the program address

that it labels. If the label precedes an EQU or SET, then ASM gives

the label the value that results from evaluating the operand field.

Except for the SET statement, an identifier can label only one

statement.

When a label appears in the operand field, ASM substitutes its

value during assembly. This value can then be combined with other

operands and operators to form the operand field for a particular

instruction.

4.3.2		Numeric Constants

A numeric constant is a 16-bit value in one of several bases.

The base, called the radix of the constant, is denoted by a trailing

radix indicator. The radix indicators recognized by ASM are defined

in Table 4-2, below.

MP/M II Programmer's Guide	4.3		Forming the Operand

Table 4-2. ASM Radix Indicators

| Indicator | Base |
| --- | --- |
| B binary constant (base 2) 0 octal constant (base 8) Q octal constant (base 8) D decimal constant (base 10) H hexadecimal constant (base 16) |  |

Q is accepted as an alternate radix indicator for octal numbers to

minimize confusion between the letter 0 and the digit 0. Any

numeric constant that does not terminate with a radix indicator is

assumed to be a decimal constant.

A constant is thus composed as a sequence of digits, followed

by an optional radix indicator, where the digits are in the

appropriate range for the radix. That is, binary constants must be

composed of the digits 0 and 1 octal constants can contain digits in

the range 0-7, while decimal constants contain decimal digits.

Hexadecimal constants contain decimal digits as well as hexadecimal

digits A (10D), B (11D), C (12D), D(13D), E (14D), and F(15D).

Note that the leading digit of a hexadecimal constant must be a

decimal digit so that ASM cannot confuse a hexadecimal constant with

an identifier (a leading 0 will always suffice) . A constant

composed in this manner must evaluate to a binary number that can be

contained within a 16-bit counter; otherwise, it is truncated to

the least significant 16-bits. Similar to identifiers, imbedded $'s

are allowed within constants to improve their readability. Finally,

ASM translates the radix indicator to upper-case if a lower-case

letter is encountered. The following are all valid numeric

constants:

1234		1234D		1100B		1111$0000$1111$OOOOB

1234H		OFFEH		33770 	33$77$22Q

3377o		Ofe3h		1234d		Offffh

4.3.3		Reserved Words

Several reserved character sequences have predefined meanings

in the operand field of a statement. The names of 8080 registers,

when encountered by the assembler, are translated to the values

shown in Table 4-3.

MP/M II Programmer's Guide	4.3		Forming the operand

Table 4-3. 8080 Registers

| Register Letter | Value |
| --- | --- |
| A 7 B 0 C 1 D 2 E 3 H 4 L 5 M 6 SP 6 PSW 6 |  |

Again, lower-case names have the same values as their upper-case

equivalents. Machine instructions can also appear in the operand

field; if so, they evaluate to their internal codes. For

instructions that require operands in which the specific operand

becomes a part of the binary bit pattern of the instruction (for

example, MOV A,B), the value of the instruction (in this case MOV)

is the bit pattern of the instruction with zeroes in the optional

fields (e.g., MOV produces 40H).

When the $ symbol occurs in the operand field but not imbedded

within an identifier or numeric constant, its value becomes the

address of the next instruction to generate, not including the

instruction contained within the current logical line.

4.3.4		String Constants

String constants represent sequences of ASCII characters, and

are represented by enclosing the characters within apostrophe

symbols, '. All strings must be fully contained within the current

physical line, thus allowing ! symbols within strings, and must not

exceed 64 characters in length. The apostrophe character can be

included within a string by entering it as a double apostrophe, '',

which becomes a single apostrophe when read by the assembler.

Except for the DB pseudo operation, the string length is restricted

to either one or two characters, which become an 8-bit or 16-bit

value, respectively. Two-character strings become a 16-bit

constant, with the second character as the low-order byte, and the

first character as the high-order byte.

MP/M II Programmer4s Guide	4.3		Forming the Operand

The value of a character is its corresponding ASCII code (see

Appendix I). There is no case translation within strings, and so

both upper- and lower-case characters can be represented. Note,

however, that only graphic (printing) ASCII characters are allowed

within strings. Some examples of valid strings are

'A'		‘AB’		'ab’		‘c’

‘’’’		‘a’’’		‘’’’’’ 	’’’”’

'Walla Walla Wash.'

'She said ''Hello'' to me.'

'I said ''Hello'' to her.'

4.3.5 Arithmetic and Logical Operators

The operands described above can be combined in normal

algebraic notation using any combination of properly formed

operands, operators, and parenthesized expressions. The operators

recognized in the operand field are summarized in Table 4-4.

Table 4-4. Arithmetic and Logical Operators

| Operation | Result |
| --- | --- |
| A + b unsigned arithmetic sum of a and b A - b unsigned arithmetic difference between a and b + b unary plus (produces b) - b unary minus (identical to 0 - b) a * b unsigned multiplication of a and b a / b unsigned division of a by b a MOD b remainder after a / b a NOT b logical inverse of b: all O's become l's, l's become O's - b is a 16-bit value a AND b bit-by-bit logical and of a and b a OR b bit-by-bit logical or of a and b a XOR b bit-by-bit logical exclusive or of a and b a SHL b the value that results from shifting a to the left by an amount b, with zero fill a SHR b the value that results from shifting a to the right by an amount b, with zero fill |  |

MP/M II Programmer's Guide	4.3		Forming the Operand

In Table 4-4, a and b represent simple operands such as labels,

numeric constants, reserved words, one- or-two character strings, or

fully enclosed parenthesized subexpressions such as the examples

below.

10+20		10h+37Q	Ll /3	(L2+4)  SHR 3

('a' and 5fh) + ‘0’	('B'+B) OR (PSW+M)

(1+(2+c)) shr (A-(B+1))

Note that all computations performed at assembly time are 16

bit unsigned operations. Thus, -1 is computed as 0-1, which results

in the value Offffh (i.e. , all 1 's) . The resulting expression must

fit the operation code in which it is used. If, for example, the

expression is used in a ADI (add immediate) instruction, then the

high-order eight bits of the expression must be zero. For example,

the operation "ADI -l" produces an error message because -1 becomes

Offffh, which cannot be represented as an 8-bit value. "ADI (-l)

AND OFFH" is acceptable because the "AND" operation zeroes the high

order bits of the expression.

4.3.6		Precedence of Operators

As a convenience to the programmer, ASM assumes that operators

have a relative precedence of application. This allows you to write

expressions without nested levels of parentheses. Expressions have

assumed parentheses defined by relative precedence. The order of

application of operators in unparenthesized expressions is listed

below. operators listed first have highest precedence; they are

applied first in an unparenthesized expression. operators listed

last have lowest precedence. Operators listed on the same line have

equal precedence, and are applied from left to right as they are

encountered in an expression.

1) / MOD SHL SHR

2) - +

3) NOT

4) AND

5) OR XOR

Due to this hierarchy, the expressions shown to the left below are

interpreted by the assembler as the fully parenthesized expressions

shown to the right:

a * b + c				(a * b) + c

a + b * c				a + (b * c)

a MOD b * c SHL d			((a MOD b) * c) SHL d

a OR b AND NOT c + d SHL e	a OR (b AND (NOT (c + (d SHL e))))

MP/M II Programmer's Guide	4.3		Forming the Operand

Balanced parenthesized subexpressions can always override the

assumed parentheses, and so the last expression above could be

rewritten to force application of operators in a different order,

such as:

(a OR b) AND (NOT c) + d SHL e

This	expression has the assumed parentheses:

(a OR b) AND HNOT c) + (d SHL e) )

Note that an unparenthesized expression is well-formed only if the

expression which results from inserting the assumed parentheses is

well-formed.

4.4		Assembler Directives

Assembler directives set labels to specific values during the

assembly, perform conditional assembly, define storage areas, and

specify starting addresses in the program. Each assembler directive

is denoted by a "pseudo operation" that appears in the operation

field of the line. The acceptable pseudo operations are summarized

in Table 4-5, and described individually in the following sections.

Table 4-5. ASM Directives

| Symbol | Function |
| --- | --- |
| ORG set the program or data origin END end program, optional start address EQU numeric "equate" SET numeric "set" IF begin conditional assembly ENDIF end of conditional assembly DB define data bytes DW define data words DS define data storage area |  |

MP/M II Programmer's Guide	4.4		Assembler Directives

4.4.1		The ORG Directive

The ORG statement takes the form:

label	ORG 	expression

where "label" is an optional program label, and "expression" is a

16-bit expression, consisting of operands that are defined ahead of

the ORG statement. The assembler begins machine code generation at

the location specified in the expression. There can be any number

of ORG statements within a particular program; however, there are

no checks to ensure that the programmer is not defining overlapping

memory areas. Note that most programs written for the MP/M II

system begin with the following ORG statement:

ORG 100H

This starts machine code generation at the base of an MP/M II

transient program area. To prepare a page-relocatable program for

execution under MP/M II, assemble the source program twice, adding

100H to each ORG statement during the second assembly. Concatenate

the two hex files generated by the assemblies using PIP, then submit

the concatenated file to the GENMOD utility which produces a file of

type PRL.

If a label is specified in the ORG statement, then the label is

given the value of the expression. This label can then be used in

the operand field of other statements to represent this expression.

4.4.2		The END Directive

The END statement is optional in an assembly language program,

but if it is present it should be the last statement because all

subsequent statements are ignored. The two forms of the END

directive are:

label		END

label		END 	expression

where the label is optional. If the first form is used, the

assembly process stops and the default starting address of the

program is taken as 0000. Otherwise, the expression is evaluated

and becomes the program starting address. This starting address is

included in the last record of the Intel-formatted machine code

"hex" file which results from the assembly. Thus, most CP/M

assembly language programs end with the statement:

END		100H

which results in the default starting address of 100H.

MP/M II Programmer's Guide	4.4		Assembler Directives

4.4.3		The EQU Directive

The EQU (equate) statement sets up synonyms for particular

numeric values. The form is:

label		EQU 	expression

where the label must be present, and must not label any other

statement. The assembler evaluates the expression and assigns this

value to the identifier given in the label field. The identifier is

usually a name that describes the value in a more human-oriented

manner. Then this name can be used throughout the program to

11parameterize" certain functions. Suppose for example, that data

received from a teletype appears on a particular input port, and

data is sent to the teletype through the next output port in

sequence. The series of equate statements could define these ports

for a particular hardware environment, as shown below:

TTYBASE		EQU	10H		;BASE PORT NUMBER FOR TTY

TTYIN			EQU	TTYBASE	;TTY DATA IN

TTYOUT		EQU	TTYBASE+1	;TTY DATA OUT

At a later point in the program, the statements that access the

teletype could appear as shown below:

IN		TTYIN		;READ TTY DATA TO REG-A

. . .

OUT		TTYOUT	;WRITE DATA TO TTY FROM REG-A

This makes the program more readable than if the absolute I/O ports

had been used. Further, if you redefine the hardware environment to

start the teletype communications ports at 7FH instead of 10H, you

need only change the first statement to:

TTYBASE		EQU	7FH	;BASE PORT NUMBER FOR TTY

and the program can be reassembled without changing any other

statements.

4.4.4		The SET Directive

The SET statement is		similar to the EQU, taking the form:

label		SET 	expression

It differs from SET in that the label can occur on other SET

statements within the program. The expression is evaluated and

becomes the current value associated with the label. Thus, the EQU

statement defines a label with a single value, while the SET

statement defines a value that is valid from the current SET

statement to the next SET statement where the label occurs. SET is

most often used to control conditional assembly.

MP/M II Programmer's Guide	4.4		Assembler Directives

4.4.5		The IF and ENDIF Directives

The IF and ENDIF statements define a range of assembly language

statements to be included or excluded during the assembly process.

The form is:

IF	expression

statement#1

statement#2

. . .

statement#n

ENDIF

Upon encountering the IF statement, the assembler evaluates the

expression following the IF. All operands in the expression must be

defined ahead of the IF statement. If the least significant bit of

the evaluated expression is a 1, then statement#1 through

statement#n are assembled; if the least significant bit of the

evaluated expression is zero, then the statements are listed but not

assembled. Conditional assembly is often used to write a single

11generic" program which includes a number of possible run-time

environments, with only a few specific portions of the program

selected for any particular assembly. The following program

segments for example, might be part of a program that communicates

with either a teletype or a CRT console (but not both) by selecting

a particular value for TTY before the assembly begins.

TRUE		EQU	OFFFFH	;DEFINE VALUE OF TRUE

FALSE		EQU	NOT TRUE	;DEFINE VALUE OF FALSE

TTY		EQU	TRUE		;TRUE IF TTY, FALSE IF CRT

TTYBASE 	EQU	10H		;BASE OF TTY I/O PORTS

CRTBASE 	EQU	20H		;BASE OF CRT I/O PORTS

IF	TTY		;ASSEMBLE RELATIVE TO TTYBASE

CONIN		EQU	TTYBASE	;CONSOLE INPUT

CONOUT	EQU	TTYBASE+l	;CONSOLE OUTPUT

ENDIF

IF	NOT TTY	;ASSEMBLE RELTIVE TO CRTBASE

CONIN		EQU	CRTBASE	;CONSOLE INPUT

CONOUT	EQU	CRTBASE+l	;CONSOLE OUTPUT

ENDIF

. . .

IN	CONIN		;READ CONSOLE DATA

. . .

OUT	CONOUT	;WRITE CONSOLE DATA

In this case, the program assembles for an environment where a

Teletype is connected, based at port 10H. The statement defining

TTY could be changed to:

TTY	EQU	FALSE

and, in this case, the program assembles for a CRT based at port

20H.

MP/M II Programmer's Guide	4.4		Assembler Directives

4.4.6		The DB Directive

The DB directive allows the programmer to define initialized

storage areas in single precision (byte) format. The statement form

is:

label		DB	e#l, e#2, ..., e#n

where e#l through e#n are either expressions that evaluate to 8-bit

values (the high-order eight bits must be zero) , or are ASCII

strings of length no greater than 64 characters. There is no

practical restriction on the number of expressions included on a

single source line. The expressions are evaluated and placed

sequentially into the machine code file starting at the current

program address generated by the assembler. String characters are

similarly placed into memory starting with the first character and

ending with the last character. Strings of length greater than two

characters cannot be used as operands in more complicated

expressions; that is, they must stand alone between the commas.

Note that ASCII characters are always placed in memory with the

parity bit reset (0), and that there is no translation from lower

to upper-case within strings. The optional label can reference the

data area throughout the remainder of the program. Examples of

valid DB statements are:

data:		DB	0,1,2,3,4,5

DB	data and Offh,5,377Q,1+2+3+4

signon:	DB	‘please type your name',cr,lf,O

DB	'AB' SHR 8, 'C', 'DE' AND 7FH

4.4.7		The DW Directive

The DW statement is similar to the DB statement except that it

initializes two byte words of storage instead of single bytes. The

form is:

label 	DW	e#1, e#2, . . ., e#n

e#l through e#n are expressions that evaluate to 16-bit results.

Note that ASCII strings of one or two characters are allowed, but

strings longer than two characters are not acceptable. In all

cases, the data storage is consistent with the 8080 processor: the

least significant byte of the expression is stored first in memory,

followed by the most significant byte. Here are some examples:

doub:		DW 	Offefh,doub+4,signon-$,255+255

DW	‘a’,5,’ab','CD',6 shl 8 or llb

MP/M II Programmer's Guide	4.4		Assembler Directives

4.4.8		The DS Directive

The DS statement reserves an area of uninitialized memory, and

takes the form:

label	DS 	expression

where the label is optional. The assembler begins subsequent code

generation after the area reserved by the DS. Thus, the DS

statement given above has exactly the same effect as the following

statements:

label:	EQU	$	;LABEL VALUE IS CURRENT CODE LOCATION

ORG		$+expression   ;MOVE PAST RESERVED AREA

4.5		Operation Codes

Assembly language operation codes are the principal part of

assembly language programs, and form the operation field of the

instruction. In general, ASM accepts all the standard mnemonics for

the Intel 8080 microcomputer, which are given in detail in the Intel

manual "8080 Assembly Language Programming Manual." Labels are

optional on each input line and, if included, take the value of the

instruction address immediately before the instruction is issued.

Table 4-7 lists the individual operators briefly, but you should

reference the Intel manual for detailed descriptions.

Table 4-7 lists each operation code in its most general form

with a specific example, then gives a short explanation with any

special restrictions. In the Form column, "en" symbolizes an

expression. Table 4-6, below, defines the "en" symbols.

Table 4-6. Expression Symbols

| Symbol | Represents |
| --- | --- |
| e3 a 3-bit value in the range 0-7 that can be one of the predefined registers A, B, C, D, E, H, L, M, SP, or PSW. e8 an 8-bit value in the range 0-255. e16 a 16-bit value in the range 0-65535. |  |

The expressions can be formed from an arbitrary combination of

operands and operators. In some cases, the operands are restricted

to particular values within the allowable range, such as the PUSH

instruction. Table 4-7 notes such cases as they are encountered.

The operation codes summarized in Table 4-7 fall into six

categories. Jump, Call and Return instructions can test the

condition flags set in the CPU and transfer control to another

MP/M IT Programmer's Guide	4.5		Operation Codes

location. Immediate Operand instructions load single- or double

precision registers or single-precision memory cells with constant

values. These also include instructions that perform immediate

arithmetic or logical operations on the accumulator (register A).

Increment and Decrement instructions are provided for both

single- and double-precision registers. Data Movement instructions

transfer data from memory to the CPU and from the CPU to memory.

Arithmetic Logic Unit instructions perform arithmetic and logical

operations on the single-precision accumulator. Control

instructions enable and disable interrupts, halt program execution,

and perform a no-operation function.

Table 4-7. ASM Operation Codes

| Form | Example | Explanation |
| --- | --- | --- |
| Jumps, Calls and Returns JMP e16 JMP Ll Jump unconditionally to label JNZ e16 JNZ L2 Jump on non zero condition to Label JZ e16 JZ 100H Jump on zero condition to label JNC e16 JNC L1+4 Jump on no carry to label ic e16 JC L3 Jump on carry to label JPO e16 JPO $+8 Jump on odd parity to label JPE e16 JPE L4 Jump on even parity to label JP e16 JP GAMMA Jump on positive result to label jmp e16 JM al Jump on minus to label CALL e16 CALL Sl Call subroutine unconditionally CNZ e16 CNZ S2 Call subroutine if non zero flag CZ e16 CZ 100H Call subroutine on zero flag CNC e16 CNC Sl+4 Call subroutine if no carry set cc e16 CC S3 Call subroutine if carry set CPO e16 CPO S+8 Call subroutine if parity odd CPE e16 CPE S4 Call subroutine if parity even |  |  |

MP/M II Programmer's Guide	4.5		Operation Codes

Table 4-7. (continued)

| Form | Example | Explanation |
| --- | --- | --- |
| Jumps, Calls and Returns CP e16 CP GAMMA Call subroutine if positive Result CM e16 CM bl$c2 Call subroutine if minus flag RST e3 RST 0 Programmed "restart", equivalent to CALL 8*e3, except one byte instruction RET Return from subroutine RNZ Return if non zero flag set RZ Return if zero flag set RNC Return if no carry RC Return if carry flag set RPO Return if parity is odd RPE Return if parity is even RP Return if positive result RM Return if minus flag is set Immediate Operand Instructions MVI e3,e8 MVI B,255 Move immediate data to register A, B, C, D, E, H, L, or M (memory) ADI e8 ADI 1 Add immediate operand to A without carry ACI e8 ACI OFFH Add immediate operand to A with Carry SUI e8 SUIL + 3 Subtract from A without borrow (carry) SBI e8 SBI L AND 11B Subtract from A with borrow (carry) ANI e8 ANI $ AND 7FH Logical "and" A with immediate data |  |  |

MP/M II Programmer's Guide	4.5		Operation Codes

Table 4-7. (continued)

| Form | Example | Explanation |
| --- | --- | --- |
| Immediate Operand Instructions XRI e8 XRI 1111$0000B "Exclusive or" A with immediate Data ORI e8 ORI L AND 1+1 Logical "or" A with immediate data CPI e8 CPI 'a' Compare A with immediate data (same as SUI except register A not changed) LXI e3,el6 LXI B,100H Load extended immediate to register pair (e3 must be equivalent to B, D, H, or SP) Increment and Decrement Instructions INR e3 INR E Single precision increment register(e3 produces one of A, B, C, D, E, H, L, M) DCR e3 DCR A Single precision decrement Register (e3 produces one of A, B, C, D, E, H, L, M) INX e3 INX SP Double precision increment register pair (e3 must be equivalent to B, D, H, or SP) DCX e3 DCX B Double precision decrement register pair (e3 must be equivalent to B, D, H, or SP) Data Movement Instructions MOV e3,e3 MOV A,B Move data to leftmost element from rightmost element (e3 produces one of A, B, C, D, E, H, L, or M). MOV M,M is Disallowed LDAX e3 LDAX B Load register A from computed address (e3 must produce either B or D) STAX e3 STAX D Store register A to computed address (e3 must produce either B or D) |  |  |

MP/M II Programmer's Guide	4.5		Operation Codes

Table 4-7. (continued)

| Form | Example | Explanation |
| --- | --- | --- |
| Data Movement Instructions LHLD e16 LHLD Ll Load HL direct from location e16 (double precision load to H and L) SHLD e16 SHLD L5+x Store HL direct to location e16 (double precision store from H and L to memory) LDA e16 LDA Gamma Load register A from address e16 STA e16 STA X3-5 Store register A into memory at e16 POP e3 POP PSW Load register pair from stack, set SP (e3 must produce one of B, D, 11, or PSW) PUSH e3 PUSH B Store register pair into stack, set SP (e3 must produce one of B, D, H, or PSW) IN e8 IN 0 Load register A with data from port e8 OUT e8 OUT 255 Send data from register A to port e8 XTHL Exchange data from top of stack with HL PCHL Fill program counter with data from HL SPHL Fill stack pointer with data from HL XCHG Exchange DE pair with HL pair Arithmetic Logic Unit Operations ADD e3 ADD B Add register given by e3 to accumulator without carry (e3 must produce one of A, B, C, D, E, H, or L) ADC e3 ADC L Add register given by e3 to A with carry, (e3 must produce one of A, B, C, D, E, H, or L) |  |  |

MP/M II Programmer's Guide	4.5 Operation Codes

Table 4-7. (continued)

| Form | Example | Explanation |
| --- | --- | --- |
| Arithmetic Logic Unit Operations SUB e3 SUB H Subtract register given by e3 from A without carry, (e3 must produce one of A, B, C, D, E, H, or L) SBB e3 SBB 2 Subtract register given by e3 from A with carry, (e3 must produce one of A, B, C, D, E, H, or L) ANA e3 ANA 1+1 Logical "and" of Register given by e3 with A, (e3 must produce one of A, B, C, D, E, H, or L) XRA e3 XRA A "Exclusive or" of register given by e3 with A, (e3 must produce one of A, B, C, D, E, H, or L) ORA e3 ORA B Logical "or" of register given by e3 with A, (e3 must produce one of A, B, C, D, E, H, or L) CMP e3 CMP Compare register given by e3 with A, (e3 must produce one of A, B, C, D, E, H, or L) DAA Decimal adjust register A based upon last arithmetic logic unit operation CMA Complement bits in register A STC Set carry flag to 1 CMC Complement carry flag RLC Rotate bits left, (re)set carry as a side effect (high-order A bit becomes carry) RRC Rotate bits right, (re)set carry as side effect (low order A bit becomes carry) RAL Rotate carry/A register to left (carry is involved in the rotate) |  |  |

MP/M II Programmer's Guide	4.5		Operation Codes

Table 4-7. (continued)

| Form | Example | Explanation |
| --- | --- | --- |
| Arithmetic Logic Unit Operations RAR Rotate carry/A register to right (carry is involved in the rotate) DAD e3 DAD B Double precision add register pair e3 to HL (e3 must produce B, D, H, or SP) Control Instructions HLT Halt the 8080 processor DI Disable the interrupt system EI Enable the interrupt system NOP No operation |  |  |

MP/M II Programmer’s Guide	4.6		Error Messages

4.6		Error Messages

When ASM finds errors within the assembly language program, it

lists them as single-character codes in the leftmost position of the

source listing. The line in error is also echoed at the console so

that the source listing need not be examined to determine if errors

are present. Table 4-8 defines the error codes.

Table 4-8. Assembly Error Codes

| Code | Meaning |
| --- | --- |
| D Data error: element in data statement cannot be placed in the specified data area E Expression error: expression is ill-formed and cannot be evaluated L Label error: label cannot appear in this context may be duplicate label N Not implemented: features that will appear in future ASM versions (e.g., Macros) are recognized and flagged, but are unsupported in this version 0 Overflow: expression is too complicated, has too many pending operators to be computed; simplify it P Phase error: label does not have the same value on two subsequent passes through the program R Register error: the value specified as a register is not compatible with the operation code S Syntax error: the fields of this statement are ill Formed and cannot be processed properly; may be due to invalid characters or misplaced delimiters U Undefined symbol: label operand in this statement has not appeared elsewhere on the left side of a statement that generates machine code or reserves memory space, as in a MOV instruction, a DS pseudo operation, or an EQU or SET directive V Value error: operand encountered in expression is improperly formed |  |

MP/M II Programmer's Guide	4.6		Error Messages

Several error message can be printed at the terminal if a disk

error condition occurs. Table 4-9 summarizes these error messages.

Table 4-9. ASM Terminal Messages

| Message | Meaning |
| --- | --- |
| NO SOURCE FILE PRESENT The file specified in the ASM command does not exist on disk. NO DIRECTORY SPACE The disk directory is full; erase files that are not needed, then retry. SOURCE FILE NAME ERROR Improperly formed ASM filename wildcard ? and * characters are not allowed. SOURCE FILE READ ERROR Source file cannot be read properly by the assembler; type file at console to determine the point of error. OUTPUT FILE WRITE ERROR output files cannot be written properly; most likely cause is a full disk; erase and retry. CANNOT CLOSE FILE Output file cannot be closed; check to see if disk is write protected. |  |

Section 5

RDT

5.1		RDT Overview

The Relocatable Debugging Tool (RDT) allows the user to test

and debug programs in the MP/M II environment. Multiple RDTs can be

relocated for execution in a non-banked system, or assigned absolute

memory locations for execution in a bank-switched system. RDT

commands are a superset of the CP/M debugger, DDT (see the command

summary in Table 5-1.) The additional commands allow RDT to debug

relocatable code and save patched programs. However, there is one

important difference between RDT and DDT. RDT is a PRL file and DDT

is a COM file. Thus RDT can debug both COM and PRL files, while DDT

can only debug COM files. Note: RDT cannot read a file that is

password protected.

5.2		Invoking RDT

Invoke RDT by entering one of the following commands:

RDT

RDT filespec

The first command simply loads and executes RDT. After

displaying its sign-on message and prompt character, RDT is ready to

accept operator commands. The second command is similar to the

first, except that after RDT is loaded, it loads the file specified

by filespec.

The second command is equivalent to first invoking RDT and then

using the I (Input) command to insert a filename into the default

FCB at Base+005CH, as shown in the following sequence.

OA>RDT

00:22:55 A:RDT		.PRL	(USER n)

[MP/M] DDT VERS 2.0

NEXT PC

0100 0100

-1 filespec

-R

-

At this point, the program named by filespec is loaded and ready for

debugging.

MP/M II Programmer's Guide	5.3		RDT Command Conventions

5.3		RDT Command Conventions

When RDT is ready to accept a command, it prompts the user with

a hyphen, -. In response, the user can type a command line or a

CONTROL-C (represented as ^C) to end the debugging session (see

Section 5.4) . A command line can have up to 32 characters, and must

be terminated with a carriage return. While entering commands, use

the standard MP/M II line-editing functions (^X, ^H, etc.) to

correct typing errors. RDT does not process the command line until

a carriage return is entered.

The first character of each command line determines the command

action. Table 5-1 summarizes RDT commands. RDT commands are

defined individually in Section 5.5.

Table 5-1. RDT Command Summary

| Command | Action |
| --- | --- |
| A enter assembly language statements *B set or reset bitmap bits D display memory in hexadecimal and ASCII F fill memory block with a constant G begin execution with optional breakpoints I set up file control block and command tail L list memory using assembler mnemonics M move memory block *N normalize and relocate program to RDT's memory segment R read disk file into memory S set memory to new values T trace program execution U untraced program monitoring *V compute parameter value for W command *W write contents of memory block to disk X examine and modify CPU state *RDT only |  |

The command character can followed by one or more arguments,

which may be hexadecimal values, filenames or other information,

depending on the command. RDT assumes all values the user enters

are hexadecimal. If the user enters more than four digits, RDT

truncates them on the left; that is, RDT only uses the last four.

Arguments should be separated from each other by commas or spaces.

Note: no spaces are allowed between the command character and the

first argument.

MP/M II Programmer's Guide	5.4		Terminating RDT

5.4		Terminating RDT

The user terminates RDT by typing a TC in response to the

hyphen prompt. RDT responds with the query:

Abort (Y/N) ?

Note: MP/M II does not have the SAVE facility found in CP/M so if

RDT is used to patch a file, the user must write the file to disk

using the W command before terminating RDT.

5.5		RDT Commands

This section defines RDT commands and their arguments. RDT

commands give the user control of program execution and allow the

user to display and modify system memory and the CPU state.

5.5.1		The A (Assemble) Command

The A command assembles 8080 mnemonics directly into memory.

The form is:

Aa

where a is the hexadecimal address where assembly is to start. RDT

responds to the A command by displaying the address of the memory

location where assembly is to begin. At this point, the user enters

assembly language statements. When a statement is entered, RDT

converts it to machine code, places the value(s) in memory, and

displays the address of the next available memory location. This

process continues until the user enters a blank line or a line

containing only a period.

RDT responds to invalid statements by displaying a question

mark,?, and redisplaying the current assembly address.

5.5.2		The B (Bitmap Bit Set/Reset) Command

The B command enables the user to update the bitmap of a page

relocatable file. The user reads the file in, makes changes to the

code, and then determines the bytes that need relocation (i.e. the

high-order address bytes of jump instructions). The user then

updates the bitmap with the B command. There are two parameters

specified: the address to be modified, followed by a 0 to reset a

byte previously marked for relocation or a 1 to mark a byte for

relocation. The form is:

Ba,n

where a is the hexadecimal address, and n is either a 0 or 1.

MP/M Ii Programmer's Guide	5.5		RDT Commands

5.5.3		The D (Display) Command

The D command displays the contents of memory as 8-bit

hexadecimal values and in ASCII. The forms are:

D

Ds

Ds,f

where s is the hexadecimal address where the display is to start,

and f is the address where the display is to finish. In response to

the first form shown above, RDT displays memory from the current

display address for 16 display lines. The response to the second

form is similar to the first, except that the display address is

f first set to the address s. The third form displays the memory

block between locations s and f.

Memory is displayed on one or more display lines. Each display

line shows the values of up to 16 memory locations. For the first

three forms, the display line appears as follows:

aaaa bb bb . . . bb cc . . . c

where aaaa is the display address in hexadecimal: bbs represents

the contents of the memory locations in hexadecimal, and cs

represents the contents of memory in ASCII. Any non-displayable

ASCII characters are represented by periods.

During a long display, type any character at the console to

abort the D command.

5.5.4		The F (Fill) Command

The F command fills an area of memory with a byte constant.

The form is:

Fs,f,b

where s is a hexadecimal starting address of the block to be filled,

f is the ending address, and b is the hexadecimal byte constant.

RDT stores the 8-bit value b in locations s through f by first

storing b at address s, then incrementing the value of s and testing

it against f. The process repeats until s exceeds f.

MP/M II Programmer's Guide	5.5		RDT Commands

5.5.5		The G (Go) Command

The G command transfers control to the program being tested,

and optionally sets one or two breakpoints. The forms are:

G

G,bl

G,bl,b2

Gs

Gs,bl

Gs,bl,b2

where s is a hexadecimal address where program execution is to

start, and bl and b2 are hexadecimal addresses of breakpoints.

In the first three forms, no starting address is specified, so

RDT starts execution of the program under test at the current value

of the program counter. (Use an X command to determine the current

value of the counter). The first form transfers control to the

user’s program without setting any breakpoints. The next two forms

respectively set one and two breakpoints before passing control to

the user's program. The next three forms are analogous to the first

three, except that the user's program counter is first set to s.

Once control is transferred to the program under test, it

executes in real time until a breakpoint is encountered. At this

point, RDT regains control, clears all breakpoints, and indicates

the address at which execution of the program under test was

interrupted as follows:

*aaaa

where aaaa is the hexadecimal address where the break occurred.

When a breakpoint returns control to RDT, the instruction at the

breakpoint address has not yet been executed.

5.5.6		The I (Input Command Tail) Command

The I command inserts a filename into the default FCB at

Base+005CH, relative to the base of the segment in which RDT is

loaded. The form is:

I<command tail>

where <command tail> is a character string that usually contains one

or more filenames. The first filename is parsed into the default

FCB at Base+005CH. The optional second filename (if specified) is

parsed into the second part of the default FCB beginning at

Base+006CH.

MP/M II Programmer's Guide	5.5		RDT Commands

5.5.7		The L (List) Command

The L command lists the contents of memory in assembly

language. The forms are:

L

Ls

Ls,f

where s is the hexadecimal address where the list is to start, and f

is the hexadecimal address where the list is to finish.

The first form lists 11 lines of disassembled machine code from

the current list address. The second form sets the list address to

s and then lists 11 lines of code. The last form lists disassembled

code from s through f. In all three cases, the list address is set

to the next unlisted location in preparation for a subsequent L

command. When RDT regains control from a program being tested (see

G, T and U commands) , the list address is set to the current value

of the program counter.

Abort long displays by typing any key during the list process.

Or, enter ^S to halt the display temporarily.

5.5.8		The M (Move) Command

The M command moves a block of data values from one area of

memory to another. The form is:

Ms,f,d

where s is the hexadecimal starting address of the block to be

moved; if is the address of the final byte to be moved, and d is

the destination address of the f first byte to receive the data.

Note: if d is between s and f, part of the block being moved is

overwritten before it is moved, because data is transferred starting

from location s.

5.5.9		The N (Normalize) Command

The N command adjusts the relocatable addresses of the page

relocatable file that RDT reads into memory. The user reads the

file into memory with the R command, and then uses the N command to

prepare the file for execution within the memory segment where RDT

is executing. The form is:

N

MP/M II Programmer's Guide	5.5		RDT Commands

5.5.10		The R (Read) Command

The R command is used in conjunction with the I command to read

files from disk into the TPA in preparation for debugging. The

forms are:

R

Rb

where b is an optional bias address that is added to each program or

data address as it is loaded. The load operation must not overwrite

any of the system parameters from OOOH through OFFH (i.e., the base

page of the TPA where RDT is loaded) . If b is omitted, RDT assumes

b=OOOOH. The R command requires a previous I command, specifying a

valid filename. The load address for each record from a HEX file is

obtained from each individual HEX record. RDT assumes any file

specified as type COM contains machine code in the Intel hex format.

Other files are assumed to be in pure binary format.

The user can issue any number of R commands following an I

command to re-read the program being debugged, if the program does

not destroy the default FCB at Base+005CH.

Recall that the sequence:

OA>RDT

-Ifilespec

-R

is equivalent to:

OA>RDT filespec

When the user issues the R command, RDT responds with a load

message in the form:

NEXT	 PC

nnnn pppp

where nnnn is the next address following the loaded program, and

pppp is the assumed program counter taken from the last record if a

HEX file is specified; for other files, it is assumed to be the

base of the TPA.

5.5.11		The S (Set) Command

The S command can examine or alter the contents of an

individual byte in memory. The form is:

Sa

where a is the hexadecimal address to be examined or altered.

MP/M II Programmer's Guide	5.5		RDT Commands

RDT displays the memory address and its current contents on the

following line in the following form:

aaaa bb

where aaaa is the hexadecimal address, and bb is the byte contents

of memory in hexadecimal. The user can then choose to alter the

memory location or to leave it unchanged. If the user enters a

valid hexadecimal value, RDT replaces the contents of the byte in

memory with the new value. If no value is entered, the contents of

memory are unaffected and the contents of the next address are

displayed. In either case, RDT continues to display successive

memory addresses and values until either a period or an invalid

value is entered.

5.5.12		The T (Trace) Command

The T command traces program execution for 1 to OFFFFH program

steps. The forms are:

T

Tn

where n is the number of instructions to execute before returning

control to the console.

In response to the first form, RDT displays the CPU state and

executes the next program step. The program terminates immediately,

with the termination address displayed in the form:

*hhhh

where hhhh is the next address to execute. The user sets the

display address (used in the D command) to the value of registers H

and L, and sets the list address (used in the L command) to the

value hhhh. The user can then examine the CPU state at program

termination by using the X command.

The second form is similar to the first, except that RDT traces

program execution for n steps before a breakpoint occurs. The user

can force a breakpoint in the trace mode by typing a rubout

character. RDT again displays the CPU state before each program

step in the same format as described in the X command.

In either case, RDT transfers control to the program under test

at the address indicated by the program counter. If the user does

not specify n, RDT executes only one instruction. The user can

abort a long trace before n steps are executed by typing any

character at the console.

Note: Program tracing stops at the interface to MP/M II, and

resumes after return from MP/M II to the program under test. Thus,

MP/M II functions that access I/O devices such as disk drives, run

in real time and avoid I/O timing problems. Programs running in

MP/M II Programmer's Guide	5.5		RDT Commands

trace mode execute approximately 500 times slower than real time

since RDT gets control after each instruction is executed.

5.5.13		The U (Untrace) Command

The U command is identical to the T command except that the CPU

state is displayed only before the first instruction is executed,

rather than before every step. The forms are:

U

Un

where n is the number of instructions to execute before returning

control to the console. Abort U command by striking any key at the

console.

5.5.14		The V (Value) Command

The V command facilitates use of the W command by computing the

parameter to follow the "W". A single parameter immediately follows

the "V" which is the NEXT location following the last byte to be

written to disk.

Normally, the user reads in the file, edits it, and then writes

it back to disk. The read command, R, produces -a value for NEXT.

This value can be entered as a parameter following the V command,

and RDT computes and displays the number of sectors to be written

out using the W command. The form is:

V

5.5.15		The W (Write) Command

The W command writes the contents of a contiguous block of

memory to disk. The form is:

Wn

where n is the value of the parameter obtained from the V command,

and is the number of sectors to be written to disk. The user enters

the value for n in hexadecimal.

MP/M II Programmer's Guide	5.5		RDT Commands

5.5.16		The X (Examine CPU State) Command

The X command allows the user to examine and alter the CPU

state of the program under test. The forms are:

X

Xr

Xf

where r is the name of one of the 8080 CPU registers and f is the

abbreviation of one of the CPU flags. The first form displays the

CPU state in the format:

CfZfMfEfIf A=bb B=dddd D=dddd H=dddd S=dddd P=dddd inst

where f is the flag value, 0 or 1; bb is the byte value, and dddd

is the double byte quantity corresponding to the register pair. The

"inst" field contains the disassembled instruction that occurs at

the location addressed by the CPU state's program counter.

The second form displays and allows the user to alter the

register values, where r is one of the registers listed in Table 5

2.

The third form displays and allows the user to alter the values

of the flags listed in Table 5-2.

Table 5-2. 8080 CPU Flags and Registers

| Flag | Definition | Values |
| --- | --- | --- |
| C Carry Flag (0/1) Z Zero Flag (0/1) M Minus Flag (0/1) E Even Parity Flag (0/1) I Interdigit Carry (0/1) Register Definition Values A Accumulator (0-FF) B BC register pair (0-FFFF) D DE register pair (0-FFFF) H HL register pair (0-FFFF) S Stack Pointer (0-FFFF) P Program Counter (0-FFFF) |  |  |

In each case, RDT first displays the flag or register value,

and then accepts input at the console. If the user enters a value

in the proper range, RDT alters the flag or register. Entering a

carriage return does not alter the value. Note: RDT displays the

BC, DE, and HL registers as register pairs. Thus, typing B alters

the BC register pair; D alters the DE register pair, etc.

Section 6

Other Programming Utilities

6.1 GENHEX

Syntax:

genhex{d:}filename{.typ}xxx

GENHEX accepts a COM file as input and changes it to a HEX

file. This utility is useful for generating HEX files as input for

the GENMOD utility.

If no filetype is specified, GENHEX assumes a type of COM. In

the syntax line above, xxx is the offset specified for the HEX file.

GENHEX is non-destructive. That is, it does not alter the original

COM file. The following is an example of a GENHEX command:

OA>GENHEX B:PROGRAM.COM 100

6.2 GENMOD

Syntax:

genmod {d:}filename.hex{d:}filename.prl$nnnn

GENMOD produces a PRL file from a HEX file. The user first

concatenates two HEX files generated from the same source file.

The HEX files are offset from each other by 100H bytes. GENMOD

accepts the concatenated file as input, and then prepares a PRL

file by generating a header record, a code and data segment, and a

bit map.

In the syntax line above, nnnn is an optional parameter that

can be used to specify an additional amount of memory required by

the program beyond the code space. The form of parameter is 11$11

followed by four HEX digits. For example, if a program is written

to use all of available memory for buffers, specifying the optional

parameter ensures a minimum buffer allocation. GENMOD is non

destructive. That is, it does not alter the original HEX file. The

following is an example of a GENMOD command:

OA>GENMOD B:PROGRAM.HEX A:PROGRAM.PRL.$1000

MP/M II Programmer's Guide         6.3	PRLCOM

  - PRLCOM

Syntax:

prlcom{d:}filename.prl{d:}filename.com

PRLCOM accepts a source PRL file, and produces a COM file by

removing the header record and the bit map. PRLCOM is non

destructive. That is, it does not alter the original PRL file.

The destination file can be on the same or a different drive, but if

it already exists, PRLCOM queries the user:

Destination File Exists, delete (Y/N)?

Responding with N aborts PRLCOM. The following is an example of a

PRLCOM command:

OA>PRLCOM B:PROGRAM.PRL A:PROGRAM.C

6.4		DUMP

Syntax:

dump {d:}filename.typ

DUMP displays the contents of a disk file in hexadecimal

format. The following is an example of a DUMP command:

OA>DUMP PROGRAM.COM

The filename must be unambiguous (i.e. no wildcard characters)

Note: DUMP does not display the contents of a password protected

file. DUMP displays the file's contents at the console, 16 bytes at

a time, with the absolute byte address listed to the left of each

line as shown in the example below:

0000 CD 8A 02 1F D2 10 02 CD 58 02 32 64 03 CD D3 02 0010

CD 71 02 43 66 D9 01 57 OE 01 2D F5 05 3A 2E 04 0020 FE CA

A2 E5 B3 32 02 E6 45 00 00 00 00 00 00 00 0030 00 00 00 00

00 00 00 00 00 00 00 00 00 00 00 00

.

.

.

The user can send the output to the printer by typing CONTROL-P

before entering the DUMP command, and start and stop the output at

the console with CONTROL-S/CONTROL-Q. Type any key to abort the

DUMP program.

MP/M II Programmer's Guide            6.5  LOAD

6.5		LOAD

Syntax:

load {d:}filename{.typ}

LOAD accepts as input a HEX file and produces as output a COM

file, which can then be executed. LOAD is non-destructive. That

is, it does not alter the original HEX file. If no filetype is

specified in the command line, LOAD looks for a file of type HEX on

the disk. The following is an example of a LOAD command:

OA>LOAD B:PROGRAM.HEX

Section 7

PRL File Generation

7.1		PRL Format

A Page Relocatable Program is stored on disk as a file of type

.PRL. The format is shown in Table 7-1.

Table 7-1. PRL File Format

Address			Contents

0001-0002H		Program size

0004-0005H		Minimum buffer requirements (additional

memory)

0006-OOFFH		Currently unused, reserved for future

allocation

0100H + Program size = Start of bit map

The bit map is a string of bits identifying those bytes in the

source code that require relocation. There is one byte in the bit

map for every 8 bytes of source code. The most significant bit (bit

7) of the f first byte of the bit map indicates whether or not the

first byte of the source code requires relocation. If the bit is

lion", it indicates that relocation is required. The next bit, (bit

6), of the first byte corresponds to the second byte of the source

code, and so forth.

7.2 Generating a PRL

The preferred technique for the generation of a . PRL file is to

use the Digital Research Link-80 which is capable of generating a

.PRL file from a REL relocatable object file. This technique is

described in detail in the Link-80 Manual. A sample link command is

shown below:

OA>Iink dump[op]

An alternate method of generating a PRL file is to use the

Digital Research assembler, ASM. This technique is described below.

A Page Relocatable Program can be generated by assembling the

source code twice; during the second assembly, 100H is added to

each ORG statement. The two HEX files generated by assembling the

MP/M II Programmer's Guide	7.2		Generating a PRL

source code twice are concatenated with PIP, and the resulting file

is then provided as input to GENMOD, which produces an output file

of type PRL. The GENMOD utility is described in Section 6.2.

Appendix G contains a sample Page Relocatable Program. The

code in the example program illustrates the required use of ORG

statements to access the BDOS and the default FCB. The following

points should be noted:

O The initial ORG is at OOOOH. This establishes the equate

for the symbol BASE, the base of the relocatable segment.

O The ORG 100H statement establishes the actual beginning of

code for the program. During the second assembly, these

two ORG statements are changed to 100H and 200H,

respectively.

O The first assembly generates a file that can be changed

into an executable COM file with the LOAD utility. In

fact, it is desirable to first debug the program as a COM

file and then make it a PRL file.

O It is vital to use BASE to offset all memory segment base

page references. The program cannot make BDOS calls to

absolute 0005H. In this example, BASE is used to offset

the BDOS, FCB, and BUFF equates. If a program needs to

determine the top of its memory segment, the following

equate and code sequence can be used:

MEMSIZE 	EQU 	BASE+6

LHLD	MEMSIZE ;HL = TOP OF MEMORY SEGMENT

The following steps show how to generate a Page

Relocatable File for this example using the Digital

Research Assembler, ASM:

1. Prepare the program, DUMP.ASM in the example, with proper

ORG statements as described above.

2. Assuming a system disk is on drive A and the DUMP.ASM

file is on drive B, enter the commands:

1A>ASM B:DUMP

;assemble the DUMP.ASM file

1A>ERA B:DUMP.HXO

1A>REN B:DUMP.HX0=B:DUMP.HEX

1A>PIP LST:=B:DUMP.PRN[T8]

1A>ERA B:DUMP.PRN

MP/M II Programmer's Guide	7.2		Generating a PRL

3. Edit the DUMP.ASM file, adding 100H to each ORG

statement. This can be done by concatenating a preamble

to the program that contains the two initial ORG

statements. A submit file to perform this function,

named ASMPRL.SUB is provided on the distribution

diskette.

1A>ASM B:DUf4P.BBZ

;assemble the DUMP.ASM file a second time

1A>PIP B:DUMP.HEX=B:DUMP.HXO,B:DUMP.HEX

;concatenate the HEX files

1A>GENMOD B:DUMP.HEX B:DUMP.PRL

;generate the relocatable DUMP.PRL file

Section 8

RSP Generation

8.1		RSPs and Resident System Procedures

Resident System Processes are included with MP/M II during

system generation. GENSYS searches the directory for all files with

the type RSP, displays the filenames, and then prompts the user as

to whether it should be included in the generated system file,

MPM.SYS.

MP/M II also supports a special type of RSP called the Resident

System Procedure. A Resident System Procedure provides a method of

serially utilizing a block of code as a system resource. A Resident

System Procedure is created by an RSP. The RSP creates a queue with

the name of the Resident System Procedure and sends it a single two

byte message containing the address of the procedure to be accessed

serially. The RSP then terminates itself.

8.2		Generating an RSP

The method of generating an' RSP is analogous to that of

generating a Page Relocatable Program (as described in Section 7)

with the following exceptions:

O If LINK-80 is used, the output file type of RSP is

specified with the [or] option.

O The GENMOD output file is designated RSP rather than PRL

O The code in the RSP is ORGed at OOOH rather than 100H.

8.3		RSP Code

Appendix F contains a sample Resident System Process. The code

in the example program illustrates the required structure of an RSP

as well as the BDOS/XDOS access mechanism. This example should be

studied carefully and the following points noted:

o The first two bytes of an RSP are set to the address of the

BDOS/XDOS entry point. The address is filled in by the

loader; an RSP can simply access the BDOS/XDOS by loading

HL from the base of the program area and then executing a

PCHL instruction.

O The Process Descriptor for the RSP must immediately follow

the first two bytes that contain the address of the

BDOS/XDOS entry point. It is important to note the manner

in which the Process Descriptor is initialized. DS

MP/M II Programmer's Guide	8.3		RSP Code

instructions are used where storage is simply allocated,

while DB and DW instructions are used where data in the

Process Descriptor must be initialized. Note: the stack

pointer field of the Process Descriptor points to the

address immediately following the stack allocation. This

is the return address and is the actual process entry

point.

O The HEX file generated by assembling the RSP must span the

entire program and data area. To ensure this, use a DW

instruction to define the first two bytes of the RSP that

contain the address of the BDOS/XDOS entry point. Using a

DS instruction does not generate any HEX file code for

those two bytes. The end of the program and data area must

be defined in a similar manner. If the RSP has DS

instructions preceding the END statement, it is necessary

to place a DB statement after the DS statements and before

the END statement.

8.4		Banked RSPs

MP/M II supports a form of an RSP called a banked RSP which

consists of two parts: a resident portion and a banked portion.

The resident portion has the Process Descriptor for the RSP, and any

other data structures such as queues, which must be in common

memory. The banked portion of filetype BRS contains the remainder

of the RSP, usually including the actual code, stack and other data

structures. The resident portion of a banked RSP must follow the

rules given in the previous section for RSPs. The presence of a

banked portion of an RSP is specified by setting the Process

Descriptor memory segment index to zero rather than FFH. The name

provided in the Process Descriptor is used to obt6in the banked

portion of the RSP that has a file type of BRS. The following

points should be noted about a BRS:

o Bytes 0000-0001H of the banked RSP are reserved for the

address of the resident portion of the RSP. Thus, a banked

RSP must access the BDOS/XDOS functions by indirectly

loading from the two bytes at relative 0000-0001H which

point to the base of the resident portion of the RSP which

in turn contain the BDOS/XDOS entry point address.

O Bytes 0002-0003H of the banked RSP must contain the initial

stack pointer value for the process. Thus the stack for

the banked RSP is in the banked portion of the RSP and

should be initialized such that the return address on top

of the stack is the banked RSP entry point address.

O Bytes 0004-OOOBH of the banked RSP must contain an ASCII

name for the process. This is used for display purposes

during GENSYS and MPMLDR execution.

Section 9

SPR Generation

9.1		System Page Relocatable Files

System Page Relocatable Files are placed in the MPM.SYS file

during system generation. A number of SPR files are provided as

part of the standard MP/M II: the resident and banked portions of

the BDOS, named RESBDOS.SPR and BNKBDOS.SPR; the resident and

banked portions of the XDOS, named XDOS.SPR and BNKXDOS.SPR; and

the banked TMP, named TMP.SPR.

Another SPR file named the RESXIOS.SPR or BNKXIOS.SPR contains

a user customized XIOS that is unique to the hardware on which MP/M

II is executing. This section gives an overview of the generation

technique for this custom SPR. A detailed discussion of the

generation of RESXIOS.SPR or BNKXIOS.SPR is provided in Section 1.3

of the MP/M II System Guide.

9.2		Generating an SPR

The method of generating an SPR is analogous to that of

generating a Page Relocatable Program (as described in Section 7)

with the following exceptions:

o if LINK-80 is used, the output file of type SPR is

specified with the [os] option.

O the GENMOD output file is designated SPR rather than PRL.

O the code in the SPR is ORGed at OOOH rather than 100H.

Appendix A

Flag Assignments

+-------+

:  0    :	Reserved

+-------+

:  1    :	System time unit tick

+-------+

:  2    :	One second interval

+-------+

:  3    :	One minute interval

+-------+

:  4    :	Undefined

:       :

:   .   :	.

:       :

:   .   :	.

:       :

:   .   :	.

+-------+

:   31  :

+-------+

Figure A-1. Flag Assignments

Appendix B

Process Priority Assignments

| 0 - 31 32 - 63 64 - 197 198 199 200 201 - 254 255 | Interrupt handlers System processes Undefined Terminal Message Processes Command Line Interpreter Default user priority User processes Idle process |
| --- | --- |

Appendix C

BDOS Function Summary

Table C-1. BDOS Function Summary

| Number | Function Name | Input Parameters | Returned values |
| --- | --- | --- | --- |
| 0 System Reset none none 1 Console Input none A = char 2 Console Output E = char none 3 Raw Console Input none A = char 4 Raw Console Output E = char none 5 List Output E = ch6r none 6 Direct Console I/O see def see def 7 Get I/O Byte Not supported under MP/M II 8 Set I/O Byte Not supported under MP/M II 9 Print String DE = .Buffer none 10 Read Console Buffer DE = .Buffer see def 11 Get Console Status none A = 00/01 12 Return Version Number none HL= Version# 13 Reset Disk System none see def 14 Select Disk E = Disk Number see def 15 open File DE = .FCB A = Dir Code 11 Close File DE = .FCB A = Dir Code 17 Search for First DE = .FCB A = Dir Code 18 Search for Next none A = Dir Code 19 Delete File DE = .FCB A = Dir Code 20 Read Sequential DE = .FCB A = Err Code 21 Write Sequential DE = .FCB A = Err Code 22 Make File DE = .FCB A = Dir Code 213 Rename File DE = .FCB A = Dir Code 24 Return Login Vector none HL= Login Vect* 25 Return Current Disk none A = Cur Disk# 26 Set DMA Address DE = .DMA none 27 Get Addr(Alloc) none HL= Alloc 28 Wtite Protect Disk none see def 29 Get R/O Vector none HL= R/O Vect* 30 Set File Attributes DE = .FCB see def 3l Get Addr(disk parms) none HL= DPB 32 Set/Get User Code see def see def 33 Read Random DE = .FCB A = Err Code 34 Write Random DE = .FCB A = Err Code 35 Compute File Size DE = .FCB ro, rl, r2 36 Set Random Record DE = .FCB rO, 11, L2 37 Reset Drive DE = drive Vect A = Err Code 38 Access Drive DE = drive Vect none 19 Free Drive DE = drive Vect none 40 Random w 0-fiil DE = .FCB A = Err Code 41 Test and Write Record DE = .FCB HL = Err Code 42 Lock Rtcord DE = .FCB HL = Err Code (Current DMA Addr -> File ID) |  |  |  |

MP/M II Programmer's Guide	Appendix C		BDOS Function Summary

Table C-1. (continued)

| Number | Function Name | Input Parameters | Returned values |
| --- | --- | --- | --- |
| 43 Unlock Record DE = .FCB HL = Err Code (Current DMA ADDR -> File ID) 44 Set Multi-Sector Count E = #of Sectors A = Rtn Code 45 Set BDOS Error Mode see def none 46 Get Disk FrLe Spece E = Disk # see def 47 Chain To Program see def none 48 Flush Buffers none see def 100 Set Directory Label DE = .FCB HL = Dir Code 101 Return Directory Label E = Disk # A = Label Data 102 Read File XFCB DE = .XFCB HL = Dir Code 103 Write File XFCB DE = .XFCB HL = Dir Code 104 Set Date and Time DE = .TOD none 105 Get Date and Time DE = .TOD none 106 Set Default Password DE = .Password none 107 Return Serial Number DE = .serialnmb serialnmb set |  |  |  |

The following abbreviations are used in the table.

char = ASCII character

Dir = Directory

Err = Error

Vect = Vector

Note: DE refers to register pair DE; HL refers to register pair

HL, and that A = L, and B = H upon return.

Appendix D

XDOS Function Summary

Table D-1. XDOS Function Summary

| Number | Function Name | Input Parameters | Returned values |
| --- | --- | --- | --- |

128 Absolute Memory Rqst DE = .MD A = Err Code 129 Relocatable Mem Rqst DE = .MD A = Err Code 130 Memory Free DE = .MD none 131 Poll E = Device none 132 Flag Wait E = Flag A = Err Code 133 Flag Set E = Flag A = Err Code 134 Make Queue DE= .QCB none 135 Open Queue DE= .UQCB A = Err Code 136 Delete Queue DE = .QCB A = Err Code 137 Read Queue DE = .UQCB none 138 Conditional Read Queue DE = .UQCB A = Err Code 139 Write Queue DE = .UQCB none 140 Conditional Write Queue DE = .UQCB A = Err Code 141 Delay DE = #ticks none 142 Dispatch none none 143 Terminate Process E = Term. Code none 144 Create Process DE = .PD none 145 Set Priority E = Priority none 146 Attach Console none none 147 Detach Console none none 148 Set Console E = Console none 149 Assign Console DE = .APB A= Err Code 150 Send CLI Command DE = .CLICMD none 151 Call Resident Sys Proc DE = .CPB HL= result 152 Parse Filename DE = .PFCB see def 153 Get Console Number none A= console # 154 System DE~ta Address none HL = Sys Data Addr 155 Get Date and Time DE = .TOD none 156 Return PD Addr none HL = PD Addr 157 Abort Spec. Process DE = .ABTPB A = Err Code 158 Attach List none none 159 Detach List none none 160 Set List E = List # none 161 Cond. Attach List none A = Err Code 162 Cond. Attach Console none A = Err Code 163 NIPM Version Number none HL = Version# 164 Get List Number none A = list #

The following abbreviations are used in the table.

Addr = Address

Cond. = Conditional

Proc = Process

Rqst = Request

Spec. = Specified

term. = Terminate

Appendix E

Sample Page Relocatable Program

********************************************

*  Note:                                   *

*       This program listing has been      *

*  included only as a sample and may not   *

*  reflect changes required by later MP/M  *

*  releases. For this reason the reader    *

*  should assemble and list the program    *

*  as provided on the distribution disk.   *

********************************************

page	0

0000			org	0000h

0000		base	equ	$

0100			org 	0100h

;note: either baseOlOO.asm or base02OO.asm must be ap

;to the beginning of this file before assembling.

;	title	'file dump program'

;	file dump program, reads an input file and

;	prints in hex

;copyright (c) 1975, 1976, 1977,.1978, 1979, 19

;digital research

;box 579, pacific grove

;california, 93950

0005	=	bdos		equ	base+5	;dos entry point

0001	=	cons		equ	1		;read console

0002	=	typef	equ	2		;type function

0009	=	printf	equ	9		;buffer print entry

000b	=	brkf		equ	11		;break key function

000f	=	openf	equ	15		;file open

0014	=	readf	equ	20		;read function

005c =	fcb		equ	base+5ch	;file control block address

0080	=	buff		equ	base+80h	;input disk buffer address

;non graphic characters

000d	=	cr		equ	Odh		;carriage return

000a	=	lf		equ	Oah		;line feed

;file control block definitions

005c	=	fcbdn	equ	fcb+O	;disk name

005d	=	fcbfn	equ	fcb+l	;pfile name

0065	=	fcbft	equ	fcb+9	;disk file type (3 characters)

0068	=	fcbrl	equ	fcb+12	;file's current reel number

006b =	fcbrc	equ	fcb+15	;file's record count (0 to 128

007c =	fcbcr	equ	fcb+32	;current (next) record number

MP/M II Programmer's Guide	Appendix E		Sample PRL

007d =	fcbln		equ	fcb+33	;fcb length

;		set up stack

0100 210000			lxi	h,O

0103 39			dad	sp

;		entry stack pointer in hl from the ccp

- 0104 22lfO2			shld	oldsp

;		set sp to local stack area (restored at finis)

- 0107 316102			lxi	sp,stktop

;		read and print successive buffers

010a cdc601			call	setup	;set up input file

OlOd feff			cpi	255		;255 if file not present

010f c2lbOl			jnz	openok	;skip if open is ok

;		file not there, give error message and return

0112 llfdOl			lxi	d,opnmsg

0115 cdalOl			call	err

0118 c35601			jmp	finis	;to return

openok:	;open operation ok, set buffer index to end

Ollb 3e8O			mvi	a,80h

Olld 32ldO2			sta	ibp		;set buffer pointer to 80h

;		hl contains next address to print

0120 210000			lxi	h,O		;start with 0000

gloop:

0123	e5			push	h		;save line position

0124	cda701		call	gnb

0127	el			POP	h		;recall line position

0128	da5601		jc	finis	;carry set by gnb if end file

012b	47			mov	b,a

;		print hex values

;		check for line fold

012c 7d			mov	a,l

012d e60f			ani	Ofh		;check low 4 bits

012f c24401			jnz	nonum

;		print line number

- 0132 cd7701			call	crlf

;		check for break key

- 0135 cd5eOl			call	break

;		accum lsb = 1 if character ready

0138	Of			rrc			;into carry

0139	da5lOl		jc	purge	;don't print any more

013c	7c			mov	a,h

013d	cd9401		call	phex

0140	7d			mov	a,l

0141	cd9401		call	phex

nonum:

0144 23			inx	h		;to next line number

0145	3e2O			mvi	a,’ ‘

0147	cd6aOl		call	pchar

014a	78			mov	a,b

014b	cd9401		call	phex

MP/M II Programmer's Guide	Appendix E		Sample PRL

014e	c32301		jmp	gloop

purge:

0151 OeOl			mvi	c,cons

0153 cd05OO			call	bdos

finis:

- ;		end of dump, return to cap

;		(note that a jmp to 0000h reboots)

0156 cd7701			call	crlf

0159 2alfO2			lhld	oldsp

015C f9			sphl

;		stack pointer contains cap's stack location

015d c9			ret		;to the ccp

suDroutines

break:	;check break key (actually any key will do)

015e e5d5c5			push h! push d! push b; environment saved

0161 OeOb			mvi	c,brkf

0163 cd05OO			call	bdos

0166 cldlel			pop b! pop d! pop h; environment restored

0169 c9			ret

pchar:	;print a character

016a e5d5cS			push h! push d! push b; saved

016d OeO2			mvi	c,typef

016f 5f			mov	e,a

0170 cd05OO			call	bdos

0173 cldlel			pop b! pop d! pop h; restored

0176 c9			ret

crlf:

0177	3eOd			mvi	a,cr

0179	cd6aOl		call	pchar

017c	3eOa			mvi	a,lf

017e	cd6aOl		call	pchar

0181	c9			ret

pnib:	;print nibble in reg a

0182 e60f			ani		Ofh	;low 4 bits

0184 fe0a			cpi		10

0186 d28eOl			jnc		plo

;		less than or equal to 9

0189 c630			adi		‘0’

018b c39001			jmp		prn

;		greater	or equal to 10

018e c637			pl0:	adi	‘a’ - 10

0190 cd6aOl	prn:		call	pchar

0193 c9			ret

MP/M II Programmer's Guide	Appendix E		Sample PRL

phex:	;print hex char in reg a

0194	f5			push	psw

0195	Of			rrc

0196	Of			rrc

0197	Of			rrc

0198	Of			rrc

0199	cd8201		call	pnib			;print nibble

019C	fl			POP	psw

019d	cd8201		call	pnib

OlaO	c9			ret

Err:		;print error message

;		d,e addresses message ending with

Olal OeO9			mvi	c,printf	;print buffer function

Ola3 cd05OO			call	bdos

Ola6 c9			ret

gnb:		;get next byte

Ola7 3aldO2			lda	ibp

Olaa fe8O			cpi	80h

Olac c2b801			jnz	go

;read another buffer

Olaf cdd30l			call	diskr

Olb2 b7			ora	a		;zero value if read ok

Olb3 cab.801		jz	go		;for another byte

;		end of data, return with carry set for eof

Olb6 37			stc

Olb7 c9			ret

go:		;read the byte at buff+reg a

Olb8 5f			mov	e,a		;ls byte of buffer index

Olb9 1600			mvi	d,O		;double precision index to de

Olbb 3c			inr	a		; i nd ex= i nd ex+l

Olbc 32ldO2			sta	ibp		;back to memory

;		pointer	is incremented

;		save the current file address

Olbf 218000			lxi	h,buff

O.lc2 19			dad	d

;		absolute character address is in hl

Olc3 7e			mov	a,m

;		byte is	in the accumulator

Olc4 b7			ora	a		;reset carry bit

Olc5 c9			ret

setup:	;set up file

;		open the file for input

Olc6	af			xra	a			;zero to accum

Olc7	327cOO		sta	fcbcr		;clear current record

Olca	115cOO		Ixi	d,fcb

Olcd	OeOf			mvi	c,openf

Olcf	cd05OO		call	bdos

;		255 in accum if open error

MP/M II Programmer's Guide	Appendix E		Sample PRL

Old2 c9			ret

diskr:	;read disk file record

Old3 e5d5c5			push hl push d! push b

Old6 115cOO			lxi	d,fcb

Old9 Oe14			mvi	c,readf

Oldb cd05OO			call	bdos

Olde cldlel			pop b! pop d! pop h

Olel c9			ret

;		fixed message area

signon:

Ole2 46696c6520		db	'file dump mp/m version 1.0$’

opnmsg:

Olfd OdOa4e6f2O		db	cr,lf,'no input file present on disk$'

;		variable area

021d		ibp:		ds	2	;input buffer pointer

021f		oldsp:	ds	2	;entry sp value from ccp

;		stack area

- 0221				ds	64	reserve 32 level stack

stktop:

0261				end

Appendix F

Sample Resident System Process

*********************************************

*  Note:                                    *

*  This program listing has been            *

*  included only as a sample and may not    *

*  reflect changes required by later MP/M   *

*  releases. For this reason the reader     *

*  should assemble and list the program     *

*  as provided on the distribution disk.    *

*********************************************

page	0

title	'type file on console'

;file type program, reads an input file and pri

;it on the console

;copyright (c) 1979, 1980

;digital research

;p.o. box 579

;pacific grove, ca 93950

0000					org	0000h		;standard rsp start

001a		=	ctlz		equ	lah			; control-z used for e

0002		=	conout	equ	2			; bdos conout function

0009		=	printf	equ	9			; print buffer

0014		=	readf	equ	20			; read next record

000f		=	openf	equ	15			; open fcb

0098		=	parsefn	equ	152  		; parse file name

0086		=	mkque	equ	134  		; make queue

0089		=	rdque	equ	137  		; read queue

0091		=	stprior	equ	145  		; set priority

0093		=	detach	equ	147  		; detach console

; bdos entry point address

bdosadr:

0000 0000				dw	$-$	ldr will fill this i

; type process descriptor

typepd:

0002 0000				dw	0			;link

0004 00				db	0			;status

0005 Oa				db	10			;priority (initial)

0006 1001				dw	stack+38		;stack pointer

0008 5459504520		db	'type    ‘	;name in upper case

pdconsole:

MP/M II Programmer's Guide	Appendix F		Sample RSP

0010				ds	1			;console

0011 ff			db	Offh			;memseg

0012				ds	2			;b

0014				ds	2			;thread

0016 3601			dw	buff			;disk set dma address

0018				ds	1			;user code & disk sel

0019				ds	2			;dcnt

001b				ds	1			;searchl

001C				ds	2			;searcha

00le				ds	2			;active drives

0020				ds	20			;register save area

0034				ds	2			;scratch

; type linked queue control block

typelqcb:

0036 0000			dw	0			;link

0038 5459504520		db	'type    ‘	;name in upper case

0040 4800			dw	72			;msglen

0042 0100			dw	I			;nmbmsgs

0044				ds	2			;dqph

0046				ds	2			;nqph

0048				ds	2			;mh

004a				ds	2			;mt

004c				ds	2			;bh

004e				ds	74			;buf(72 + 2 byte lin

; type user queue control block

typeuserqcb:

0098 3600			dw	typelqcb		; pointer

009a 9cOO			dw	field		; msgadr

;field for message read from type linked qcb

field:

009C				ds	1			;disk select

console:

- 009d				ds	1			;console

filename:

009e				ds	72			;message body

; parse file name control block

pcb:

OOe6 9eOO			dw				;filename	file name address

OOe8 1201			dw	fcb			;file control block a

;type stack & other local data structures

stack:

MP/M II Programmer's Guide	Appendix F		Sample RSP

00ea				ds	38			;20 level stack

0110 		ba0:		dw	type			;process entry point

0112		fcb:		ds	36			;file control block

0136		buff:		ds	128			;file buffer

;bdos call prdeedure

bdos:

Olb6 2aOOOO			lhld	bdosadr		;hl = bdos address

Olb9 e9			pchl

;type main program

type:

Olba	Oe86			mvi	c,mkque

Olbc	113600		lxi	d,typelqcb

Olbf	cdb60l		call	bdos			; make typelqcb

Olc2	Oe9l			mvi	c,stprior

Olc4	llc800		lxi	d,200

Olc7	cdb601		call	bdos			; set priority to 200

forever:

Olca	Oe89			mvi	c,rdque

Olcc	119800		lxi	d,typeuserqcb

Olcf	cdb60l		call	bdos			; read from type queue

Old2	0698			mvi	c,parsefn

Old4	lle600		lxi	d,pcb

Old7	cdb601		call	bdos			; parse the file name

Olda	23			inx	h

Oldb	7c			mov	a,h

Oldc	b5			ora	1			; test for Offffh

Oldd	calf02		jz	error

OleO	3a9dOO		Ida	console

Ole3	321000		sta	pdconsole		; typepd.console = con

Ole6	OeOf			mvi	c,openf

Ole8	111201		Ixi	d,fcb

Oleb	cdb601		call	bdos			; open file

Olee	3c			inr	a			;test return code

Olef	calf02		jz	error		;if it was Offh, no f

Olf2	af			xra	a			;else,

Olf3	323201		sta	fcb+32		;set next record to

new$sector:

Olf6	Oe14			mvi	c,readf

Olf8	111201		lXi	d,fcb

Olfb	cdb60l		call	bdos			;read next record

Olfe	b7			ora	a

Olff	c22702		jnz	done			;exit if eof or error

0202	213601		Ixi	h,buff		;point to data sector

# MP/M II Programmer's Guide	Appendix F		Sample RSP

0205	Oe8O			mvi	c,128		;get byte count

next$byte:

0207	7e			mov	a,m			;get the byte

0208	5f			mov	e,a			;save in e

0209	fela			cpi	ctlz

020b	ca2702		jz				;done	exit if eof

020e	c5			push	b			;save byte counter

020f	e5			push	h			;save address registe

0210	OeO2			mvi	c,conout

0212	cdb60l		call	bdos			;write console

0215	el			POP	h			;restore pointer

0216	cl			POP	b			;and counter

0217	23			inx	h			;bump pointer

0218	Od			dcr	c			;dcr byte counter

0219 c207O2			jnz	next$byte		; more in this sector

021c c3f601			jmp	new$sector	;else, we need a new

error:

021f 112fO2			lxi	d,err$msg		;point to error messa

0222 OeO9			mvi	c,printf		; get function code to

0224 cdb60l			call	bdos

done:

0227 Oe93			mvi	c,detach

0229 cdb60l			call	bdos			;detach the console

022c c3ca0l			jmp	forever

err$msg:

022f OdOa46696c		db	Odh,Oah,'file not found or bad file na

0251				end

Appendix G

Acronyms and Conventions

Throughout this manual, the following conventions are used in

describing the physical modules of MP/M II, its functional parts,

and data structures:

I		PHYSICAL MODULES

BDOS		- Basic Disk Operating System

XDOS		- Extended Disk Operating System

XIOS		- Extended I/O System

II		FUNCTIONAL PARTS

CLI		- Command Line Interpreter

TMP		- Terminal Message Processor

III		DATA STRUCTURES

FCB		-	File Control Block

MD		-	Memory Descriptor

PD		-	Process Descripotr

QCB		-	Queue Control Block

UQCB		-	User Queue Control Block

XFCB		-	Extended File Control Block

IV		NAMING CONVENTIONS

Filenames:

PRL		-	Page Relocatable File

SPR		-	System Page Relocatable

RSP		-	Resident System Process (or Procedure)

BRS		-	Banked RSP

System entry point  - First letter(s) are capitals and function

numbers are in parentheses

Appendix H

Glossary

BCD:	See binary coded decimal.

binary coded decimal: Representation of decimal numbers using

binary digits. See Appendix I for binary representations of ASCII

codes.

block: Basic unit of disk space allocation under MP/M II. Each

disk drive has a fixed block size defined in its Disk Parameter

Block in the XIOS. The block size can be lK, 2K, 4K, 8K or 16K

consecutive bytes. Blocks are numbered relative to zero so that

each block in a file is unique and has a byte displacement of the

Block Number times the Block Size.

boolean:  Variable that can only have two values; 	usually

interpreted as true/false, or on/off.

Checksum Vector: Contiguous data area in the XIOS with one byte for

each directory sector to be checked. A Checksum Vector is

initialized and maintained for each logged-in drive. Each directory

access by the system results in a checksum calculation which is

compared with that in the Checksum Vector. If there is a

discrepancy, the drive is set to read-only status. This prevents

the user from inadvertently switching disks without logging-in the

new disk. If not logged-in, the new disk is treated the same as the

old one and data on it may be destroyed by writing to it.

COM: Filetype for MP/M II command files. These are machine

language object modules ready to be loaded and executed. Any file

with this type may be executed by simply typing the filename after

the drive prompt (e.g.OA>) . For example, the program PIP.COM can be

executed by simply typing PIP.

command: Set of instructions that are executed when the command

name is typed after the system prompt. These instructions can be

"built-in" to the MP/M II system or can reside on disk as a file of

type COM, or PRL. In general, MP/M II commands consist of three

parts: the command name, the command tail, and a carriage return

CSV: See checksum vector.

default buffer: 128-byte buffer maintained at 0080H in Page Zero.

When the CLI loads a COM file,the CLI initializes this buffer to the

command tail, i.e. any characters typed after the COM filename. The

first byte at 0080H contains the length of the command tail while

the command tail itself begins at 0081H. A binary zero terminates

the command tail. The I command under DDT initializes this buffer

in the same way as the CLI does.

MP/M II Programmer's Guide	Appendix H 		Glossary

default FCB: One of two FCBs maintained at 005CH and 006CH

respectively, in Page Zero. The CLI initializes the first default

FCB from the first delimited field in the command tail and

initializes the second default FCB from the next field in the

command tail.

delimiters: ASCII characters that separate constituent parts of a

file specification. The CLI  recognizes certain delimiter characters

as :.=;<> , blank and carriage return. Several MP/M II commands

also treat []()$ as delimiter characters. It is advisable to

avoid the use of delimiter characters and lower-case characters in

filenames.

directory: Portion of a disk containing entries for each file on

the disk and locations of the blocks allocated to the files. Each

file directory element is in the form of a 32-byte FCB, although one

file can have several elements depending on its size. The maximum

number of directory elements supported is specified in the drive's

Disk Parameter Block.

directory element: 32-byte element associated with each disk file.

A file can have more than one directory element associated with it.

There are four directory elements per directory sector. Directory

elements can also be refereed to as directory FCBs.

directory entry: File entry displayed when using the DIR command.

This term also refers to a physical directory element (FCB).

Disk Parameter Block: Table residing in the XIOS that defines the

characteristics of a drive in the disk subsystem used with MP/M II.

The address of the DPB is in the Disk Parameter Header at DPbase +

OAH. Drives with the same characteristics can use the same Disk

Parameter Header, and thus the same DPB. However, drives with

different characteristics must each have their own Disk Parameter

Header and DPB. The address of the drive's Disk Parameter Header

must be returned in registers HL when the BDOS calls the SELDSK

entry point in the BIOS. BDOS Function 31 returns the DPB address.

Disk Parameter Header: 16-byte area in the XIOS that contains

information about the disk drive and a scratchpad area for certain

BDOS operations. Given n disk drives, the Disk Parameter Headers

are arranged in a table with the first row of 16 bytes corresponding

to drive 0, and the last row corresponding to drive n-l.

DPB:	See Disk Parameter Block.

DPH: See Disk Parameter Header.

EX: See extent.

extent: 16K consecutive bytes in a file. Extents are numbered from

0 to 31. One extent can contain 1, 2, 4, 8 or 16 blocks. EX is the

extent number field of an FCB and is a one byte field at FCB + 12,

where FCB labels the first byte in the FCB. Depending on the Block

Size and the maximum data Block Number, an FCB can contain 1, 2, 4,

MP/M II Programmer's Guide	Appendix H 		Glossary

8 or 16 extents. The EX field is normally set to 0 by the user but

contains the current extent number during file I/O. The term 'FCB

Folding' describes FCBs containing more than one extent. In CP/M

version 1.4, each FCB contains only one extent. Users attempting to

perform Random Record I/O and maintain CP/M 1.4 compatibility should

be aware of the implications of this difference.

FCB:	See File Control Block.

file: Collection of data containing from zero to 242,144 records.

Each record contains 128 bytes and can contain either binary or

ASCII data. ASCII data files consist of lines of data delineated by

carriage return line feed sequences, meaning that one 128-byte

record might contain one or more lines of text. Files consist of

one or more extents, with 128 records per extent. Each file has one

or more directory elements yet shows as only one directory entry

when using the DIR command.

File Control Block: Thirty-six consecutive bytes designated by the

user for file I/O functions. The FCB fields are explained in

Section 2.2.3. The term FCB also refers to a directory element in

the directory portion of the allocated disk space. These contain

the same first 32 bytes of the FCB explained in Section 2.1 lacking

only the Current Record and Random Record Number bytes.

HEX file format: Absolute output of ASM and MAC for the Intel 8080.

A HEX file contains a sequence of absolute records which gives a

load address and byte values to be stored starting at the load

address. (see Section 4).

I/0:	See Input/Output.

Input/Output: Operations or routines that handle the input and

output of data in the computer system.

logical drive: Logically distinct region of a physical drive. A

physical drive can be divided into one or more logical drives, and

designated with specific drive references (i.e., d:a or d:f , etc.) .

Thus at the user interface, it appears that there are several disks

in the system.

Page Zero: Memory region between OOOOH and 0100H that holds

critical system parameters and functions primarily as an interface

region between user programs and the BDOS module.

parse:  Separate a command line into its constituent parts.

physical drive: Peripheral hardware device used for mass storage of

data within the computer system.

read-only: Condition in which a drive can be read but not written

to. A drive can be set to read-only status by using the SET or STAT

utilities or the SET File Attributes function (BDOS Function 30) . A

drive can also be set to read-only status if the checksum computed

on a directory access does not match that stored in CSV when the

MP/m II Programmer's Guide	Appendix H Glossary

drive is logged-in. This protects the user from switching disks

without executing a disk reset.

record: Smallest unit of data in a disk file that can be read or

written. A record consists of 128 consecutive bytes whose byte

displacement in a file is the product of the Record Number times

128. A 128-byte record in a file occupies one 128-byte sector on

the disk. If the blocking and deblocking algorithm is used, several

records can occupy each disk sector.

reentrant code: Code that one process can use while another is

already executing it. The data for reentrant code is typically kept

on the stack.

R/O: See read-only.

sector: Basic unit of data read and written on the disk by the

XIOS. A sector can be one 128-byte record in a file or a sector of

the directory. In some disk subsystems, the disk sector size is

larger than 128 bytes, usually a power of two such as 256, 512, 1024

or 2048 bytes. These disk sectors are referred to as Host Sectors.

When the Host Sector size is larger than 128 bytes, Host Sectors

must be buffered in memory, and the 128-byte sectors must be blocked

and deblocked from them.

spooling: Printing a file from disk. The SPOOL program, which is

detached from a console, can print a file from a disk. This leaves

your console free for other tasks while your file is being printed.

stack: Reserved area of memory where the processor saves the return

address when it receives a Call instruction. When the processor

encounters a Return instruction, it restores the current address on

the stack to the Program Counter. Data such as the contents of the

registers can also be saved on the stack. The Push instruction

places data on the stack and the Pop instruction removes it. 8080

stacks are 16 bits wide; instructions operating on the stack add

and remove stack items one word at a time. An item is pushed onto

the stack by decrementing the stack pointer (SP) by two and writing

the item at the SP address. In other words, the stack grows

downward in memory.

track: Concentric ring on the disk; the standard IBM single

density diskettes have 77 tracks. Each track consists of a fixed

number of numbered sectors. Tracks are numbered from 0 to one less

than the number of tracks on the disk. Data on the disk media is

accessed by combinations of track and sector numbers.

user:		Logically distinct subdivision of the directory. Each

directory can be divided into 16 user areas.

vector: Memory location; an entry point into the operating system

used for making system calls or interrupt handling.

wildcard character: Either ? or * characters. The BDOS directory

search functions match ? with any single character and * with

multiple characters.

Appendix I

ASCII and Hexadecimal Conversions

This appendix contains tables of the ASCII symbols, including

their binary, decimal, and hexadecimal conversions.

Table I-1. ASCII Symbols

| Symbol | Meaning | Symbol | Meaning |
| --- | --- | --- | --- |
| ACK acknowledge FS file separator BEL bell GS group separator BS backspace HT horizontal tab CAN cancel LF line feed CR carriage return NAK negative acknowledge DC device control NUL null DEL delete RS record separator DLE data link escape SI shift in EM end of medium SO shift out ENQ enquiry SOH start of heading EOT end of transmission SP space ESC escape STX start of text ETB end of transmission SUB substitute ETX end of text SYN synchronous idle FF form feed US unit separator VT vertical tabulation |  |  |  |

MP/M II Programmer's Guide	Appendix I		ASCII/Hex Conversions

Table 1-2. ASCII Conversion Table

| Binary | Decimal | Hexadecimal | ASCII |
| --- | --- | --- | --- |
| 0000000 000 00 NUL 0000001 001 01 SOH (CTRL-A) 0000010 002 02 STX (CTRL-B) 0000011 003 03 ETX (CTRL-C) 0000100 004 04 EOT (CTRL-D) 0000101 005 05 ENQ (CTRL-E) 0000110 006 06 ACK (CTRL-F) 0000111 007 07 BEL (CTRL-G) 0001000 008 08 BS (CTRL-H) 0001001 009 09 HT (CTRL-I) 0001010 010 OA LF (CTRL-J) 0001011 Oil OB VT (CTRL-K) 0001100 012 0C FF (CTRL-L) 0001101 013 OD CR (CTRL-M) 0001110 014 OE SO (CTRL-N) 0001111 015 OF SI (CTRL-0) 0010000 016 10 DLE (CTRL-P) 0010001 017 11 DCI (CTRL-Q) 0010010 018 12 DC2 (CTRL-R) 0010011 019 13 DC3 (CTRL-S) 0010100 020 14 DC4 (CTRL-T) 0010101 021 15 NAK (CTRL-U) 0010110 022 16 SYN (CTRL-V) 0010111 023 17 ETB (CTRL-W) 0011000 024 18 CAN (CTRL-X) 0011001 025 19 EM (CTRL-Y) 0011010 026 IA SUB (CTRL-Z) 0011011 027 1B ESC (CTRL-[) 0011100 028 IC FS (CTRL-\) 0011101 029 1D GS (CTRL-]) 0011110 030 1E RS (CTRL-^) 0011111 031 IF US (CTRL-_) 0100000 032 20 (SPACE) oiO0001 033 21 ! 0100010 034 22 “ 0100011 035 23 # 0100100 036 24 $ 0100101 037 25 % 0100110 038 26 & 0100111 039 27 ‘ 0101000 040 28 ( 0101001 041 29 ) 0101010 042 2A * 0101011 043 2B + 0101100 044 2C , 0i0iIOl 045 2D - 0101110 046 2E . 0101111 047 2F / 0110000 048 30 0 0110001 049 31 1 0110010 050 32 2 |  |  |  |

MP/M II Programmer's Guide		Appendix I	ASCII/Hex Conversions

Table 1-2. (continued)

| Binary | Decimal | Hexadecimal | ASCII |
| --- | --- | --- | --- |
| 0110011 051 33 3 0110100 052 34 4 0110101 053 35 5 0110110 054 36 6 0110111 055 37 7 0111000 056 38 8 0111001 057 39 9 0111010 058 3A : 0111011 059 3B ; 0111100 060 3C < 0111101 061 3D = 0111110 062 3E > 0111111 063 3F ? 1000000 064 40 @ 1000001 065 41 A 1000010 066 42 B 1000011 067 43 C 1000100 068 44 D 1000101 069 45 E 1000110 070 46 F 1000111 071 47 G 1001000 072 48 H 1001001 073 49 1 1001010 074 4A i 1001011 075 4B K 1001100 076 4C L 1001101 077 4D M 1001110 078 4E N 1001111 079 4F 0 1010000 080 50 P 1010001 081 51 Q 1010010 082 52 R 1010011 083 53 S 1010100 084 54 T 1010101 085 55 U 1010110 086 56 V 1010111 087 57 w 1011000 088 58 X 1011001 089 59 y 1011010 090 5A z 1011011 091 5B [ 1011100 092 5C 1011101 093 5D ] 1011110 094 5E ^ 1011111 095 5F < 1100000 096 60 ‘ 1100001 097 61 a 1100010 098 62 b 1100011 099 63 c 1100100 100 64 d |  |  |  |

MP/M II Programmer's Guide		Appendix I	ASCII/Hex Conversions

| Binary | Decimal | Hexadecimal | ASCII |
| --- | --- | --- | --- |
| 1100101 101 65 e 1100110 102 66 f 1100111 103 67 g 1101000 104 68 h 1101001 105 69 i 1101010 106 6A j 1101011 107 6B k 1101100 108 6C l 1i0iIOl 109 6D m 1101110 110 6E n 1101111 111 6F o 1110000 112 70 p 1110001 113 71 q 1110010 114 72 r 1110011 115 73 s 1110100 116 74 t 1110101 117 75 u 1110110 118 76 v 1110111 119 77 w 1111000 120 78 x 1111001 121 79 y 1111010 122 7A z 1111011 123 7B { 1111100 124 7C ¦ 1111101 125 7D } 1111110 126 7E ~ 1111111 127 7F DEL |  |  |  |

Index

A						command Line Interpreter CLI,

3, 15, 101, 104

Abort Specified Process, 145		Compute File Size, 91

Absolute Memory Request, 126		compute-bound process, 5, 8

Access date and time stamp, 67	Conditional Attach Console,

Access Drive, 45, 93			148

allocation vector, 83			Conditional Attach List, 147

ambiguous file reference, 33,		Conditional Read Queue, 131

70					Conditional Write Queue, 132

Archive Attribute, 32			console, 12, 13, 23

ASM, 151, 187				Console Input, 57

Assembler Directives, 159		console management, 8

assembler parameters, 151		Console Output, 58

assembly language statements,		constants, 154

153					control characters, 24

Attach Console, 23, 58, 135		CP/M programs, 1, 12, 18

Attach List, 23, 146			Create Process, 134

attribute bits, 32			Creation date and time stamp,

79

B						current user number, 15, 24

Bad Sector error, 48			D

bank-switched memory, 2, 9, 11

banked RSP, 13, 19, 192			data area, 24

BASE, 188					data block size, 28

Base Page Areas, 53			date and time, 109

base page fields, 56			date stamp, 37

base page initialization, 17		deblocking, 44

Basic Disk Operating System,		default drive, 14

24					default FCB, 55

BDOS file system, 26, 28		default mode, 102

bitmap, 18, 175, 187			Delay, 133

bit vector, 84				delay execution, 10

blocking and deblocking, 44		Delete File, 72

breakpoint, 177				Delete mode, 36

BRS, 192					Delete Queue, 130

burst mode, 43				delimiters, 27

Dequeue list, 6

C						Detach Console, 136

Detach List, 146

Call Resident System			direct console I/O, 60

Procedure, 139			Direct Memory Address, 82

case translation, 153			directory area, 24

Chain To Program, 104			Directory Codes, 49, 50, 51

checksum verification, 40		directory functions, 25

circular queue, 116			Directory Label, 25, 34, 35,

CLI, 34					   37, 70, 105, 106

Close File, 31, 37, 68			disk directory area, 28

command line characters, 56		disk parameter block, 45, 86

command line format, 15, 55		Disk System Reset, 44

Dispatch, 4, 8, 133

display address, 180		G

drive capacity, 28

drive related functions, 25	generation process, 40

drive reset operation, 45	GENHEX, 183

drive select code, 26, 27	GENSYS, 191

drive-related functions, 25	GET ADDR(ALLOC), 83

DUMP, 184				GET ADDR(DISK PARMS), 86

Get Console Number, 143

E					Get Console Status, 64

Get Date and Time, 109, 144

edit control characters, 63	Get Disk Free Space, 103

Enqueue list, 6			Get List Number, 149

Error Codes, 49, 50, 51, 69	GET READ/ONLY VECTOR, 84

Error Flag, 49, 51

error handling XDOS, 125	I

error messages, 48

error mode, 25, 47		initializing an FCB, 31

Expressions, 154			Intel hex format, 151

extended error codes, 51, 52,	Inter-process communication,

68				  2

extended errors, 47, 48, 114	Interface Attributes, 33, 39,

extended file, 43			  66, 67

Internal Data Segment, 124

F					internal date and time, 109

FCB, 107				K

FCB checksum, 40

FCB format, 33			key fields, 92

FCB length, 29

File Access, 42			L

file access functions, 25

file attributes, 32		Link-80, 187

File Control Block FCB, 29	Linked Queues, 118

File directory elements, 31	list address, 178

file format, 29			list device, 23

File ID, 31, 38, 42, 67, 79,	List Device Management, 8

97				List Output, 59

file naming conventions, 28	load address, 179

File R/O error, 48		lock list, 13, 31, 40, 42

file references, 24		Lock Record, 97

File Security, 40			locked mode, 38

file size, 28, 91			log-in operation, 44

file specification, 26		logical drive, 24, 28

file system, 26, 40, 42		logical interrupt system, 7

file type field, 24, 26

file types, 28			M

filename field, 24, 26

flag over-run, 7			macros, 152

Flag Set, 7, 129			Make File, 32, 36, 37, 39, 78

flag under-run, 7			Make Queue, 129

Flag Wait, 7, 128			Memory Descriptor, 121

Flush Buffers, 44, 104		Memory Free, 127

Free Drive, 45, 94		memory segment, 9, 17, 19

Free Drive call, 41		memory segment index, 121

Function 6 Entry Parameters,

60

memory structure, 11		Q

miscellaneous functions, 25

MP/M II system processes, 1	qualified reset, 45

MPMLDR, 44				Queue Naming Conventions, 121

multi-sector count, 25, 43, 73Queues, 6, 16, 20, 116

Multi-Sector I/O, 43

Mutual exclusion queues, 6,	R

121

R/O error, 48

N					radix, 154

random record number, 29

nibble, 51				Raw Console Input, 58

nucleus, 4				Raw Console Output, 59

Numeric Constants, 154		RDT arguments, 174

RDT commands, 174, 175

0					Read Console Buffer, 62

Read File, 37

open File, 32, 39, 66		Read File XFCB, 107

open file item, 67		Read mode, 36

open mode, 39			Read Queue, 131

open Queue, 130			Read Random, 87

operation codes, 164		Read Sequential, 73

operators, 157			Read/only attribute, 32

ORG statements, 188		read/only mode, 38, 42, 67

Ready List, 4, 124

P					record, 29

record buffer, 44

Page Relocatable Programs, 18,record locking, 40, 42

187				register A, 49

Parse Filename, 27, 140		register passing conventions,

password, 24, 110			  21

password field, 26, 56		register storage allocation,

Password protection, 36		  115

passwords, 36, 37			relocatable addresses, 178

performance, 3			Relocatable Debugging Tool

permanent drives, 45, 46	RDT, 173

physical error codes, 52, 66,	Relocatable Memory Request,

68, 71			  127

physical errors, 47		removeable drive, 45, 46

physical file size, 91		Rename File, 80

Poll, 128				Reserved Words, 155

polling, 8				Reset Disk System, 65

Print String, 62			Reset Drive, 44, 93

PRL File Format, 187		Resident System Procedure,

PRLCOM, 184				  20, 191

process, 23, 40, 41, 42		Resident System Processes, 2.

process anming conventions,	  13, 191

116				Return and Display mode, 102

Process Descriptor, 4, 23,	return codes, 49

111, 192			Return Current Disk, 81

Process Descriptor address, 23Return Directory Label Data,

process priority, 5, 113		106

process states, 4			Return Login Vector, 81

program load, 18			return modes, 102

return MP/M Version Number,

Return Process Descriptor, 145	time stamping, 25

Return Serial Number, 110		time stamps, 37

Return Version Number, 64		TOD, 37

RSP Code, 191				trace mode, 181

S						U

schedule execution, 10			Unlock Record, 42, 99

Search For First, 70			unlocked mode, 38, 42

Search For Next, 70, 71			Update date and time stamp, 69

Select Disk, 65				User 0, 34

Select error, 48				user directories, 33

Send CLI Command, 138			user number, 33, 34

sequential I/O processing, 43

SET BDOS Error Mode, 102		V

Set Console, 136

Set Date and Time, 109			virtual file size, 91

Set Default Password, 110

Set Directory Label, 35, 105		W

SET DMA Address, 82

Set Error Mode, 47			wait loop, 3, 8, 10

Set File Attributes, 32, 85		Write File, 36

Set List, 147				Write File XFCB, 108

Set Multi-Sector Count, 43,		Write mode, 36

51, 101				Write Protect Disk, 46, 83

Set Priority, 135				Write Queue, 132

Set Random Record, 92			Write Random function, 89

SET/GET USER CODE, 86			Write Random With Zero Fill,

shared access mode, 42			  95

Source files, 29				Write Sequential, 76

Sparse files, 29

SPR files, 193				X

SUBMIT, 14

System Attribute, 32			XFCB, 34, 108

system call user stacks, 12		XIOS, 44

system console, 2, 14

System Data Address, 143		8080 CPU Flags, 182

System Data Page, 122			8080 CPU Registers, 182

system drive, 17

system generation, 2, 19

System Page Relocatable

Files, 193

System Reset, 57

system stacks, 114

System timing, 2, 10

T

Terminal Message Process TMP,

3, 14

Terminate Process, 134

Test and Write, 42

Test and Write Record, 95

the default FCB, 55

time of day, 10

MP/M Operating System

Release 2.1

Release Notes

Copyright © 1982

Digital Research

P.O. Box 579

160 Central Avenue

Pacific Grove, CA 93950

(408) 649-3896

TWX 910 360 5001

All Rights Reserved

Dear MP/M II User:

Digital Research has developed the MP/M JIT.M. operating system

in response to numerous customer requests to add file sharing

capability to MP/M Release 1.1. The design of MP/M II is a

reflection of our goal to provide you with a state of the art

operating system that can be configured for a wide variety of

computer hardware.

This shipment contains the version 2.1 release of our MP/M II

operating system. We have been pleased with the response to MP/M II

Release 2.0 and hope to see comparable response to MP/M II Release

2.1 regarding design, possible extensions, and errors in

implementation. We hope to maintain the same level of confidence

that the computer industry has had in our CP/M 0 operating system.

On the basis of our experience and the experience of MP/M II

users, we estimate it requires less than a week to implement a

simple polled MP/M II on a computer that has a running version of

CP/M Release 2.2. Implementing a highly optimized MP/M II system

with full interrupts and bank switched memory can require several

weeks. Of course, the time to perform such a reconfiguration will

vary widely depending on the experience of the programmer and the

complexity of the hardware.

Note: Make sure you use the SET or STAT command to make the

USER.PRL file into a system file.

Contact the Digital Research Technical Support staff (408) 375

6262 if you experience difficulties reconfiguring MP/M II. By

sending in your registration card you can insure that we will mail

MP/M II application notes and patches that correct implementation

errors.

Sincerely,

TECHNICAL SUPPORT

MP/M II T.M. Operating System

Release 2.1

Extended File Locking

Addendum to the MP/M II Operating SZstem Programmer's Guide

Copyright © 1982 by Digital Research

MP/M II is a trademark of Digital Research.

Compiled January 1982

Extended file locking is a new facility implemented in release

2.1 of MP/M II T.M. . Extended file locking enables a process to

maintain a lock on a file even after the file is closed. This

facility allows a process to rename, set the attributes, or delete a

file without having to contend with the possibility of interference

from other processes after the file is closed. Also, a process can

reopen a file with an extended lock and continue normal file

processing. For example, a process can open a file, perform file

operations on the file, close the file, rename the file, reopen the

file under its new name, and proceed with file operations, without

ever losing the file's lock list item and control over the file.

Extended file locking is only available to files that are

opened in the default open mode (locked mode). To extend a file's

lock, set interface attribute FV when closing the file. This

attribute is only interrogated by the Close function when it is

closing a file permanently. Thus, interface attribute F51 must be

reset when the close call is made. Also, if a file has been opened

N times (more than once), this attribute is only interrogated when

the file is closed for the Nth time.

To maintain an extended file lock through a Rename File call or

a Set File Attributes call, set interface attribute F5' of the

referenced FCB when making the call. This attribute is only honored

for extended file locks, not normal locks. Setting attribute F5'

also maintains an extended file lock for the Delete File function,

but setting this attribute also changes the nature of the Delete

function to an XFCB-Only delete. If successful, all three of these

functions delete a file's extended lock item when with attribute F51

reset. On the other hand, if they return with an error code, the

extended lock item is not deleted.

A standard open call can be made to resume file operations on a

file with an extended lock. The open mode, however, is restricted

to the default locked mode. The following list illustrates uses of

extended locks.

1

MPM/II Release 2.1	Extended File Locking

-Open file EXLOCK.TST in locked mode.

-Perform file operations on the file EXLOCK.TST using the open

FCB.

-Close file EXLOCK.TST with interface attribute F6’ set to

retain the file's lock item.

-Use the Rename File function to change the name of the file to

EXLOCK.NEW with interface attribute F5’ set to retain the

file's extended lock item.

-Open the file EXLOCK.NEW in locked mode.

-Perform file operations on the file EXLOCK.NEW using the opened

FCB.

-Close file EXLOCK.NEW with interface attribute F6' set to

retain the file's lock item.

-Set the Read-Only attribute and release the file's lock item by

using the Set File Attributes function with interface attribute

F51 reset. At this point, the file EXLOCK.NEW becomes

available for access by another process.

MP/M II T.M. Operating System

Release 2.1

Compatibility Attributes

Addendum to	the MP/M II Operating System Programmer's Guide

Copyright © 1982 by Digital Research

CP/M is a registered trademark of Digital Research.

MP/M and MP/M II are trademarks of Digital Research.

Compiled February 1982

The MP/M II T.M. file system introduced some new restrictions

relating to file operations that were not present in MP/M T.M. 1. 1 or

CP/M© . For example, if a process opens a file in the default mode

(locked), MP/M II does not allow other processes on the system to

open, delete, or rename the file until the process opening the file

either closes the file or terminates. In addition, MP/M II does not

allow a process to perform file operations with an FCB that has not

been activated by a successful open or make operation, or with an

FCB that has been deactivated by a close operation. These

restrictions protect an MP/M II user from interference from other

users on his open files. To illustrate, it is this protection that

enables an MP/M II user to edit a file with the assurance that

another user cannot delete or modify his file during his edit

session.

The new restrictions added to MP/M II provide file security

when multiple users are running the system. The preceding example

describes restrictions required to prevent collisions on file

activity between independent processes. Another new MP/M II

restriction sets limits on how a process can modify open FCBs.

These limits are enforced by checksum verification of open FCBs and

protect the integrity of the MP/M II file system from corrupted

FCBs. Note that the new MP/M II restrictions are not intended to

protect a user from his own actions. Instead, they ensure that the

activity of one user does not adversely affect other users on the

system.

Generally, the new MP/M II file system restrictions create

little difficulty for new application development. In fact, they

enforce good programming practice. However, because of these new

restrictions, some CP/M and MP/M software written before MP/M II's

release does not run on MP/M II. In addition, multiple copies of

some software do not run because the default open mode for MP/M II

is a locked mode in which only one process can open a file.

To address these problems, Digital Research has added

compatibility attributes to MP/M II, Release 2.1. The compatibility

attributes are defined as attributes Fl' through F4’ of program

files. A new GENSYS option determines whether the attributes are to

be activated. If activated, the Command Line Interpreter (CLI)

interrogates these attributes during program loading and modifies

the MP/M II ground rules for the loaded program as described below.

1

MP/M II Release 2.1	Compatibility Attributes

Note that the compatibility attributes should not be used with new

software. They are intended for use with working software developed

for CP/M and MP/M 1.1. This especially applies to compatibility

attribute F4' , which disables FCB checksum verification on read and

write operations. Use this attribute sparingly and only with

programs that are known to work.

COMPATIBILITY ATTRIBUTE DEFINITIONS

F1’	MP/M 1.1 Default Open. Processes running with this attribute

have all files opened in locked mode marked as Read-Only in

the System Lock List. This allows all processes with this

attribute set to read and write to common files with no

restrictions. There is, however, no record locking provided.

In addition, this attribute also allows a process to write to

a file opened by another process in Read-Only mode. To be

safe, all static files such as program and help files should

be made Read-Only when this compatibility attribute is used.

F2’	Partial Close default. Processes running with this attribute

have their default close mode changed from permanent close to

partial close. This attribute is for programs that close a

file to update the directory but continue to use the file.

Note that MP/M II assumes a process has finished with a file

when the number of closes issued to the file equals the

number of opens. A side effect of this attribute is that

files opened by a process are not released until the process

terminates. It might be necessary to set the System Lock

List parameters to high values when using this attribute.

F3’	Ignore Close Checksum Errors. This attribute changes the way

Close Checksum errors are handled for a process. Usually, a

message is printed on the console, and the process is

terminated. When this attribute is set and a checksum error

is detected during a close operation, the file is closed if a

lock list item exists for the file. Otherwise, an

unsuccessful close error code is returned to the calling

process.

F4’	Disable FCB Checksum verification for read and write

operations. Setting this attribute also sets attributes F2'

and F3'. This attribute should be used carefully because it

effectively disables MP/M II's file security. Use this

attribute only with software that is known to work.

MP/M II Release 2.1	Compatibility Attributes

PROCEDURE FOR USING THE COMPATIBILITY ATTRIBUTES

1)	Answer yes to the GENSYS question "Enable Compatibility

Attributes (N) ?".

2)	Use the MP/M II Utility SET to set the desired combination

of computability attributes in the program name.

EXAMPLES:

OA>SET filespec [Fl=on]

OA>SET filespec [Fl=on,F3=on]

OA>SET filespec [F4=on]

If you have a program that runs under CP/M or MP/M 1.1 but does

not run properly under MP/M II, use the following guidelines to

select the compatibility attributes to set for the program.

1)	If the program ends with the message, "File Currently

Opened" when multiple copies of the program are run, set

compatibility attribute F1’. As an alternative, you might

consider placing all common static files under User 0 with

the SYS and R/O attributes set.

2)	If the program terminates with the message, "Close Checksum

Error", set compatibility attribute F3’.

3)	If the program terminates with an I/O error, try running the

program with attribute F2' set. If the problem still

occurs, try attributes F2' and F3’. If the problem still

persists, then try attribute F4’. Use attribute FV only

as a last resort.

It might be necessary to increase the GENSYS parameters that

set the maximum number of files a process can open and the size of

the System Lock List when using compatibility attributes F2’ and

F4’. This might be required because both default to partial closes.

As a result, system lock list entries consumed by opening files are

not released until the process ends. Generally, if a process ends

with the message "No Room in System Lock List" or "Open File Limit

Exceeded", it usually indicates that the above GENSYS parameters

need to be increased. Another option is to patch in a BDOS Free

Drive call at a point in the program where no files are active.

Note that a Free Drive call specifying all drives, purges all file

system lock entries belonging to the calling process.

3

MP/M II Release 2.1	Compatibility Attributes

When GENSYS activates compatibility attributes, the Command

Line Interpreter copies the settings for attributes Fl' through F4’

of the filename of the loaded program into byte lDH of the process

descriptor as shown below:

PROCESS DESCRIPTOR BYTE IDH

(Bits defined 7-0 high-order to low-order)

Bit 7 set = Fl

Bit 6 set = F2

Bit 5 set = F3

Bit 4 set = F4

MP/M II T.M. Operating System

Release 2.1

Programming Guidelines

Addendum to the MP/M - II Operating System Programmer's Guide

Copyright © 1982 by Digital Research

CP/M is a registered trademark of Digital Research.

MP/M and MP/M II are trademarks of Digital Research.

Compiled January 1982

This guideline provides additional discussion on the

information presented in the MP/M II TM Operating System Programmer’s

Guide. In particular, this (5-ocument emphasizes those areas of MP/M

II where restrictions exist that did not exist in versions 1 and 2

of CP/M (C)and version 1 of MP/M T.M. The intent is to enable the MP/M

II application programmer to avoid potential problems with new

software. As a prerequisite, the reader should be familiar with the

material presented in the MP/M II Operating System Programmer's

Guide .

1)	Always use the following sequence when performing file operations

that require an open file. Under MP/M II, these operations are

the BDOS read, write, lock, and unlock record commands.

Activate a file's FCB with a BDOS Open or Make function call

before using the FCB in a file operation. Verify that the Open

or Make operation was successful. MP/M II only accepts FCBs

activated by a successful Open or Make call for open file

operations. If an FCB that has not been activated is used,

MP/M II returns a checksum error.

Perform all file operations using activated FCBs. Note that

MP/M II does not deactivate an activated FCB when it returns

error codes for file operations. Generally, only the current

record and random record fields of an activated FCB should be

modified. In addition, all file operations with an activated

FCB must be made under the user number that was in effect when

the FCB was activated. A similar restriction applies to

activated FCBs that specify the default drive. All file

operations specifying such an FCB must be made under the

current drive that was in effect when the FCB was activated.

Item 3 in this list covers the complete rules regarding

activated FCB modification.

1

1

MP/M II Release 2.1	Programming Guidelines

o	If a process has completed file operations on a file but still

has a significant amount of processing left to do, the file

should be closed. This applies even if the file was not

modified. With some exceptions, the lock list entry associated

with a file in the system lock list is not released until a

file is permanently closed (MP/M II Operating System

Programmer's Guide see Section 2.2.9.) MP/M II restricts

access to a file by other processes while a lock list item for

the file resides in the system lock list. It is not necessary

to close input files if a process is about to end. At

termination, all lock items belonging to a process are

released. Output files, however, must always be closed or data

might be lost. Note that a successful permanent close

operation deactivates the FCB and removes the file's item from

the system lock list. If the deactivated FCB is used in a

subsequent open file operation, MP/M II returns a checksum

error.

2)	If a process opens the same file more than once, a matching

number of close commands must be issued to the file to remove the

file's lock list item from the system lock list. Thus, if a file

has been opened N times, the first N-1 close operations issued to

the file default to partial close operations. Only the last

close, close operation N, is interpreted as a permanent close.

By definition, a permanent close is a close operation that

removes the referenced file's item from the system lock list.

Note that only one lock list item is allocated in the system lock

list for a file regardless of the number of FCBs a process has

opened for the file.

3)	The following list specifies how an activated FCB can be changed

without affecting the FCB checksum. MP/M II returns a checksum

error code and does not perform the requested operation if an FCB

with an invalid checksum is used in an open file operation.

o	FCB(O) cannot specify a new drive.

o	With the exception of interface attributes FV and F6' for the

BDOS Close function, FCB(l) through FCB(11) cannot be changed.

o	The high-order 3 bits of FCB(12) cannot be changed. The low

order 5 bits can be changed. Note that when a file is opened

in the default open mode (locked mode), the high-order 3 bits

of this FCB field are set to zeros.

o	FCB(13) cannot be changed.

o	FCB(14) and FCB(15) can be changed.

o	FCB(16) through FCB(31) cannot be changed.

O	FCB(32) through FCB(35) can be changed.

2

MP/M II Release 2.1	Programming Guidelines

If compatibilty with future releases of MP/M and CP/M is a

requirement, programs should restrict open FCB modification to

the FCB fields 32 through 35. In particular, Digital Research

does not support techniques that involve modifying fields 12, 14,

and 15 of open FCBs.

4)	Processes that access a printer must issue a Detach List device

to free the printer before another process can use the printer.

If the Detach List call is not made, a process that accesses a

printer continues to own the printer until it ends.

5)	CP/M programs that create submit files for chaining must be

modified to work under MP/M II. MP/M II requires a different

filename for submit files, that includes the originating console

number, and requires that a submit flag be set in the System Data

Page. The technique for creating and executing submit files is

described in MP/M II Application Note 07. Note that MP/M II also

has a Program Chain (Function 47) command that provides an

efficient mechanism for program chaining.

6)	CP/M programs that make direct BIOS calls for disk I/O do not

work under MP/M II. MP/M II does support direct XIOS calls for

the console and printer, but not to the disk. If programs must

make direct XIOS disk calls, a technique strongly discouraged in

a multi-user environment, two levels of indirection must be used

to obtain the real XIOS jump table address. The second level of

indirection is required because an intercept table handles the

console and printer.

The following two steps should be performed in a program before

making direct XIOS calls to a disk. The f first step is to make a

BDOS Write Protect Disk (Function 28) call to the disk to ensure

that no other process has open files on the disk. Secondly, the

MXDisk mutual exclusion queue message should be read to prevent

other programs from making BDOS disk function calls while your

program is making direct XIOS calls. After completing your

direct XIOS calls, write back the MXDisk message and then reset

the drives you have set to Read-Only.

7)	The following procedure is a protocol that multiple processes can

use to coordinate record update and addition operations to a

shared file. Each process must open the shared file in unlocked

mode. This procedure also assumes that records containing binary

zeros are null records.

3

MP/M II Release 2.1	Programming Guidelines

O Attempt to lock the record.

O If the lock attempt fails because another process has locked

the record, delay and repeat the procedure.

o	If the lock attempt fails because the record does not exist in

the file, add a record initialized to binary zeros to the file

with the BDOS Write Random with Zero Fill command and repeat

the procedure. Note that files opened in unlocked mode are

extended in block units and not in record units as is the case

for files opened in the default locked mode.

O	If the lock attempt succeeds, read the record, update it, and

then unlock it.

8)	Multiple FCB I/O is a technique that involves opening each extent

for a file independently and maintaining them in a table in

memory. Then random I/O is handled by selecting the proper FCB

from the table, setting the current record field to the proper

record number within the extent, and making a sequential Read or

Write command. When processing is completed, each FCB is closed.

The maximum file size that can be accessed with this technique is

512K bytes. This limits the maximum table size to 32 FCBs. Note

that this technique provides a method of performing random I/O

that is compatible with CP/M 1.4.

Multiple FCB I/O has to be performed carefully under MP/M

II because of the restrictions MP/M II places on file operations

to provide file security. Generally, an FCB should not be used

in file I/O unless it has been activated and it should not be

modified while it is activated (see items 1 and 3). In addition,

the number of opens and closes issued to a file is important (see

item 2). Note that all 32 bytes of each extent's FCB should be

maintained in the open FCB table. Also, verify that interface

attribute F8' is set to 1 in all FCBs if the first FCB has F8’

set to 1. F8' set to 1 indicates the file was opened under user

0 although the current user number is nonzero (see Function 15 in

the MP/M II Operating System Programmer's Guide).

4

MP/M II T.M. Operating System Release 2.0

Application Note 01, 9/14/81

Copyright © 1981 by Digital Research

MP/M and MP/M II are trademarks of Digital Research.

Compiled September 1981

SUPPRESSING THE MP/M T.M.LOADER DISPLAY

Applicable products and version numbers: MP/M II  Release 2.0

Program: 	MPMLDR.COM

When the MP/M II loader reads the MPM.SYS file, it displays a

load map on console #0. In some applications you might want to

suppress this display.

To suppress the load map display on console #0, type the

following RET instruction into the LDRBIOS.ASM file using any

standard editor. The RET instruction replaces the console output

code.

;Loader BIOS jump vector:

. . .

jmp	conout

. . .

conout:

ret

Assemble LDRBIOS.ASM to create LDRBIOS.HEX. Integrate the new

LDRBIOS.HEX file into the MPMLDR.COM file according to instructions

provided in the MP/M II Operating System Guide. Then,

update the system tracks of the boot disk with the new loader.

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.

1

MP/M II T.M.  Operating System Release 2.0

Application Note 02, 9/14/81

Copyright © 1981 by Digital Research

MP/M II is a trademark of Digital Research.

SETTING AND RESETTING THE RAW CONSOLE I/O MODE

Applicable products and version numbers: MP/M II". Release 2.0

Some application programs require raw input from the console.

Raw input implies that the operating system takes no action on

special characters, such as CTRL-C.

Execute the following code to place an application program into

a raw console input mode.

MVI		C,9CH

CALL		XDOS	; get process descriptor address

LXI		D,6

DAD		D

MOV		A,M

ORI		80H	; turn 'on' the high-order bit of first

MOV		M,A	; character in the process name

Execute the following code to exit the raw console input mode.

MVI		C,9CH

CALL		XDOS	; get process descriptor address

LXI		D,6

DAD		D

MOV		A,M

ANI		7FH	; turn 'off' the high-order bit of first

MOV		M,A	; character in the process name

Functions 3, 4, and 6 place the system into raw console input

mode. All other console I/O functions reset the system to normal

console input mode.

Raw console input mode can cause problems. You cannot abort a

process running in raw mode because the system ignores all control

characters. To abort a process, use Function 11 before using any

disk I/O functions. Function 11 returns the system to normal

console mode.

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.

1

MP/M II T.M. Operating System Release 2.0

Application Note 03, 9/14/81

Copyright © 1981 by Digital Research

MP/M II is a trademark of Digital Research.

CHANGING PRL FILE MINIMUM BUFFER SIZE REQUIREMENTS

Applicable products and version numbers: MP/M T.M. Release 2.0

You might want to allocate a larger default buffer for a

program such as the editor. You can change the minimum buffer size

requirements for PRL files. The following procedure demonstrates

how to change the minimum buffer size requirements for ED from 4k to

8k bytes.

OA>ddt ed.prl

[MP/M] DDT VERS 1.1

NEXT PC

2300 0100

-sl04

0104 00 00

0105 10 20

0106 .

-v2300

0044

-ied.prl

-w44

-go

Bytes 4 and 5 of the PRL header record (relative to the base)

contain the low- and high-order bytes for the minimum buffer size

specification.

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.

MP/M	II T.M. Operating System Release 2.0

Application Note 04, 9/14/81

Copyright © 1981 by Digital Research

MP/M and MP/M II are trademarks of Digital Research.

ACCESSING THE INTERNAL MP/M TM TOD

Applicable products and version numbers: MP/M II Release 2.0

Some application programs might require access to the internal

MP/M II time and date fields to set initial values. Execute the

following code sequence at the end of your MP/MT.M. XIOS system

initialization procedure. Place the code at the end because the

XDOS call to obtain the system data page address might cause

interruptions.

MVI 	C,9AH

CALL XDOS		;obtain the system data page address

;*** warning ***

;the XDOS call enables interrupts

LXI	D,OOFCH

DAD	D		; hl	pointer -> TOD

MOV	E,M

INX	H

MOV	D,M		; de	TOD

The assembly language subroutine TODCNV.ASM distributed on the

MP/M II release disk converts from ASCII string representation of

the time and date to MP/M II internal time and date format.

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.

1

MP/M II T.M. Operating System Release 2.0

Application Note 05, 9/14/81

Copyright © 1981 by Digital Research

MP/M II is a trademark of Digital Research.

DMA DISK CONTROLLERS WITH BANKED MEMORY SYSTEMS

Applicable products and Version Numbers: MP/M II Release 2.0

Be extra careful with bank switched memory systems that have

Direct Memory Access disk controllers. Bank switching is not

allowed during a transfer of data from the disk controller to a

target bank.

DMA from the disk controller is obtained through common memory

then copied from common memory into the user buffer that you wanted.

Sectors larger than 128 bytes are placed in a common memory buffer.

The specified sector is then transferred to the target buffer. This

is a reasonable technique in systems where deblocking is required

Use the following procedure if DMA is to occur directly into

the user buffer bypassing common memory. Set a DMA active flag to

true before each DMA operation. Reset the flag following each

operation.

MVI	A,FFH

STA	DMACTVE

;		initiate DMA operation

;		perform flag wait or poll for operation complete

XRA A

STA DMACTVE

Place the following code sequence in the XIOS select memory

procedure to ensure that the bank cannot be switched during a DMA

operation:

SELMEMORY:

LDA	DMACTVE

ORA	A

jz	OKTOSWITCH	;jump if not in DMA operation

; Next, the bank to be switched can be

; compared with the current bank. If

; it matches, the DMA operation will not be affected.

jz	OKTOSWITCH	;no bank change required

1

MP/M II Release 2.0, Application Note 05, 9/14/81 (cont'd)

; A new bank is specified and a DMA operation is in

; progress. A busy wait must now be performed to wait

; until the DMA operation is complete.

*** warning ***

;The selmemory call is made from inside the dispatcher

;therefore interrupts are disabled and nothing must

;be done that could force a dispatch.

BUSYWAIT:

IN	DMASTATUSPORT	;This is a "BUSY-WAIT"

ANI	DMADONE

jz	BUSYWAIT		;loop until the DMA is complete

Place the following code into the remaining select memory procedure.

OKTOSWITCH:

. . .

. . .

RET

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.

MP/M II T.M. Operating System Release 2.0

Application Note 06, 9/14/81

Copyright © 1981 by Digital Research

MP/M II is a trademark of Digital Research.

USING THE SEND CLI COMMAND XDOS FUNCTION

Applicable products and version numbers:		MP/M II Release 2.0

Use of the Send CLI Command XDOS Function can effectively

implement a menu driven application program. The following steps

outline use of the SEND CLI XDOS Function.

1)	Change the priority of the calling process so that it is

higher (actually a lower value) than the TMP.

2)	Obtain the console number of the calling process.

3)	Assign the console to the Command Line Interpreter.

4)	Issue the send CLI command function call.

5)	Issue an ATTACH console function to get the console back

after the initiated process has terminated.

6)	Restore the priority of the calling process to its original

value (usually 200).

Segments of a menu driven program named MENU appear in the

following example.

;XDOS Function Equate Table

setpriority		equ	145

attachconsole	equ	146

assignconsole	equ	149

sendCLIcommand	equ	150

getconsole		equ	153

MENU:

mvi	e,190

mvi	c,setpriority

call	BDOS			;set priority to 190

mvi	c,getconsole

call	BDOS			;get console # in A reg

sta	AssignPB		;fill in

sta	CLIcommand+l	;console fields

lxi	d,AssignPB

mvi	c,assignconsole

1

NP/M 11 Release 2.0, Application Note 06, 9/14/81 (cont'd)

call	BDOS			;assign console to CLI

inr	a

jz	cannotassign	;assign failed

lxi	d,CLIcommand

mvi	c,sendCLIcommand

call	BDOS			;send CLI command

mvi	c,attachconsole

call	BDOS			;attach console

mvi	e,200

mvi	c,setpriority

call	BDOS			;set priority back to 200

AssignPB:

db	$-$			;console number

db	Icli	I		;name (cli is lower case)

db	0

CLIcommand:

db	0		;default disk / user code

db	$-$		;console number

db			this is an ASCII string terminated with a

;null that is exactly as you would run the

;program from the console. e.g.

'PIP LST:=MYPROG.LST[PT8]',O

. . .

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.

2

MP/M II T.M. Operating System Release 2.0

Application Note 07, 9/14/81

Copyright © 1981 by Digital Research

MP/M II is a trademark of Digital Research.

CREATING A SUBMIT FILE FROM AN APPLICATIONS PROGRAM

Applicable products and version numbers: MP/M II T.M. Release 2.0

The following procedure shows you how to create a submit file

from an applications program and force its execution. The procedure

to terminate a submit file job is included.

1)	Obtain the temporary file drive from the system data page

2)	Obtain the console number at which the program is executing.

3)	Create the $n$.SUB file. Use n to specify the console

number.

4)	Set the appropriate submit flag in the array to on. The

array is contained in the system data page.

;BDOS / XDOS Function Equate Table

closefile		equ	16

searchfirst		equ	17

deletefile		equ	19

makefile		equ	22

getconsole		equ	153

getsysdatadr	equ	154

subflgofst		equ	128

mvi		c,getsysdatadr

call		BDOS

lxi		d,196		;temp file drive offset

dad		d

mov		a,m

sta		FCB

mvi		c,getconsole

call		BDOS

sta		console

adi		lot

sta		FCB+2		;put console # in fname

lxi		d,FCB

mvi		c,searchfirst

call		BDOS		;see if file there

1

MP/M II Release 2.0, Application Note 07, 9/14/81		(cont'd)

inr	a

jz	nofile

lxi	d,FCB

mvi	c,deletefile

call	BDOS 		; delete old version first

nofile:

lxi	d,FCB

mvi	c,makefile

call	BDOS 		; make the $n$.SUB file

Now, write the records into the $n$.SUB file as

follows:

-one line of the submit file per record

-last record first (i.e. in reverse order

that they are to be executed

-each record in the following form:

[CNTI[ASCII command line][NULL]

where:	CNT = # chrs in cmnd ln, 1 byte

ASCII command line <= 125 chrs

NULL	zero, 1 byte

d,FCB

lx1

mvi	c,closefile

call	BDOS		;close the $n$.SUB; file

mvi	c,getsysdatadr

call	BDOS		;get system data page adr

lda	console	;retrieve the saved con #

adi	subflgofst	;add offset to base of

mov	e,a		;submit flag array

mvi	d,O

dad	d		;DE = submitflag(console);

mvi	m,Offh	;set 'on' submit flag

; terminate the program

FCB:

DB			;disk drive, usually A:

DB	‘$n$’ 	;filename

DB	'SUB' 	;filetype

DB	0		;file extent

DS	20		;remainder of FCB

console:

ds	1		;temp loc for console #

MP/H II Release 2.0, Application Note 07, 9/14/81 (cont'd)

Terminate the operation of a submit job by zeroing a submit

flag located in the SYSTEM DATA PAGE region of memory. To locate

and zero the submit flag for a console use the following code

procedure.

XDOS Function Equate Table

getconsole		equ	153

getsysdatadr	equ	154

subflgofst		equ	128

mvi		c,getconsole

call		BDOS		; get console #

push		psw		; save console #

mvi		c,getsysdatadr

call		BDOS		; get system data page adr

POP		psw		; restore console #

adi		subflgofst

mov		l,a		; hl = address of sub. flag

mvi		m,O		; zero submit flag

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.

3

MP/M II T.M. Operating System Release 2.0

Application Note 08, 9/14/81

Copyright © 1981 by Digital Research

MP/M and MP/M II are trademarks of Digital Research.

CP/M is a registered trademark of Digital Research.

Wordstar is a registered trademark of

MicroPro International Corporation.

FILE SHARING

Applicable products and version numbers: MP/M II T.M. Release 2.0

Multiple users can share files using the MP/M II file system.

An applications program such as Wordstar requires that files be

open while the program is running. Multiple users of the

application will need to share the open files. Usually under MP/M

II, sharing of files causes problems if the applications program is

not structured to open files in Read-Only mode. The default mode

for the open function is locked mode which prevents the sharing of

files. Files are opened in locked mode for earlier versions of both

CP/M 0 and MP/M T.M. as well.

To enable file sharing, place all files to be shared under USER

0 on the default disk. Using the SET utility, assign the attributes

SYS (System) and RO (Read-Only) to the files. The BDOS opens the

file in Read-Only mode regardless of which mode the open function

specified. An example is shown below.

OA>set wsmsgs.com [SYS,R0]

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.

1

MP/M II T.M. Operating System Release 2.0

Application Mote 09, 9/14/81

Copyright © 1981 by Digital Research

MP/M II is a trademark of Digital Research.

PROGRAM CONTROL OF THE CONTROL-P SWITCH

Applicable products and version numbers: MP/M II T.M. Release 2.0

An applications program might need to echo console I/0 to the

printer while under program control. Use the following procedures

to set and clear the CTRL-P flags. The array of flags is located at

the SYSTEM DATA PAGE address + 126.

Setting CTRL-P Flag

mvi		c,9ah		; Get System Data Page address

call		BDOS

lxi		d,126

dad		d		; add 126 to Sys. Data Page addr.

mov		e,m

inx		h

mov		d,m		; DE	addr. of CTRL-P array

push		d

mvi		c,Oa4h	; Get List Number

call		BDOS

mov		e,a

mvi		d,O

POP		h

dad		d

mvi		m,Offh	; set CTRL-P flag

; cons. I/O is echoed from now on

Clearing CTRL-P Flag

mvi		c,9ah		; Get System Data Page address

call		BDOS

lxi		d,126

dad		d		; add 126 to Sys. Data Page addr.

mov		e,m

inx		h

mov		d,m		; DE	addr. of CTRL-P array

push		d

mvi		c,Oa4h	; Get List Number

call		BDOS

mov		e,a

mvi		d,O

POP		h

dad		d

mvi		m,0		; reset CTRL-P flag

; console I/O echo is now off

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.

1

MP/M II T.M. Operating System Release 2.0

Application Mote 10, 9/14/81

Copyright © 1981 by Digital Research

MP/M II is a trademark of Digital Research.

COLD BOOT STARTUP

Applicable products and version numbers: MP/M II T.M. Release 2.0

MP/M Ii can execute one command upon cold boot. However, the

system can execute any number of commands upon cold boot if the

initial command is SUBMIT.

To execute the Startup command place the Startup command

singularly into a file using standard command format. Name this

file $n$.SUP where n is the console number that executes the

command. The $n$.SUP file resides on the system drive at the

desired USER number or at USER 0 with a SYS (SYSTEM) attribute.

Examples are shown below.

Startup file: 	$0$.SUP

Command in the

Startup file:	SUBMIT START$O$

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.

MP/M II T.M. Operating System Release 2.0

Application Note 11, 9/14/81

Copyright © 1981 by Digital Research

MP/M II is a trademark of Digital Research.

SUBMIT ENHANCEMENTS

Applicable products and version numbers: MP/M II T.M. Release 2.0

Enhancements to SUBMIT include the following new features and

facilities.

INCREASED $n$.SUB FILE SIZE: SUBMIT file size is now unlimited.

The $n$.SUB file originally was limited to one extent, 128 lines.

CHANGING THE USER NUMBER: To change the current USER number in

SUBMIT, include the USER command in the SUBMIT file.

INCLUDE FILES: An include file is a standard SUBMIT file subject to

all SUBMIT rules and features. Format for the INCLUDE command is

demonstrated below.

$INCLUDE filename parml parm2 parm3 ...

The filename in the INCLUDE command must have the filetype SUB to

indicate a SUBMIT file and parameters are standard SUBMIT

parameters. An INCLUDE file can nest up to four SUBMITs in a SUBMIT

command.

EMBEDDED CONTROL CHARACTERS: Control characters can be embedded in

a SUBMIT file by preceding the capitalized character with an ASCII

up arrow ^. For example, type ~X to embed a CTRL-X. Embedded

control characters are not interpreted by MP/M II, but can be of use

to programs that SUBMIT executes.

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.

I

MP/M II T.M. Operating System  Release 2.0

Application Note 12, 9/14/81

Copyright © 1981 by Digital Research

MP/M II is a trademark of Digital Research.

SPOOL UTILITY MODIFICATIONS

Applicable products and version numbers: MP/M II Release 2.0

SPOOL can return an error message if the file to be spooled is

not found. To utilize this modification, SPOOL is divided into a

transient portion (SPOOL.PRL) and a resident portion. The transient

portion parses the command tail, opens the file, passes the file to

the spool queue (named SPOOLQ) , and displays an error message if the

open sequence on the file fails. Then, the transient portion ends

itself.

Issue a SPOOLQ command if you do not want to use a memory

segment to spool a file. Error messages are not returned, however.

Sample commands to spool a file are shown below.

SPOOL filel.typ,file2.typ .,.

The SPOOL process passes the command tail, checks for errors,

and sends the file to the spool queue (SPOOLQ).

SPOOLQ filel.typ,file2.typ ...

The command tail is sent to the spool queue (SPOOLQ) bypassing

error checking or error reporting.

The SPOOL utility sets its priority to 201. Most processes

execute ahead of the SPOOLER. To change the SPOOLER priority, the

SPOOL.BRS file is modified. Make sure you have a back-up copy of

SPOOL.BRS before using RDT to make the following changes.

A>rdt spool.brs

-s3b5

03B5 C9 c8

03B6 00 .

-ispool.brs

-w14

-go

A>gensys

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.

1

MP/M II T.M. Operating System Release 2.0

Application Note 13, 9/14/81

Copyright © 1981 by Digital Research

MP/M II is a trademark of Digital Research.

RECORD LOCKING/UNLOCKING

Applicable products and version numbers: MP/M II Release 2.0

Record locking/unlocking allows multiple processes to share

access of one file. Files are opened in the UNLOCKED mode. A

record locked by one process can only be read by a different

process, however, a locked record can be modified by the initial

process. Avoid reading locked records to prevent reading data that

is being updated. To avoid reading locked records let the process

try to lock the record. If the attempt fails, do not read the

record. The following code segment demonstrates how to lock

records.

mvi		c,2ch	;set multi-sector cnt.

mvi		e,#	;# = num. of sectors

call		bdos	;1<= # <=16

mvi		c,2ah	;lock record

lxi		d,fcb	;record to be locked

call 		bdos

The following code segment demonstrates how to unlock records.

mvi		c,2ch	;set multi-sector cnt.

mvi		e,#	;# = num. of sectors

call		bdos	;1<= # <=16

mvi		c,2bh	;unlock record

lxi		d,fcb	;record to be unlocked

call 		bdos

fcb:

db		O,’DATA’,AOH,20H,20H,20H,'DAT’,O

ds		20

db		10,0,0	;beginning at record 10

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.

1

MP/M II T.M. Operating System Release 2.0

Application Note 14, 9/14/81

Copyright © 1981 by Digital Research

MP/M II is a trademark of Digital Research.

GENSYS ENHANCEKKNTS

Applicable products and version numbers: MP/M II T.M. Release 2.0

Enhancements to GENSYS include the following new features and

facilities.

AUTOMATIC RESIDENT SYSTEM PROCESS INCLUSION FACILITY: The GENSYS

automatic system generation facility can be modified to include all

default disk RSP files. Type GENSYS $AR to include the RSP files

automatically. The R option must be used in conjunction with the A

option. Change the filetypes for files that you want to exclude

from GENSYS.

ERROR RECOVERY: If an error is encountered running in automatic

mode ($A option), GENSYS restarts in manual mode.

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.

1

MP/M II T.M. Operating System Release 2.0

Application Note 15, 12/1/81

Copyright © 1981 by Digital Research

MP/M II is a trademark of Digital Research.

CHANGING THE PRIORITY OF SPOOL.PRL

Applicable products and version numbers: MP/M II Release 2.0

The SPOOL utility sets its priority to 201, therefore, most

other processes execute ahead of the SPOOLER. Modify the SPOOL.PRL

file to change the SPOOLER priority. If your product serial number

is between 4-000-00001 and 4-000-00464, install MP/M II Patch 11

before changing the SPOOL.PRL default priority.

Make sure you have a back-up copy of SPOOL.PRL before using DDT

to make the following changes.

A>ren spool.sav=spool.prl

A>ddt spool.sav

[MP/M II] DDT VERS 2.0

NEXT PC

0980 0100

-s269

0269 C9 c8

026A CD .

-ispool.prl

-Wll

-go

A>

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.

1

MP/M II T.M. Operating System Release 2.0

Application Note 16, 12/1/81

Copyright © 1981 by Digital Research

MP/M II is a trademark of Digital Research.

CHANGING THE BACKSPACE AND RUBOUT KEY FUNCTIONS

Applicable products and version numbers: Mp/M II T.M. Release 2.0

Program: RESBDOS

Under MP/M II, the BACKSPACE key or CTRL-H (ASCII 08H) does a

destructive backspace deleting the last character in the command

buffer. The RUBOUT key (ASCII 7FH) or DELETE key deletes the last

character in the command buffer and echoes it to the screen.

Procedure to reverse the BACKSPACE and RUBOUT key functions:

A>ren resbdos.sav=resbdos.spr

A>ddt resbdos.sav

[MP/M II] DDT VERS 2.0

NEXT PC

Of8O 0100

-scOa

OCOA 08 7f

OCOB C2

-sc22

OC22 7F 08

OC23 C2 .

-iresbdos.spr

-wld

-go

OA>gensys

Procedure to make RUBOUT identical to BACKSPACE:

A>ren resbdos.sav=resbdos.spr

A>ddt resbdos.sav

[MP/M II] DDT VERS 2.0

NEXT PC

Of8O 0100

-lc26

OC26 MOV A,B

OC27 ORA A

OC28 Jz 09F6

-ac26

OC26 jmp aOe

OC29

-bb28,1

-iresbdos.spr

-wld

-go

OA>gensys

MP/M II Release 2.0, Application Note 16

RESBDOS, 12/1/81 (cont'd)

Procedure to make BACKSPACE identical to RUBOUT:

A>ren resbdos.sav=resbdos.spr

A>ddt resbdos.sav

[MP/M II] DDT VERS 2.0

NEXT PC

Of8O 0100

-IcOe

OCOE MOV A,B

OCOF ORA A

oclo iz 09F6

-acOe

OCOE jmp, a26

ocil

-bblO,l

-iresbdos.spr

-wld

-go

OA>gensys

Licensed users are granted the right to include these

enhancements in MP/M II Release 2.0 software.