Description of AX.25 'C' code  -- Phil Karn, KA9Q     October 21, 1984

Updated:      June 9, 1985

Overview

The code currently implements AX.25 Level 2 as defined in the Version 2.0

document published by the ARRL. Depending on the choice of low level driver

routines, it will use either the Zilog SIO (native to the 820, but requiring

an external receive clock recovery "state machine" circuit) or the

Zilog 8530 (requiring the FADCA daughter board, or "FAD board") for HDLC

communications. In both cases, an external modem between the SIO or 8530

and the radio is needed.

This code contains a number of extensions to AX.25 which are new with

Version 2.0:

1. Idle link keep-alive polls (timer T3)

2. Acknowledgement delay (timer T1)

3. "C" bits and correct P/F bit usage according to LAPB

Item 3 is upward compatible in that it is not used whenever the software

detects an "old" (i.e., pre-Version 2.0) implementation of AX.25 (which

at this point includes all other AX.25 implementations.)  The keep-alive

polls require the proper use of the P/F bit and will not be sent to

an old implementation.

This code also supports multiple simultaneous logical links. This is an

implementation rather than a protocol issue; it involves keeping individual

"link descriptors" for each station, and using a hash lookup algorithm to

find the appropriate link descriptor whenever a frame is received. Support is

also provided for multiple physical HDLC ports so that more than one channel

can be used.

These extensions required me to separate and classify the commands and

parameters (from a unit like the TAPR TNC) into groups that are applied on a

per-link, -line or -terminal basis as appropriate.

Digipeating works, but currently it will repeat a frame only on the physical

link on which it was received. With relatively little work it would be

possible to do cross-channel digipeating, although with the work underway to

define and implement higher level protocols it is not clear this is a "clean"

thing to do.  Hooks are provided for true level-3 packet switching, e.g.,

with the ARPA Internet Protocol (IP), assuming there is memory left.

A number of things which are in the TAPR TNC are not here. Some of

these I haven't gotten around to implementing yet. Others, such as

CW identification and the Vancouver protocol are obsolete in my opinion

and I don't really feel like putting them in.

Software description

General

All I/O is interrupt driven. Full use is made of the 820's interrupt

capability; queues are emptied or filled with characters going to

and from the terminal and SIO or 8530.  A top-level loop in main.c executes

higher level functions which operate on the "other ends" of the interrupt

queues.  The CTC timer ticks at a 10 hertz rate and provides all real-time

functions.  A halt instruction at the end of the main loop causes the CPU

to wait until an interrupt occurs; this might help reduce RFI and/or

power consumption on a CMOS version of the Z-80.  It also makes possible

the use of a "CPU speedometer" which monitors the HALT line from the CPU

and indicates the amount of real time being used by the software.

ax25.h

The master file of global data structures, defined constants, etc. Included

by just about every .c file, although it should probably be broken

up into more manageable pieces.

820.h, 820.c, line.c, inout.asm, 820ints.asm, fad.h, fad.c

These files hopefully contain all of the machine-dependent code.

820.h, 820.c and inout.asm contains the usual stuff to do bit and port

level twiddling on the Z-80 family chips and the 820 machine.

820ints.asm sets up the register save and restore sequences necessary to call

a C routine directly from an interrupt.

line.c implements routines to read and write raw HDLC frames which, despite

my best efforts at portability, are tied closely to the way the Zilog SIO

operates. It would probably have to be rewritten for a different HDLC chip.

If you are using a FADCA daughter board (with the 8530 add-on), the two files

fad.h and fad.c replace the three files 820.h, 820.c and line.c.

queue.c

Provides primitives getframe() and putframe() which operate on linked-list

queues of an important internal structure called, naturally enough, a "frame".

Two other primitives, allocframe() and freeframe() dynamically create and

destroy frame structures from a common storage heap.

"Fifos" containing individual characters are also provided for terminal

I/O; putf() and getf() operate on these.

addr.c, hash.c

Provides those addressing functions which are unique to AX.25.

It was my intention here to segregate our changes to X.25 to these

files as much as possible.  The function "addrproc" examines the

address fields in the raw frames received from the channel; it also performs

digipeating (if enabled).  It passes frames which the link level protocol

should see (addressed to us and passed all digipeaters) up to the next level

with a command/response indication (if any) derived from the C bits

in the address fields. It uses the hash algorithms in hash.c to find the

appropriate link table entry for the frame, creating a new entry if needed.

sendframe() provides a complementary service for the transmitter.

It takes the link table pointer along with the various components of the

frame (command/response indication, control and data fields), builds

up a frame and queues it on the appropriate HDLC line queue.

link.c

This is the heart of the Level 2 protocol. process() implements all state

transitions and side effects (e.g., timer control) which are required each

time a frame is received. Most of the function consists of nested switch

statements, corresponding to the combinations of received frame type and

current state which dictate each action. This implementation style (as

opposed to a state table) was a matter of personal choice. I found switch

statements easier to read and easier to implement special side effects

(such as changing state variables). I am not that adverse to using a state

table version, but this one seems to work...

After a frame has been processed, the response, if any, is sent to

the transmitter.  Properly received I-frames are passed up to the

next level.

ackours(), frmr(), setstate() and sendctl() are subsidiaries to process();

they are pulled out mainly to keep process() from becoming excessively large

(it still is.) ltxstart() is a "kick" or "priming" routine that tries to

move transmit data through the system whenever possible. It is called from a

number of places whenever it might have just become possible to do this

(e.g., when an acknowledgement has been received) but of course, it frequently

can do nothing. This overhead is an unhappy artifact of the lack of a "real"

operating system base that would support processes and their associated

sleep/wakeup facilities.

timer.c

This file contains functions which implement real-time timers using a

continuous 10 Hz clock tick.  Each timer has three states: stopped,

running and expired. A timer is a data structure which is put on a

doubly-linked list whenever it is running, and automatically removed

when it is stopped or expires. This allows the clock-tick routine

(which could gobble up a fair bit of time since it runs frequently) to

be as simple and fast as possible. User routines must poll the state

of the timer to see if it has expired; there is no direct automatic

execution of a function upon expiration. timer_srv(), called from the

main loop, does this by calling various other routines in the file as

appropriate to recover from link timeouts, etc.  Note that the only thing

done by the clock interrupt handler proper is the setting of a flag which

the higher level routines act on; this is to prevent a lot of computation

from occurring at interrupt time, possibly starving other interrupts.

term.c

Handles all terminal related processing, e.g, input line editing and received

I-frame output processing.  A number of low-level utility functions

for terminal output via a common fifo (e.g., put string and format

functions) are also provided.  Output processing uses an interrupt-driven

queue; while this isn't needed on the 820 because of the memory-mapped

display (in fact it slows it down a bit), writing it in this manner

provides for output flow control and also allows the code to be easily

modified to support a terminal on a UART port, such as the B channel of the

SIO if it isn't needed for HDLC.

trace.c

Handles the display of frames. There are three display modes, encoded into

the "tdata" argument of dump(): 0, meaning no display; 1, providing an

abbreviated display containing the address and control fields broken out

into "human-readable" form, and 2, which in addition provides a hex, ascii

and shifted-ascii raw dump of the frame.

control.c

Top level control functions for level 2.  These include link connect and

disconnect, as well as interfaces for higher level code to write data buffers

to a link.  A main-loop function, level2(), is there to move data on queues

in and out of the level two code.  The hook for level 3, protosw(), is also

here; this is where protocol demultiplexing for I-frames belonging a given

level 3 protocol e.g., DoD IP (or even, God forbid, X.25 level 3...gak!)

will be done. The current level 3 "protocol", hex F0, is interpreted to mean

"send directly to the terminal".

command.h, command.c, cmdtab.c, cmdrout.c

These files implement the command parser. This is a surprisingly large

section of the whole code, reflecting all of the data structures which may

be examined and changed by the user. command.c contains general common

routines to parse command lines and search the tables, while cmdtab.c

contains the tables to bind command strings to function names. cmdrout.c

contains (most of) the functions which implement these user commands.

misc.c

misc.c contains a number of fairly low level utility routines which do such

things as convert ascii strings to AX.25 shifted-ascii format and back,

compare addresses, classify control fields, construct digipeater reply paths,

etc.

init.c

Various hardware and data structure initialization routines, collected

into one place.

space.c

space.c is where space for the external data data structures is defined.

Some C compilers complain bitterly if you have global definitions of the

form "int x;" in more than one file. My solution is to make the include

file contain definitions of the form "EXTERN int x;". In all .c files except

for space.c, EXTERN is defined as "extern"; in space.c, the "EXTERN" prefix

is dropped. This is all done with a little preprocessor magic.  A bit kludgey,

perhaps, but it is a technique now widely used by C programmers, and it DOES

allow you to keep all your definitions in one place.

mkconf.c

This is a standalone program (not linked with the TNC code) which generates

a PARAM.PAD file with some fairly reasonable defaults. There is no need to

modify these defaults because you can always load up the PARAM.PAD file with

the 'load' command, change them and 'save' them back to disk. mkconf just

allows you to get started.

Summary

This is a necessarily sketchy outline of what I've done here. Hopefully,

I have done enough of a good job commenting the code that along with this

overview you shouldn't have too much trouble understanding what's going

on.

Naturally, like any good software hacker, I'm hard at work making improvements

and additions to this code, so things are subject to change. In particular,

I have the following planned:

1. Adding DoD Internet Protocol as a Layer 3 module, along with the ARP

(address resolution protocol).

2. Improved support for serial terminals. There should be a command to

switch the console over to the B channel of the SIO; better yet, the

code should support multiple terminals in the way it now supports

multiple HDLC channels, but there's only so much space on a Z80.

3. Speed improvements. I'm rather disappointed with the speed of this code

on the 820, although a 2.5 Mhz Z-80 isn't the fastest machine in the world.

Channel speed is completely dominated by the interrupt routines in line.c,

and while I've made them as tight as I can, some improvement might be had

by coding sections of them in assembler.  However, I'd MUCH rather have DMA

hardware available, as this is the only way to get the really dramatic

speedup we need to run speeds like 56 kb and up.

4. Improved deadlock protection. There is a lot of dynamic memory allocation

going on, and although I throttle links with RNRs when necessary, there are

still other ways that deadlock might occur, e.g., by having a very large

number of entries in the link table (there is currently no enforced limit.)

I hope this first release of my code is useful to you. Just because I'm

continuing to work on it doesn't keep you from also doing so - I would dearly

love to get back in return any enhancements or bug fixes. I'd also like

comments on the program itself: does it solve your needs? What design flaws

really bother you? Are the commands too awkward? Etc. Remember though that

this is a labor of love for me, and I certainly won't be able to please

everybody.